3GPP TSG-RAN WG4 Meeting #112	R4-2411655

Maastricht, Netherlands, Aug 19 – Aug 23, 2024

Source:	Nokia

Title:	Other RX requirements for LP-WUR

Agenda item:	8.22.2.3

Document for:	Discussion

Introduction

In this paper, we present our views on LP-WUR requirements other than REFSENS, ASCS, and ACS requirements. Following agreements were made in RAN4#111 [1].

Agreements:

### Sub-topic 3-1 General for UE RF Issue 3-1-1: IBB and OBB assumption Agreement: - RAN4 further check and discuss whether same interference level of IBB and OBB as MR is assumed for LP-WUR. - With assumption that Rx dynamic range for LR may not be as good as MR Issue 3-1-2: IBB and OBB requirements Agreement: - RAN4 further discuss detailed test condition for IBB and OBB, e.g., location of LP-WUS in-band, MR channel bandwidth, whether guard RBs are allocated, power level. - FFS IBB and OBB requirements Issue 3-1-4: Intermodulation requirements Agreement: - RAN4 further discuss detailed test condition for Intermodulation requirements, e.g., location of LP-WUS in-band, MR channel bandwidth, whether guard RBs are allocated, power level. - FFS Intermodulation requirements requirement ### Sub-topic 3-2 spurious emissions and response requirements Issue 3-2-1: Spurious emissions requirements Agreement: - RAN4 conclude Spurious emissions requirements can be reused for LP-WUR. Issue 3-2-2: spurious response requirements Agreement: - RAN4 further discuss whether spurious response requirements is needed for LP-WUR. ### Sub-topic 3-3 Other Rx requirements Issue 3-3-1: Maximum input level requirements Agreement: - RAN4 further discuss whether maximum input level requirements can be reused for LP-WUR. - FFS side condition Issue 3-3-2: Narrow band blocking requirements Agreement: - FFS RAN4 no need to define narrow band blocking requirements for LP-WUR.

Discussion

Maximum input level

LP-WUR and main receiver (MR) will observe the same radio environment. Thus, for LP-WUR to reliably operate and co-exist with legacy NR signals, it should be able to handle large input signals. For the main receiver, as there is a limit for the maximum desired input signal level, thus a similar requirement should be specified for the LP-WUR.

Specify maximum input level requirements and side conditions for LP-WUR.

For the main receiver, the requirement power level varies based on channel bandwidth and modulation type. However, as possible bandwidths being considered for LP-WUS in FR1 are ≤5MHz, a single value maybe sufficient.

Possible bandwidths being considered for LP-WUS in FR1 are ≤5MHz.

A single value for the maximum input level is sufficient for LP-WUR.

The value of maximum input level depends on the maximum gNB output power and the minimum coupling loss (MCL) between the gNB and the UE. The maximum gNB output power has not changed and is same for LP-WUS and NR signal. Further, there is no change on the assumptions regarding MCL. Thus, the value of maximum input level for MR can be used for LP-WUR.

There hasn’t been any change in assumptions regarding MCL and gNB output power.

Use maximum input level of -25 dBm for LP-WUR.

The reference measurement channel for the maximum input level requirement shall be defined for the LP-WUR as the current reference channels use 64 QAM or 256 QAM. Further, the number of RBs is going to be different compared to the current ones.

Define reference channel for LP-WUR requirements.

Blocking requirements

It had already been agreed to define in-band blocking (IBB) and out-of-band blocking (OBB) requirements. However, narrow band blocking requirements were not discussed. During the last RAN4 meeting, some companies wanted to consider all legacy receiver requirements for the LP-WUR. However, we should not just define requirements for a receiver if they do not provide any additional information regarding the performance of the LP-WUR, for example for Narrow band IoT devices narrow band blocking requirements are not defined because ACS requirements are more stringent than what the narrow band blocking requirements would have been.

Narrow band blocking requirements haven’t been discussed in context of LP-WUR.

Additional requirements will not provide additional information regarding receiver performance if some more stringent requirement already exists. They will just increase the time for conformance tests.

No need to define narrow band blocking requirements.

Conclusion

The following Observations and Proposals were made:

Proposal 1: Specify maximum input level requirements and side conditions for LP-WUR.

Observation 1: Possible bandwidths being considered for LP-WUS in FR1 are ≤5MHz.

Proposal 2: A single value for the maximum input level is sufficient for LP-WUR.

Observation 2: There hasn’t been any change in assumptions regarding MCL and gNB output power.

Proposal 3: Use maximum input level of -25 dBm for LP-WUR.

Proposal 4: Define reference channel for LP-WUR requirements.

Observation 3: Narrow band blocking requirements haven’t been discussed in context of LP-WUR.

Observation 4: Additional requirements will not provide additional information regarding receiver performance if some more stringent requirement already exists. They will just increase the time for conformance tests.

Proposal 5: No need to define narrow band blocking requirements.

- References

- R4-2410569, WF for [111][136] NR_LPWUS_UERF, Vivo, RAN4#110bis, Changsha, China, 15 – 19 April 2024.