3GPP TSG-RAN WG4 Meeting #112	R4-2411657

Maastricht, Netherlands, Aug 19 – Aug 23, 2024

Source:	Nokia

Title:	RedCap NTN UEs

Agenda item:	8.25.2.1

Document for:	Discussion

Introduction

In this paper, we provide our views on the support of Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1-NTN bands. RedCap specific objectives mentioned in the WID are copied below for ease of reading.

Objectives:

- Support of Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1-NTN bands [RAN4, RAN1] - For full-duplex FDD RedCap and eRedCap UEs, define the RF and RRM requirements [RAN4] - For HD-FDD RedCap UEs and eRedCap UEs, check whether any essential changes are needed for their support (i.e. focusing on HD collision rules) by end of Q2/2024 [RAN1] - Depending on feasibility assessment above, define the RF and RRM requirements [RAN4] - Notes for this objective: GNSS (Global Navigation Satellite Systems) capabilities and simultaneous GNSS and NR-NTN operation is supported in RedCap/eRedCap UE.

The following agreements were made in the last RAN4#111 meeting [2]

## Issue 1-1 UL RB allocation for HD-FDD Refsens, both for 1Rx and 2Rx Agreement: Full RB allocation. ## Issue 1-2: HD-FDD refsens for 2 Rx Agreement: [0.5] dB tightening for n254 and n256, FFS for n255. n256 to be confirmed in RAN4#112. Note: RAN4#111 maintenance agreement on n256 refsens in R4-2408787 was not considered for this agreement. ## Issue 1-3: HD-FDD refsens for 1 Rx Agreement: 2.5 dB relaxation compared to 2Rx HD-FDD ## Issue 1-4: Frequency error Agreement: Discuss changes common to non-Redcap NTN and (e)RedCap NTN NR UE frequency error in maintenance agenda. (e)RedCap UE frequency error requirement may be impacted by this maintenance discussion. Option for further discussionn, not an agreement: - As a starting point, consider (e)Redcap UE frequency error based on existing NR UE requirements. ## Issue 1-5: Maximum output power options for further discussion: Option 1: Consider increase in PC3 power level for NTN RedCap UE which only supports HD-FDD operation for a band. Option 2: PC3 power level is the same for non-RedCap NR NTN UE and (e)RedCap NR NTN UE ## Issue 1-6: Specification structure and baseline changes Agreement: Use R4-2408816 as starting point for (e)RedCap NR NTN UE RF requirements specification structure. Further changes on requirement values and additional impacted clauses (e.g. according to issue 1-4 and 1-5) as well as other improvements can be added. - FFS for suffix letter.

Discussion

HD-FDD REFSENS for 2RX

During the last meeting, the amount by which the REFSENS for n254 and n256 should be tightened was not agreed. There are two potential numbers that were discussed,

- 0.5 dB,

- 0.8 dB.

The reasoning for 0.5dB goes back to almost 10 years old limited survey on insertion loss difference between a SAW RF filter vs SAW duplex filter. As SAW RF filters have less insertion loss compared to SAW duplex filters and since HD-FDD implementations do not need duplex filters, REFSENS can be tightened by 0.5 dB. 0.8dB number comes from more recent discussions done under Release 17 RedCap for HD-FDD case. Given that lot of improvement in device technology has happened over last 10 years, it is possible to leverage that improvement for improving the REFSENS.

Further, as n65 exactly overlaps n256 and 0.5 dB had been used in the TN spec for HD-FDD for n65, there is an argument to be made that REFSENS should not be tightened more than 0.5 dB.

Reasoning for 0.5dB improvement goes back to almost 10 years old survey on insertion loss differences.

n256 is same as n65 and 0.5dB had been used for HD-FDD case in n65.

Agree to use 0.8 dB tightening for n254.

Agree to use 0.5 dB tightening for n256.

PC3 power level

In RAN4#111, it was proposed to increase the nominal power level for PC3 for NTN RedCap UE which only supports HD-FDD operation from 23 dBm to 24 dBm. The reasoning for this was related to the challenging uplink link budget in the NTN case. In our opinion, although it is beneficial to have higher output power, doing so in the specification by changing the nominal value for PC3 is not a good way. A better way would be via introduction of a new power class, for which work should be defined in the WID. Another approach to improve the uplink link budget could be by reducing the MPR and defining a new MPR table for the NTN case.

Changing PC3 level for HD-FDD in NTN context is not a good approach.

- Better approach would be via introduction of a new power class.

Another approach to improve the uplink link budget would be via reduced MPR for HD-FDD in NTN case.

PC3 power level should be the same in NTN and TN context.

Conclusion

The following Observations and Proposals were made:

Observation 1: Reasoning for 0.5dB improvement goes back to almost 10 years old survey on insertion loss differences.

Observation 2: n256 is same as n65 and 0.5dB had been used for HD-FDD case in n65.

Proposal 1: Agree to use 0.8 dB tightening for n254.

Proposal 2: Agree to use 0.5 dB tightening for n256.

Observation 3: Changing PC3 level for HD-FDD in NTN context is not a good approach.

Observation 4: Better approach would be via introduction of a new power class.

Observation 5: Another approach to improve the uplink link budget would be via reduced MPR for HD-FDD in NTN case.

Proposal 3: PC3 power level should be the same in NTN and TN context.

- References

- RP-240775, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3, RAN#103, Maastricht, Netherlands, 18 – 22 March 2023.

- R4-2410574, WF on Rel-19 phase 3 NR-NTN UE RF requirements, Qualcomm Inc, RAN4#111, Fukuoka, Japan, 20 – 24 May 2024.