3GPP TSG-RAN WG4 Meeting #112	R4-2411658

Maastricht, Netherlands, Aug 19 – Aug 23, 2024

Source:	Nokia

Title:	Discussion on Ku band for NR NTN

Agenda item:	8.9.2

Document for:	Discussion

Introduction

In this paper, we provide our views on the support of Ku band for NR NTN. Core parts of the objectives mentioned in the WID are copied below for ease of reading.

Objectives:

The following assumptions are taken as the baseline for this work: - GSO and NGSO - NTN capable UE type 1 to 5 (38.101-5 Table 9.2.1.0-1) for above 10 GHz (no handheld devices) - FDD Mode - GNSS Receiver capability The objectives are: - Use the regulation requirements in ITU Regions 1, 2 and 3 to identify relevant adjacent band co-existence scenarios for NTN Ku Band covering the following frequency ranges, considering targeted deployment scenarios: - For ITU regions 1 and 3: - Downlink 10.70 – 12.75 GHz - Uplink 13.75-14.5 GHz - For ITU regions 2 excluding US: - Downlink 10.70 – 12.70 GHz - Uplink 13.75-14.5 GHz - For ITU regions 1 and 3 - Downlink 10.70 – 12.75 GHz - Uplink 12.75 - 13.25 GHz - For ITU regions 2 excluding US - Downlink 10.70 – 12.70 GHz - Uplink 12.70 - 13.25 GHz - Note: Appropriate TX/Rx separation for uplink downlink pairing will be required. - Conduct coexistence study. Reuse Ka band coexistence study assumptions as much as possible, with modifications where necessary, e.g. for frequency range and subcarrier spacing. Conduct new co-existence studies based on adjacent services . - Specify RF requirements for satellite access node and NTN VSAT types in 38.101-5 also considering existing regulations on antenna sizes for certain parts of the Ku band. - Priority 1: for all ITU regions, define Ku band #1a and #1b for the downlink block and uplink block B to enable the future possibility of supporting satellite services in 125 MHz (UL) and 250 MHz (DL) aggregated bandwidths based on existing channel bandwidths. Potential approaches for achieving this shall be studied, from which one shall be selected including but not limited to: - Ku band #1a and #1b as FR1-NTN supporting 20 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidths with 30 kHz subcarrier spacing - Ku band #1a and #1b as FR2-NTN supporting 50 MHz, 100 MHz, 200 MHz, 400 MHz (120kHz SCS only) channel bandwidths with 120 kHz subcarrier spacing - RAN4 will select which one of the above approaches for DL and UL will be pursued by RAN #106. - Priority 2: for all ITU regions excluding the US in region 2, using the approach in Priority 1, define Ku band #2a and #2b as appropriate for the downlink block and uplink block A to support satellite services in 125 MHz (UL) and 250 MHz (DL) aggregated bandwidths. - Specify RRM requirements Note: Support for NTN CA is not in the scope of this WID

Discussion

Co-existence studies

Given new bands are to be introduced, co-existence studies are required. Further, as suggested by the WID, existing Ka band co-existence assumptions should be reused as much as possible. A lot of work for co-existence was done for Ka band and is captured in [2]. Thus, excluding the actual frequency range numbers, the simulation scenarios, assumptions, and methodology can be reused from section 6 of [2].

Agree to reuse simulation scenarios, assumptions and methodology from TR 38.863.

Conclusion

The following Observations and Proposals were made:

Proposal 1: Agree to reuse simulation scenarios, assumptions and methodology from TR 38.863.

- References

- RP-241690, New WID on Introduction of Ku Band for NR NTN, RAN#104, Shanghai, China, 17 – 20 June 2024.

- TR 38.863, Solutions for NR to support non-terrestrial networks (NTN): Non-terrestrial networks (NTN) related RF and co-existence aspects.