3GPP TSG-RAN WG4 Meeting #112	R4-2411659

Maastricht, Netherlands, Aug 19 – Aug 23, 2024

Source:	Nokia

Title:	Discussion on HPUE TX requirements for NR NTN

Agenda item:	8.8.2.2

Document for:	Discussion

Introduction

In this paper, we provide our views on the support of high power UE RF requirements for NR NTN. Core parts of the objectives mentioned in the WID are copied below for ease of reading [1].

Objectives:

- Conduct the coexistence analysis based on TR 38.863 for the above example bands and corresponding power classes - Evaluate the impact on the uplink performance of TN (Terrestrial network) - Example bands include bands n256 and 256, n255 and 255 - PC2, PC1.5 (for NR-NTN only) and PC1 will be evaluated for ACLR and ACS - For NR-NTN and IoT-NTN: coexistence studies shall be carried out under the same conditions, including geographical separation between NTN UE and Terrestrial Networks (TN), as the co-existence studies in Rel-17 and Rel-18 - Specify high power UE (HPUE) for NR-NTN (Non-Terrestrial Networks) and IoT-NTN (NB-IoT and eMTC based NTN) in FR1-NTN bands and the corresponding LTE NTN bands for the single uplink (UL) carrier scenario - Support UE with power class 2 (PC2, 26 dBm) that applies to both handheld and non-handheld based on the co-existence study. - Support other power classes based on the co-existence and feasibility study - Non-hand held UE is prioritized - Specify RF requirements for all considered power classes for NR-NTN and IoT-NTN in FR1-NTN bands and the corresponding LTE NTN bands - Necessary Tx requirements, at least including - Investigate and if necessary, specify the solution to address potential SAR (Specific Absorption Rate restriction) requirements for handheld UE - Specify MPR and A-MPR for the example bands considering the regulation for the corresponding bands, if necessary - Necessary Rx requirements, at least including - Specify the RSD requirements on the example bands, if necessary - Specify the necessary signaling to support the above objectives

Discussion

Co-existence simulations are required for evaluating the impact on uplink performance of TN network by the high power NTN UEs. A significant amount of work for co-existence was done and is captured in [2]. Although, the scenarios will be limited as the WID only specifies impact on the uplink performance of TN i.e., victim being TN UL only for PC2 and PC1.5. Regarding PC1, WID specifies evaluation for both ACLR and ACS, thus additional scenarios with NR-NTN and IoT-NTN as the victim must be considered.

The WID specifies evaluation for both ACLR and ACS, thus additional scenarios with NR-NTN and IoT-NTN as the victim must be considered.

In general, the simulation assumptions, and methodology can be reused from section 6 of [2].

Regarding TN-NTN co-existence analysis for PC2 and PC1.5, agree to limit the simulation scenarios to one with TN UL being the victim.

Co-existence scenarios for PC1 should consider both TN UL and NTN UL being the victim.

Agree to reuse simulation assumptions and methodology from TR 38.863.

As the co-existence scenarios are very different for n255 and n256 as compared to other TN NR bands, requirements can only be derived by doing simulations and evaluation.

Co-existence scenarios are very different for n255 and n256 as compared to other TN NR bands.

This meaning scaling the values from TN specification is not a possibility for NTN in this case.

Derive the requirements via new co-existence simulation results only.

Conclusion

The following Observations and Proposals were made:

Observation 1: The WID specifies evaluation for both ACLR and ACS, thus additional scenarios with NR-NTN and IoT-NTN as the victim must be considered.

Proposal 1: Regarding TN-NTN co-existence analysis for PC2 and PC1.5, agree to limit the simulation scenarios to one with TN UL being the victim.

Proposal 2: Co-existence scenarios for PC1 should consider both TN UL and NTN UL being the victim.

Proposal 3: Agree to reuse simulation assumptions and methodology from TR 38.863.

Observation 2: Co-existence scenarios are very different for n255 and n256 as compared to other TN NR bands.

Proposal 4: Derive the requirements via new co-existence simulation results only.

- References

- RP-240857, New WID: Enhanced requirements and test methodology for NR and IoT NTN, RAN#103, Maastricht, Netherlands, 18-21 March 2024.

- TR 38.863, Solutions for NR to support non-terrestrial networks (NTN): Non-terrestrial networks (NTN) related RF and co-existence aspects.