3GPP TSG-RAN4 Meeting #112	R4-2411666

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | 0585 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for 38.101-4 on PMI req for typeII-CJT-r18 for FR1 FDD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MIMO_evo_DL_UL-Perf |  |  |  |  |  | Date: |  |  | 2024-08-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Removal of [] to finalize PMI requirements for typeII CJT |  |  |  |  |  |  |  |  |
| Summary of change: |  | [] removed |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | PMI requiremets for typeII CJT is not finalized. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2.1.9, 6.3.3.1.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-4 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# <Start of change 1>

##### 6.3.2.1.9	Multiple PMI with 8 ports Enhanced Type II Codebook for CJT

For the parameters specified in Table 6.3.2.1.9-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified in Table 6.3.2.1.9-2.

Table 6.3.2.1.9-1: Test parameters

| Parameter |  |  |  |  | Unit | Value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | TRxP #1(Note 1) | TRxP #2(Note 1) |
| Transmit TRxP of SSB |  |  |  |  |  | TRxP #1 |  |
| PDCCH configuration |  |  | TCI state |  |  | TCI State #1 |  |
|  |  |  | CORESETPoolIndex |  |  | 0 |  |
| CSI-RS for tracking |  |  | First subcarrier index in the PRB used for CSI-RS |  |  | k0=0 for CSI-RS resources 1,2,3,4 |  |
|  |  |  | First OFDM symbol in the PRB used for CSI-RS |  |  | l0 = 6 for CSI-RS resources 1 and 3 l0 = 10 for CSI-RS resources 2 and 4 |  |
|  |  |  | Number of CSI-RS ports (X) |  |  | 1 for CSI-RS resource 1,2,3,4 |  |
|  |  |  | CDM Type |  |  | ‘No CDM’ for CSI-RS resource 1,2,3,4 |  |
|  |  |  | Density |  |  | 3 |  |
|  |  |  | CSI-RS periodicity |  | Slots | 20 |  |
|  |  |  | CSI-RS offset |  | Slots | 10 for CSI-RS resources 1 and 2 11 for CSI-RS resources 3 and 4 |  |
|  |  |  | QCL info |  |  | TCI state #0 |  |
| Duplex mode |  |  |  |  |  | FDD |  |
| Bandwidth |  |  |  |  | MHz | 10 |  |
| Subcarrier spacing |  |  |  |  | kHz | 15 |  |
| Active DL BWP index |  |  |  |  |  | 1 |  |
| Propagation channel |  |  |  |  |  | TDLA30-10 |  |
| Antenna configuration per TRxP |  |  |  |  |  | High XP 8 x 2 (N1,N2) = (4,1) |  |
| Beamforming Model |  |  |  |  |  | As specified in Annex B.4.1 (Note 4) |  |
| PDSCH configuration | Mapping type |  |  |  |  | Type A |  |
|  | k0 |  |  |  |  | 0 |  |
|  | Starting symbol (S) |  |  |  |  | 2 |  |
|  | Length (L) |  |  |  |  | 12 |  |
|  | PRB bundling type |  |  |  |  | Static |  |
|  | PRB bundling size |  |  |  |  | 2 |  |
|  | Resource allocation type |  |  |  |  | Type 1 |  |
|  | RBG size |  |  |  |  | Config2 |  |
|  | VRB-to-PRB mapping type |  |  |  |  | Non-interleaved |  |
|  | VRB-to-PRB mapping interleaver bundle size |  |  |  |  | N/A |  |
| PDSCH DMRS configuration | Antenna port indexes |  |  |  |  | 1000,1001 |  |
|  | TCI state |  |  |  |  | TCI State #1 |  |
|  | DMRS Type |  |  |  |  | Type 1 |  |
|  | Number of additional DMRS |  |  |  |  | 1 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  |  |  | 1 |  |
| TCI State #1 | Type 1 QCL information |  |  | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking’ configuration |  |
|  |  |  |  | QCL Type |  | Type A |  |
|  | Type 2 QCL information |  |  | CSI-RS resource |  | N/A |  |
|  |  |  |  | QCL Type |  | N/A |  |
| Resource allocation |  |  |  |  |  | Full-overlapping |  |
| Timing offset of the second TRxP from the first TRxP |  |  |  |  | us | 0 |  |
| Frequency offset of the second TRxP from the first TRxP |  |  |  |  | Hz | 0 |  |
| Number of HARQ Processes |  |  |  |  |  | 4 |  |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  |  |  |  | 2 |  |
| ZP CSI-RS configuration |  | CSI-RS resource Type |  |  |  | Periodic |  |
|  |  | Number of CSI-RS ports (X) |  |  |  | 4 |  |
|  |  | CDM Type |  |  |  | FD-CDM2 |  |
|  |  | Density (ρ) |  |  |  | 1 |  |
|  |  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  |  | Row 5,(4) |  |
|  |  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  |  | (9) |  |
|  |  | CSI-RS periodicity and offset |  |  | slot | 5/1 |  |
| NZP CSI-RS for CSI acquisition |  | CSI-RS resource ID |  |  |  | Resource #5 | Resource #6 |
|  |  | CSI-RS resource Type |  |  |  | Aperiodic | Aperiodic |
|  |  | Number of CSI-RS ports (X) |  |  |  | 8 | 8 |
|  |  | CDM Type |  |  |  | CDM4 (FD2, TD2) | CDM4 (FD2, TD2) |
|  |  | Density (ρ) |  |  |  | 1 | 1 |
|  |  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  |  | Row 8, (4,6) | Row 8, (4,6) |
|  |  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  |  | (5) | (9) |
|  |  | CSI-RS periodicity and offset |  |  | slot | Not configured | Not configured |
|  |  | aperiodicTriggeringOffset |  |  |  | 0 | 0 |
| CSI-IM configuration |  | CSI-IM resource Type |  |  |  | Aperiodic |  |
|  |  | CSI-IM RE pattern |  |  |  | Pattern 0 |  |
|  |  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  |  | (4,9) |  |
|  |  | CSI-IM timeConfig periodicity and offset |  |  | slot | Not configured |  |
| ReportConfigType |  |  |  |  |  | Aperiodic |  |
| CQI-table |  |  |  |  |  | Table 1 |  |
| reportQuantity |  |  |  |  |  | cri-RI-PMI-CQI |  |
| csi-ReportMode |  |  |  |  |  | Mode2 |  |
| timeRestrictionForChannelMeasurements |  |  |  |  |  | Not configured |  |
| timeRestrictionForInterferenceMeasurements |  |  |  |  |  | Not configured |  |
| cqi-FormatIndicator |  |  |  |  |  | Wideband |  |
| pmi-FormatIndicator |  |  |  |  |  | Not Configured |  |
| Sub-band Size |  |  |  |  | RB | 8 |  |
| csi-ReportingBand |  |  |  |  |  | 1111111 |  |
| CSI-Report periodicity and offset |  |  |  |  | slot | Not configured |  |
| Aperiodic Report Slot Offset |  |  |  |  |  | 5 |  |
| CSI request |  |  |  |  |  | 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0 |  |
| reportTriggerSize |  |  |  |  |  | 1 |  |
| CSI-AperiodicTriggerStateList |  |  |  |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |  |
| Codebook configuration |  | CodebookType |  |  |  | TypeII-CJT-r18 |  |
|  |  | CodebookMode |  |  |  | Mode 2 |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  |  |  | (4,1) |  |
|  |  | (CodebookConfig-O1,CodebookConfig-O2) |  |  |  | (4,1) |  |
|  |  | CodebookSubsetRestriction |  |  |  | 0x FFFF |  |
|  |  | paramCombination-CJT-L-r18 |  |  |  | 7 ({4, 4}) |  |
|  |  | paramCombination-CJT-r18 |  |  |  | 4 (pν=1/4 and 1/8, β=1/2) |  |
|  |  | RI Restriction |  |  |  | 00000010 |  |
| Physical channel for CSI report |  |  |  |  |  | PUSCH |  |
| CQI/RI/PMI delay |  |  |  |  | ms | 14 |  |
| Maximum number of HARQ transmission |  |  |  |  |  | 4 |  |
| Measurement channel |  |  |  |  |  | R.PDSCH.1-6.3 |  |
| PDSCH & PDSCH DMRS Precoding configuration for random Precoding |  |  |  |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with i1 wideband granularity and i2 subband granularity |  |
| Note 1: PDSCH transmission is done from both TRxPs, where both TRxPs will send the same PDSCH data Note 2: When Throughput is measured using random precoder selection, the precoder shall be updated in each slot (1 ms granularity) with equal probability of each applicable i1, i2 combination. For each TRP, use random type I SP PMI precoding independently. Note 3: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-7), this reported PMI cannot be applied at the gNB downlink before slot#(n+7). Note 4: Randomization of the dual-cluster beam directions shall be used as specified in AnnexB.2.3.2.3A. The value of relative power ratio (p) shall be fixed as 1 during the test. |  |  |  |  |  |  |  |

Table 6.3.2.1.9-2: Minimum requirement

| Parameter | Test 1 |
| --- | --- |
|  | 1.8 |

# <End of change 1>

# <Start of change 2>

##### 6.3.3.1.9	Multiple PMI with 8 ports Enhanced Type II Codebook for CJT

For the parameters specified in Table 6.3.3.1.9-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified in Table 6.3.3.1.9-2.

Table 6.3.3.1.9-1: Test parameters

| Parameter |  |  |  |  | Unit | Value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | TRxP #1(Note 1) | TRxP #2(Note 1) |
| Transmit TRxP of SSB |  |  |  |  |  | TRxP #1 |  |
| PDCCH configuration |  |  | TCI state |  |  | TCI State #1 |  |
|  |  |  | CORESETPoolIndex |  |  | 0 |  |
| CSI-RS for tracking |  |  | First subcarrier index in the PRB used for CSI-RS |  |  | k0=0 for CSI-RS resources 1,2,3,4 |  |
|  |  |  | First OFDM symbol in the PRB used for CSI-RS |  |  | l0 = 6 for CSI-RS resources 1 and 3 l0 = 10 for CSI-RS resources 2 and 4 |  |
|  |  |  | Number of CSI-RS ports (X) |  |  | 1 for CSI-RS resource 1,2,3,4 |  |
|  |  |  | CDM Type |  |  | ‘No CDM’ for CSI-RS resource 1,2,3,4 |  |
|  |  |  | Density |  |  | 3 |  |
|  |  |  | CSI-RS periodicity |  | Slots | 20 |  |
|  |  |  | CSI-RS offset |  | Slots | 10 for CSI-RS resources 1 and 2 11 for CSI-RS resources 3 and 4 |  |
|  |  |  | QCL info |  |  | TCI state #0 |  |
| Duplex mode |  |  |  |  |  | FDD |  |
| Bandwidth |  |  |  |  | MHz | 10 |  |
| Subcarrier spacing |  |  |  |  | kHz | 15 |  |
| Active DL BWP index |  |  |  |  |  | 1 |  |
| Propagation channel |  |  |  |  |  | TDLA30-10 |  |
| Antenna configuration per TRxP |  |  |  |  |  | High XP 8 x 4 (N1,N2) = (4,1) |  |
| Beamforming Model |  |  |  |  |  | As specified in Annex B.4.1 (Note 4) |  |
| PDSCH configuration | Mapping type |  |  |  |  | Type A |  |
|  | k0 |  |  |  |  | 0 |  |
|  | Starting symbol (S) |  |  |  |  | 2 |  |
|  | Length (L) |  |  |  |  | 12 |  |
|  | PRB bundling type |  |  |  |  | Static |  |
|  | PRB bundling size |  |  |  |  | 2 |  |
|  | Resource allocation type |  |  |  |  | Type 1 |  |
|  | RBG size |  |  |  |  | Config2 |  |
|  | VRB-to-PRB mapping type |  |  |  |  | Non-interleaved |  |
|  | VRB-to-PRB mapping interleaver bundle size |  |  |  |  | N/A |  |
| PDSCH DMRS configuration | Antenna port indexes |  |  |  |  | 1000,1001 |  |
|  | TCI state |  |  |  |  | TCI State #1 |  |
|  | DMRS Type |  |  |  |  | Type 1 |  |
|  | Number of additional DMRS |  |  |  |  | 1 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  |  |  | 1 |  |
| TCI State #1 | Type 1 QCL information |  |  | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking’ configuration |  |
|  |  |  |  | QCL Type |  | Type A |  |
|  | Type 2 QCL information |  |  | CSI-RS resource |  | N/A |  |
|  |  |  |  | QCL Type |  | N/A |  |
| Resource allocation |  |  |  |  |  | Full-overlapping |  |
| Timing offset of the second TRxP from the first TRxP |  |  |  |  | us | 0 |  |
| Frequency offset of the second TRxP from the first TRxP |  |  |  |  | Hz | 0 |  |
| Number of HARQ Processes |  |  |  |  |  | 4 |  |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  |  |  |  | 2 |  |
| ZP CSI-RS configuration |  | CSI-RS resource Type |  |  |  | Periodic |  |
|  |  | Number of CSI-RS ports (X) |  |  |  | 4 |  |
|  |  | CDM Type |  |  |  | FD-CDM2 |  |
|  |  | Density (ρ) |  |  |  | 1 |  |
|  |  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  |  | Row 5,(4) |  |
|  |  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  |  | (9) |  |
|  |  | CSI-RS periodicity and offset |  |  | slot | 5/1 |  |
| NZP CSI-RS for CSI acquisition |  | CSI-RS resource ID |  |  |  | Resource #5 | Resource #6 |
|  |  | CSI-RS resource Type |  |  |  | Aperiodic | Aperiodic |
|  |  | Number of CSI-RS ports (X) |  |  |  | 8 | 8 |
|  |  | CDM Type |  |  |  | CDM4 (FD2, TD2) | CDM4 (FD2, TD2) |
|  |  | Density (ρ) |  |  |  | 1 | 1 |
|  |  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  |  | Row 8, (4,6) | Row 8, (4,6) |
|  |  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  |  | (5) | (9) |
|  |  | CSI-RS periodicity and offset |  |  | slot | Not configured | Not configured |
|  |  | aperiodicTriggeringOffset |  |  |  | 0 | 0 |
| CSI-IM configuration |  | CSI-IM resource Type |  |  |  | Aperiodic |  |
|  |  | CSI-IM RE pattern |  |  |  | Pattern 0 |  |
|  |  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  |  | (4,9) |  |
|  |  | CSI-IM timeConfig periodicity and offset |  |  | slot | Not configured |  |
| ReportConfigType |  |  |  |  |  | Aperiodic |  |
| CQI-table |  |  |  |  |  | Table 1 |  |
| reportQuantity |  |  |  |  |  | cri-RI-PMI-CQI |  |
| csi-ReportMode |  |  |  |  |  | Mode2 |  |
| timeRestrictionForChannelMeasurements |  |  |  |  |  | Not configured |  |
| timeRestrictionForInterferenceMeasurements |  |  |  |  |  | Not configured |  |
| cqi-FormatIndicator |  |  |  |  |  | Wideband |  |
| pmi-FormatIndicator |  |  |  |  |  | Not Configured |  |
| Sub-band Size |  |  |  |  | RB | 8 |  |
| csi-ReportingBand |  |  |  |  |  | 1111111 |  |
| CSI-Report periodicity and offset |  |  |  |  | slot | Not configured |  |
| Aperiodic Report Slot Offset |  |  |  |  |  | 5 |  |
| CSI request |  |  |  |  |  | 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0 |  |
| reportTriggerSize |  |  |  |  |  | 1 |  |
| CSI-AperiodicTriggerStateList |  |  |  |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |  |
| Codebook configuration |  | CodebookType |  |  |  | TypeII-CJT-r18 |  |
|  |  | CodebookMode |  |  |  | Mode 2 |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  |  |  | (4,1) |  |
|  |  | (CodebookConfig-O1,CodebookConfig-O2) |  |  |  | (4,1) |  |
|  |  | CodebookSubsetRestriction |  |  |  | 0x FFFF |  |
|  |  | paramCombination-CJT-L-r18 |  |  |  | 7 ({4, 4}) |  |
|  |  | paramCombination-CJT-r18 |  |  |  | 4 (pν=1/4 and 1/8, β=1/2) |  |
|  |  | RI Restriction |  |  |  | 00000010 |  |
| Physical channel for CSI report |  |  |  |  |  | PUSCH |  |
| CQI/RI/PMI delay |  |  |  |  | ms | 14 |  |
| Maximum number of HARQ transmission |  |  |  |  |  | 4 |  |
| Measurement channel |  |  |  |  |  | R.PDSCH.1-6.3 |  |
| PDSCH & PDSCH DMRS Precoding configuration for random Precoding |  |  |  |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with i1 wideband granularity and i2 subband granularity |  |
| Note 1: PDSCH transmission is done from both TRxPs, where both TRxPs will send the same PDSCH data Note 2: When Throughput is measured using random precoder selection, the precoder shall be updated in each slot (1 ms granularity) with equal probability of each applicable i1, i2 combination. For each TRP, use random type I SP PMI precoding independently. Note 3: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-7), this reported PMI cannot be applied at the gNB downlink before slot#(n+7). Note 4: Randomization of the dual-cluster beam directions shall be used as specified in AnnexB.2.3.2.3A. The value of relative power ratio (p) shall be fixed as 1 during the test. |  |  |  |  |  |  |  |

Table 6.3.3.1.9-2: Minimum requirement

| Parameter | Test 1 |
| --- | --- |
|  | 1.8 |

# <End of change 2>