3GPP TSG-RAN WG4 Meeting #112	R4-2411682

Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:	8.17.3

Source: 		NTT DOCOMO, INC.

Title: 	Discussions on testability and interoperability for positioning

Document for:	Discussion

- Introduction

Issue 3-2: Requirements for case 2a

Agreement:

No requirements on LMF for positioning accuracy

FFS on RAN4 requirements for any UE reported measurements defined by other groups

Issue 3-3: Requirements for case 2b

Agreement:

No requirements on LMF for positioning accuracy

FFS on RAN4 requirements for any UE reported measurements defined by other groups

During the past meetings, the necessity of accuracy requirements has been discussed. For case 2a and 2b, following agreements are captured in WF [1].

About case 1, since the reported information from UE is not the measurement results but the pure location information, the discussion on accuracy requirements was controversial. This paper further discusses how to treat the accuracy requirements for case 1.

- Discussion

From operator point of view, we have to ensure the accuracy to use inferences for some services. In case 2, the way is simple. The UE receives reference signal for positioning and reports corresponding measurement results, or UE predicts corresponding measurement results. Thus, the accuracy can be ensured by current performance requirements for positioning. However, in case 1, since the inference is pure location information, no reference value to ensure the accuracy has not been defined. There are at least two ways to ensure the accuracy of direct positioning.

- Define new performance requirements using location information as a ground truth

- Regard predicted location information as an ensured value if the training has been done by accurate values

About option 1, obviously it is difficult to define what is the ground truth. GNSS output may be used for it, but it causes new problems, e.g. how to make the test case? About option 2, the way of thinking is same as case 2b. If training data is accurate, the predicted location information can be treated as enough accurate. Thus we propose option 2 to ensure the accuracy of case 1.

Proposal 1: No additional requirements for positioning accuracy for case 1. It can be ensured if training data is accurate.

Due to this assumption, further discussion is needed about LCM procedure, especially what value should be used to deactivate functionality/model and to fallback appropriately.

- Conclusion

In this contribution, we proposed our views on testability and interoperability for positioning especially case 1.

Proposal 1: No additional requirements for positioning accuracy for case 1. It can be ensured if training data is accurate.

References

[1] 3GPP, R4-2410570, “WF on AI/ML”, Qualcomm Incorporated