3GPP TSG-RAN WG4 Meeting #112	R4-2411683

Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:	8.22.4.2

Source: 		NTT DOCOMO, INC.

Title: 	Discussions on RRM core requirements for LP-WUS/WUR

Document for:	Discussion

- Introduction

As summarized in WF [1], there are still many open discussion points. This document proposes RRM core requirements for LP-WUS/WUR based on the WFs.

- Discussion

  - General aspects

About core requirements to be specified for LP-WUR measurement, many companies input their views. Reviewing all the views, the way seems to be aligned as follows.

- Measurement requirements for OOK based LP-WUR should be considered

- Measurement requirements for OFDM based LP-WUR should be suspended until more RAN1 progress is available

- Interruption time between LP-WUS received paging indication to MR ready for Paging monitoring, and MR going to be turned off

Based on the above items, core requirements to be specified for LP-WUR measurement should be as following proposal.

Proposal 1: At Rel-19 LP-WUR WI, for LP-WUR measurement, RAN4 specifies measurement requirements for the following:

- Measurement requirements for OOK based LP-WUR serving cell measurement based on LP-SS at Idle/Inactive state by considering:

  - Interruption time between LP-WUS received paging indication to MR ready for Paging monitoring, and MR going to be turned off

- Suspend the discussion of OFDM-based LP-WUR serving cell measurement requirement based on LP-SS at Idle/Inactive state until more RAN1 progress is available

- Other related requirements are FFS

About cases/states to be considered for RRM relaxation, totally 4 cases were proposed and case 2 to 4 are FFS.

Issue 1-1-2: Cases/states to be considered for RRM relaxation Agreement: Discuss the RAN4 requirements first for the following case #1, and FFS for case #2 to #5. RRM measurement case index; MR serving cell measurement; MR neighboring cell measurement; LR measurement. #1 Fully offloading case; Off; Off: FFS the condition and the details; ON. RAN4 to further discuss case #2 to #4: RRM measurement case index; MR serving cell measurement; MR neighboring cell measurement; LR measurement. #2 Relaxed case a; On with relaxation measurement; Off; ON. #3 Relaxed case b; On with relaxation measurement; On with relaxation measurement; ON. #4 Relaxed case c; Off; On, FFS the condition and the details; ON.

Since RAN2 is still discussing neighboring cell measurement details, it is better to wait RAN2 outcome to select case 2 to 4.

Proposal 2: It is better to wait RAN2 outcome of neighboring cell measurement to select case 2 to 4.

For MR RRM relaxation point of view, both serving and neighboring cell measurement should be relaxed to reduce UE power consumption. However the details especially for neighboring measurement relaxation should be waited as same reason as previous proposal.

Proposal 3: RAN4 specifies MR relaxation requirements for both serving cell and neighbor cell measurements at idle/inactive mode for UE supporting LP-WUR. The details are discussed according to other WGs outcomes.

- Conclusion

In this contribution, we propose possible RRM core requirements for LP-WUS/WUR

Proposal 1: At Rel-19 LP-WUR WI, for LP-WUR measurement, RAN4 specifies measurement requirements for the following:

- Measurement requirements for OOK based LP-WUR serving cell measurement based on LP-SS at Idle/Inactive state by considering:

  - Interruption time between LP-WUS received paging indication to MR ready for Paging monitoring, and MR going to be turned off

- Suspend the discussion of OFDM-based LP-WUR serving cell measurement requirement based on LP-SS at Idle/Inactive state until more RAN1 progress is available

- Other related requirements are FFS

Proposal 2: It is better to wait RAN2 outcome of neighboring cell measurement to select case 2 to 4.

Proposal 3: RAN4 specifies MR relaxation requirements for both serving cell and neighbor cell measurements at idle/inactive mode for UE supporting LP-WUR. The details are discussed according to other WGs outcomes.

References

[1] 3GPP, R4-2410296, “WF for RRM requirements for LP-WUS/WUR”, vivo