3GPP TSG-RAN WG4 Meeting #112	R4-2411684

Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:	8.24.2

Source: 		NTT DOCOMO, INC.

Title: 	Discussions on enabling transmission/reception in gaps/restrictions

Document for:	Discussion

- Introduction

-	Specify enhancements to enable transmission/reception in gaps/restrictions that are caused by RRM measurements (from inter-frequency RRM measurement gaps, or intra-frequency measurements, or other scheduling restrictions etc). [RAN1, RAN2, RAN4]

-	Specify the corresponding measurement gap and scheduling restriction to enable the identified enhancements with RRM performance impact taken into consideration, work being triggered by LS. [RAN4]

-	Specify Core requirements related to the above objectives as necessary [RAN4]

In RAN#102, new WI of XR (eXtended Reality) for NR Phase 3 was approved and other WGs have been discussed. The objectives related to RAN4 are copied from latest WID as follows [1].

This paper proposes how to consider RRM measurements in gaps/restrictions and possible affected spec. And also this paper shows our view on LS from RAN1 [2].

- Discussion

As copied above, NR XR Phase 3 considers RRM measurements in gaps/restrictions to minimize interruption. About measurement gap, RAN4 already specified several mechanisms to avoid scheduling restriction during measurement gap. Thus it is better to consider whether existing mechanisms, such as measurement without gaps, NCSG can be reused or not as a starting point.

Proposal 1: It is better to consider whether existing mechanisms, such as measurement without gaps, NCSG can be reused or not for transmission/reception in gaps as a starting point.

About scheduling restrictions other than that of caused by measurement gap, since there are many type of scheduling restrictions, it should be listed and classified whether it should be avoided or not.

Proposal 2: The existing scheduling restrictions should be listed and classified whether it should be avoided or not for XR.

Currently, RAN1 considers following schemes to enable transmission/reception in gaps/restrictions.

===

For solutions based on triggering/enabling by network signaling to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements consider the following alternatives or combinations for further down-selection:

- Alt. 1: Dynamic indication to enable Tx/Rx in particular gap(s)/restriction(s) that are caused by RRM measurements.

  - FFS: Alt 1-1: Explicit indication by DCI to skip a particular gap(s)/restriction(s);

    - Indication is included as part of scheduling DCI:

      - FFS: Bit-field size is one bit;

      - FFS: Bit-field size is >1 bit;

    - Note: Minimum time offset(s) between the end of [the first] received dynamic indication and start of corresponding gap(s)/restriction(s) occasion that is going to be skipped shall be introduced.

  - FFS: Alt 1-2: Explicit indication by DCI to indicate a time window where to skip a particular gap(s)/restriction(s);

    - Note: Minimum time offset between the end of received dynamic indication and start of gap(s)/restriction(s) occasion in time window that is going to be skipped shall be introduced.

  - FFS: Alt 1-3: Implicit indication by DCI scheduling a transmission/reception overlapping with a gap(s)/restriction(s) to skip the gap(s)/restriction(s);

    - Note: Minimum time offset between the end of received dynamic indication and start of gap(s)/restriction(s) occasion that is going to be skipped shall be introduced.

  - FFS: DCI format, DCI content, DCI bit-field size;

  - FFS: Whether indication is for one or more occasions;

  - FFS: How to consider time offset between the end of received dynamic indication and start of gap(s)/restriction(s) occasion that is going to be skipped.

- Alt. 3: Semi-static solution to enable TX/RX in gaps/restrictions that are caused by RRM measurements.

  - FFS: Alt 3-1: Configure a pattern(s) via RRC to indicate occasions where to skip gaps/restrictions;

    - FFS: Details of pattern:

      - FFS: Pattern is based on periodicity, offset and duration;

      - FFS: Pattern is based on a bitmap;

    - FFS: whether a pattern is applied to all or subset of configured MG configurations/scheduling restrictions.

  - FFS: Alt 3-3: Gaps/restrictions that are caused by RRM measurements are skipped if collided with particular semi-statically pre-configured Tx/Rx occasions.

  - FFS: Alt. 3-4: Gaps/restrictions that are caused by RRM measurements are skipped based on semi-statically configured priority information for particular semi-statically pre-configured Tx/Rx and/or particular gaps/restrictions.

===

Since there are still many FFS points in both Alt. 1 and Alt. 3, RAN4 should wait for RAN1 conclusions before going into details.

Proposal 3: Since there are still many FFS points in RAN1 discussions about how to enable Tx/Rx in gaps/restrictions, RAN4 should wait for RAN1 conclusions before going into details.

- LS reply

The incoming LS from RAN1 asks whether UE assistance information related to measurement occasions is needed or not. The candidates of measurement related information are copied below.

- Information about the maximum number of MGs/SMTC with restrictions that can be skipped within a time period.

- Information about the patterns of gap(s)/restriction(s) where skipping is feasible or acceptable.

Generally, these types of information are specified like minimum measurement samples or maximum duration between previous measurement as a minimum requirement in RAN4 RRM spec.

Observation 1: The candidates of measurement related information derived in LS are generally specified like minimum measurement samples or maximum duration between previous measurement as a minimum requirement in RAN4 RRM spec.

Therefore, no UE assistance information related to measurement occasion is needed.

Proposal 4: No UE assistance information related to measurement occasion is needed.

- Conclusion

In this contribution, we proposed our views on RRM measurements in gaps/restrictions.

Proposal 1: It is better to consider whether existing mechanisms, such as measurement without gaps, NCSG can be reused or not for transmission/reception in gaps as a starting point.

Proposal 2: The existing scheduling restrictions should be listed and classified whether it should be avoided or not for XR.

Proposal 3: Since there are still many FFS points in RAN1 discussions about how to enable Tx/Rx in gaps/restrictions, RAN4 should wait for RAN1 conclusions before going into details.

Observation 1: The candidates of measurement related information derived in LS are generally specified like minimum measurement samples or maximum duration between previous measurement as a minimum requirement in RAN4 RRM spec.

Proposal 4: No UE assistance information related to measurement occasion is needed.

References

[1] 3GPP, RP-2410791, “Revised WID on XR (eXtended Reality) for NR Phase 3”, Nokia

[2] 3GPP, R4-2411007, “LS on UE assistance information”, RAN1