3GPP TSG-RAN WG4 Meeting # 112	R4-2411686

# Maastricht, Netherlands, 19th – 23rd August, 2024

Source:	LG Electronics

Title:	Discussion on RRM requirements for NTN phase 3

Agenda item:	8.25.4

Document for:	Discussion

Introduction

In RAN#102, new WI for NTN phase 3 was agreed. RAN4 related objectives are listed below based on revised WID[1].

- Study and specify if beneficial downlink coverage enhancements targeting support for additional reference satellite payload parameters covering both GSO and NGSO constellations operating in FR1-NTN or FR2-NTN [RAN1, RAN2, RAN4] - Define additional reference satellite payload parameters assuming power sharing among satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint, such that satellite beams may not all be simultaneously active or may be active below the nominal EIRP density per satellite beam (see section 6.1.1 in TR 38.821) due to limited power and limited feeder link bandwidth. - Define the corresponding power sharing assumptions and necessary link level and system level evaluation methodology and relevant KPIs for evaluations of the coverage, to allow for identification of physical channels/signals and system-level aspects that need enhancements and the corresponding needed improvements. - Study and if needed specify solutions, including link level enhancements for FR1-NTN (e.g. for PDCCH, PDSCH) and/or system level enhancements for FR1-NTN and/or FR2-NTN, allowing dynamic and flexible power sharing between satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint. - RAN1 to report at the latest by RAN#106 with the list of targeted physical channels/signals for link level enhancements (if any), and with the targeted system-level enhancements (if any) - RAN1 should report on impact to backward compatibility, if any, for potential extension of the SSB periodicity at the latest by RAN#106, in conjunction with the targeted system-level enhancements. - Notes for this objective: - SSB channel enhancement other than SSB periodicity extension is not considered - RAN1 should consider issues such as UE’s cell search complexity and impact to initial cell selection, latency and success rate, for the above extension - The SSB periodicity enhancements potentially defined in this WID only apply to NTN operation - Antenna gain of UE shall be assumed to be -5.5dBi in case of smartphone in FR1-NTN, the UE is assumed to be a full duplex UE, and at least 2Rx are considered at the UE - NGSO to be considered in priority: LEO Set-1 @ 600 km - Rel-18 network energy saving techniques should be considered as baseline in the system level study - Uplink Capacity/Cell Throughput Enhancement for FR1-NTN [RAN1, RAN2, RAN4] - Specify Orthogonal Cover Codes (OCC) for DFT-s-OFDM PUSCH at least for multiplexing 2 or 4 UEs when PUSCH repetitions are used - Specify necessary signalling - Update RF requirements accordingly, if needed - Notes for this objective: - The enhancement is not targeting improvements/impacts of MU-MIMO capability - The enhancement is not targeted to PUSCH DMRS - No enhancement for initial access - Enhancements to PRACH are not in scope. - This feature may be applicable for UEs operating in terrestrial networks based on a common design - Support of regenerative payload [RAN3, RAN2, RAN4] - Specify the support of gNB on board in TS 38.300 - Specify, if needed, any necessary enhancements related to the intra and inter-gNB mobility, especially for Xn interface over feeder link or over ISL. [RAN3] - Note: if any additional necessary stage-3 specifications impact for e.g. NGAP is identified, RAN3 will handle it. - Support of Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1-NTN bands [RAN4, RAN1] - For full-duplex and half duplex FDD RedCap and eRedCap UEs, define the RF and RRM requirements [RAN4] - For HD-FDD RedCap UEs and eRedCap UEs, specify enhancements for mitigating issues caused by TA mismatch between actual TA used by the UE and assumed TA for the UE at the gNB [RAN1] - Enhancements are targeted for the following HD collisions cases (cases 3 and 4): - Semi-statically configured DL reception collides with semi-statically configured UL transmission - Dynamically scheduled DL reception collides with dynamic scheduled UL transmission - Note: enhancements for other HD collision cases are not targeted. Enhancements for HD collision cases 3 and 4 that may improve other HD collision cases are not precluded. - Note: potential work in RAN2, if any, starts after RAN1 has decided on the solution. - Notes for this objective: - GNSS (Global Navigation Satellite Systems) capabilities and simultaneous GNSS and NR-NTN operation is supported in RedCap/eRedCap UE.

In this contritubtion, we provide our views on each objective in terms of RRM requirements.

Discussion

Downlink coverage enhancements

The first objective is power sharing among satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint due to limited power and limited feeder link bandwidth. RAN1 is studying dynamic and flexible power sharing between satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint with SSB periodicity extension. Depending on RAN1 conclusion for solution for dynamic and flexible power sharing among satellite beams or different satellite beam patterns/size, RAN4 should check whether the outcome from RAN1 is affect RRM requirements.

- Proposal 1: After RAN1 concludes the solution for dynamic and flexible power sharing among satellite beams or different satellite beam patterns/size, RAN4 should check whether the outcome from RAN1 is affect RRM requirements.

Uplink Capacity/Cell Throughput Enhancement

This objective is for Orthogonal Cover Codes (OCC) for DFT-s-OFDM PUSCH when PUSCH repetitions are used. So, there would be no RRM related issues.

- Proposal 2: No RRM related issues on the objective for Uplink Capacity/Cell Throughput Enhancement

Regenerative payload

Based on the RRM requireiments in NTN feature, even regenerative payload is considered, RRM requirements would not different from that defined by Rel-17/18 considering that reference point is on a satellite.

- Proposal 3: No additional RRM requirements for regenerative payload are required

RedCap UEs with NR NTN operating

It was agreed to introduce NTN operation for Rel-17 and Rel-18 RedCap UEs based on the objective. For 2Rx RedCap UE, exsiting FR1 NTN RRM requirements can be resused. For 1Rx RedCap UE, new NTN RRM requirements should be defined, and the requirements could be based on 1Rx RedCap UE RRM requirements. In cell reselection in IDLE/INACTIVE state, eDRX_IDLE cycle configuration should be precluded for NTN RedCap requirements, and the same DRX cycle of exsiting NTN requirements can be considered. Our initial views on RRM requirements for RedCap NTN are below.

| RRM Category |  | Initial views |
| --- | --- | --- |
| Idle/inactive state mobility | Cell selection/re-selection | - Reuse the principle from legacy NTN cell selection/reselection for 2 Rx RedCap NTN - Add 1 Rx RedCap NTN requirements based on 1 Rx RedCap requirements - Preclude eDRX_IDLE cycle configuration |
| Connected state mobility | Handover | - Reuse the principle from legacy NTN cell selection/reselection for 2 Rx RedCap NTN - Add 1 Rx RedCap NTN requirements based on 1 Rx RedCap requirements - Add condition for RedCap UE with HD-FDD |
|  | Connection Mobility Control | - Reuse the principle from legacy NTN cell selection/reselection for 2 Rx RedCap NTN - Add 1 Rx RedCap NTN requirements based on 1 Rx RedCap requirements |
| Timing | Timing | - Reuse legacy NTN timing requirements |
| Signalling | RLM | - Reuse the principle from legacy NTN cell selection/reselection for 2 Rx RedCap NTN - Add 1 Rx RedCap NTN requirements based on 1 Rx RedCap requirements - Add condition for RedCap UE with HD-FDD |
|  | Link Recovery | - Reuse the principle from legacy NTN cell selection/reselection for 2 Rx RedCap NTN - Add 1 Rx RedCap NTN requirements based on 1 Rx RedCap requirements - Add condition for RedCap UE with HD-FDD |
|  | Active BWP switch delay | - Reuse legacy NTN active BWP switch delay requirements |
|  | Active TCI state switching delay | - Reuse legacy NTN active TCI state switch delay requirements |
|  | UE-specific CBW change | - Reuse legacy NTN active TCI state switch delay requirements |
| Measurement Procedure | NR intra-frequency measurements | - Reuse the principle from legacy NTN cell selection/reselection for 2 Rx RedCap NTN - Add 1 Rx RedCap NTN requirements based on 1 Rx RedCap requirements - Add condition for RedCap UE with HD-FDD |
|  | NR inter-frequency measurements |  |
|  | L1-RSRP Measurement |  |

- Proposal 4: For NTN RRM requirements for RedCap UE,

      - For 2Rx RedCap UE: Reuse the principle from legacy FR1 NTN RRM requirements

      - For 1Rx RedCap UE: Add new NTN RRM requirements should be defined, and the requirements could be based on 1Rx RedCap UE RRM requirements

      - For IDLE/INACTIVE mode: Preclude eDRX_IDLE cycle configuration for RedCap NTN

For HD-FDD, RAN1 is under discussion on TA mismatch between actual TA used by the UE and assumed TA, and the solution for TA mismatch would not be expected to have an impact on RAN4 requirements. Howerver, RAN4 needs to check whether the outcome from RAN1 is affect RRM requirements.

- Proposal 5: For HD-FDD, RAN4 needs to check whether the outcome from RAN1 is affect RRM requirements.

Conclusion

In this contribution, we provide views on each objective of the WID, and we propose

- Proposal 1: After RAN1 concludes the solution for dynamic and flexible power sharing among satellite beams or different satellite beam patterns/size, RAN4 should check whether the outcome from RAN1 is affect RRM requirements.

- Proposal 2: No RRM related issues on the objective for Uplink Capacity/Cell Throughput Enhancement

- Proposal 3: No additional RRM requirements for regenerative payload are required

- Proposal 4: For NTN RRM requirements for RedCap UE,

      - For 2Rx RedCap UE: Reuse the principle from legacy FR1 NTN RRM requirements

      - For 1Rx RedCap UE: Add new NTN RRM requirements should be defined, and the requirements could be based on 1Rx RedCap UE RRM requirements

      - For IDLE/INACTIVE mode: Preclude eDRX_IDLE cycle configuration for RedCap NTN

- Proposal 5: For HD-FDD, RAN4 needs to check whether the outcome from RAN1 is affect RRM requirements.

Reference

[1] RP-241667, “Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3,” Thales, CATT.