3GPP TSG-RAN WG4 Meeting # 112	R4-2411687

# Maastricht, Netherlands, 19th – 23rd August, 2024

Source:	LG Electronics

Title:	Discussion on RRM requirements for ATG CA operation

Agenda item:	8.10.4

Document for:	Discussion

Introduction

In agreed WID for ATG with CA operation [1], following objective was captured for RAN4 RRM core requirements

- Specify the RF and RRM core requirements for intra-band co-located and inter-band co-located DL CA [RAN4]: - FR1 intra-band contiguous CA - Example band combination: CA_n79C - FR1+FR1 inter-band CA - Example band combination: CA_n3A-n39A

In this contribution, we provide our views on RRM coer requirements for ATG with CA operation..

Discussion

Based on the objective in WID, CA operation in ATG scenario focuses on co-located case for intra- and inter-band DL CA. For CA operation, following additional requirmeents could be considered.

- MRTD (Maximum receive timing difference)

- SCell activation / deactivation delay

- Interruption at SCell activation / deactivation

- Interruption during measurement on deactivated SCell

- Measurement on deactivated SCell

For MRTD requirements, co-located CA operation is assumed for both intra- and inter-band. Existing MRTD for intra-band CA as 3us can be reused for ATG intra-band CA. For ATG inter-band CA, since co-located is considered, the same MRTD for intra-band CA could be applied. For inter-band CA, RAN4 needs to futher discuss whether co-location information should be provided to UE.

- Proposal 1: For MRTD requirements in ATG CA operation, existing MRTD for intra-band CA as 3us can be reusded for both ATG intra- and inter-band CA.

- Proposal 2: FFS whether co-location information in inter-band CA configuration should be provided to UE.

For SCell activation/deactivation delay, it can reuse the principle from the existing SCell activation / deactivation delay requirements as baseline for ATG. And scellWithoutSSB capability can be considered for the requirements. Since co-located based CA operation is assumed, only known SCell activation delay could be considered.

- Proposal 3: Reuse the principle from existing SCell activation / deactivation delay requirements as baseline for ATG, and scellWithoutSSB capability can be considered.

For interruption requirements, it can reuse the principle from the existing interruption requirements for SCell activation/deactivation and during measurement on deactivated SCell as baseline for ATG.

- Proposal 4: Reuse the principle from exsiting interruption requirements for SCell activation/deactivation and during measurement on deactivated SCell as baseline for ATG.

For deactivated SCell for cell identification and measurement period requirements, it can reuse the principle from the existing cell identification and measurement requirements with measCycleScell as baseline for ATG. For example, measurement period for intra-frequency measurements without gaps for deactivated SCell can be defined below.

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | ceil( 5 x Kp x N1 Note 1 x Klayer1_measurement) x measCycleSCell x CSSFintra |
| DRX cycle≤ 320ms | ceil(5 x Kp x N1 Note 1 x Klayer1_measurement) x max(measCycleSCell, 1.5 x DRX cycle)) x CSSFintra |
| DRX cycle>320ms | ceil( 5 x Kp x N1 Note 2 x Klayer1_measurement) x max(measCycleSCell, DRX cycle) x CSSFintra |
| NOTE 1: For ATG UE with the antenna array, N1 = 3 when network assistance on ATG cells reference locations is provided, otherwise N1 = 4. Otherwise, N1 = 1. |  |

- Proposal 5: Reuse the principle from the existing cell identification and measurement requirements with measCycleScell as baseline for ATG

In Rel-18 ATG RRM requirements, there are two antenna types such as omni-directional and array antenna, and different requirements have been defined. For CA operation for inter-band case, different antenna types can be assumed dependin on frequency band. RAN4 needs to clarify antenna type (i.e., the same or different) for inter-band CA is assumed to define requirements.

- Proposal 6: RAN4 needs to further discuss whether different antenna type for inter-band CA operatrion is considered.

Conclusion

In this contribution, we provide views on RRM requireiments for ATG with CA operation, and we propose

- Proposal 1: For MRTD requirements in ATG CA operation, existing MRTD for intra-band CA as 3us can be reusded for both ATG intra- and inter-band CA.

- Proposal 2: FFS whether co-location information in inter-band CA configuration should be provided to UE.

- Proposal 3: Reuse the principle from existing SCell activation / deactivation delay requirements as baseline for ATG, and scellWithoutSSB capability can be considered.

- Proposal 4: Reuse the principle from exsiting interruption requirements for SCell activation/deactivation and during measurement on deactivated SCell as baseline for ATG.

- Proposal 5: Reuse the principle from the existing cell identification and measurement requirements with measCycleScell as baseline for ATG

- Proposal 6: RAN4 needs to further discuss whether different antenna type for inter-band CA operatrion is considered.

Reference

[1] RP-241213, “Revised WID on Enhancements for Air-to-ground network for NR,” CMCC