3GPP TSG-RAN WG4 Meeting # 112	R4-2411688

# Maastricht, Netherlands, 19th – 23rd August, 2024

Source:	LG Electronics

Title:	Discussion on RRM requirements for FR2-1 L3 measurement delay reduction

Agenda item:	8.15.2.1

Document for:	Discussion

Introduction

In the last RAN4 meeting, RAN4 discussed and agreed on applicable scenario of L3 measurement delay reduction by optimizing Rx BSF, and other issues such as condition and scenario to use L3 measurement delay reduction were not much discussed. In this contritubtion, we provide our views on the issues for L3 measurement delay reduction listed in WF [1].

Discussion

Applicability requirement of L3 measurement delay reduction by optimizing Rx BSF

Applicability requirement

Agreement: Baseline: L3 delay enhancements in Rel-19 by optimizing Rx BSF for UE supporting multi-rx simultaneous reception are applicable provided that: - the target carrier(s) to be measured: only one carrier in the single FR2-1 band is configured for L3 SSB measurement and - UE serving carrier(s): UE is configured with single carrier on FR2-1 band, i.e. FR2-1 PCell without CA/DC. Note: The ‘other UE CA/DC modes (e.g., 1 or 2 FR2-1 bands CA, or FR1+FR2 CA/DC, or EN-DC)’ and/or the ‘other number of target to-be-measured carrier(s) on FR2-1 band’ can be FFS after concluding the baseline above. These extra FFS parts will NOT delay the WI completion.

Based on the agreements above, baseline scenario is that SSB based L3 measurement on only single carrier in FR2-1 band without CA/DC configuration. One thing to clarify the agreement is the single carrier to be measured for serving and target carrier can be intra or inter-frequency. So, RAN4 needs to focus on intra- / inter-frequency L3 measurement to reduce measurement delay by optiminzing Rx BSF first.

- Proposal 1: For the applicable scenario based on agreement of the last meeting,

Baseline: L3 delay enhancements in Rel-19 by optimizing Rx BSF for UE supporting multi-rx simultaneous reception are applicable provided that:

- the target carrier(s) to be measured: only one carrier in the single FR2-1 band is configured for L3 SSB measurement and

- UE serving carrier(s): UE is configured with single carrier on FR2-1 band, i.e. FR2-1 PCell without CA/DC.

Note: Target and serving carrier frequency can be the same or different.

Note: The ‘other UE CA/DC modes (e.g., 1 or 2 FR2-1 bands CA, or FR1+FR2 CA/DC, or EN-DC)’ and/or the ‘other number of target to-be-measured carrier(s) on FR2-1 band’ can be FFS after concluding the baseline above. These extra FFS parts will NOT delay the WI completion.

- Proposal 2: RAN4 to focus on SSB based intra- / inter-frequency L3 measurement to reduce measurement delay by optiminzing Rx BSF as first priority.

UE power class

L3 measurements requirements for FR2 have been defined by different scaling factor for each power class considering different number of Rx beam/antenna elemenents and no different UE behaviors were defined for each power class. Therefore, for supporting power class in Rel-19 RRM enh phase 5, the power class 3 can be first priority, but other power classes can generally apply the outcome of this WI discussion.

- Proposal 3: The supporting power class for L3 measurement delay enhancement with multi-Rx simultaneous reception is PC3 as first priority, but RAN4 should consider if other power classes could apply the outcome of the WI discussion.

Conditions to apply L3 measurement delay reduction by optimizing Rx BSF

Following conditions to apply L3 measurement delay enhancement by Rx BSF were provided in the last meeting.

WF for next meeting: - FFS: multi-Rx simultaneous reception of UE is in active mode, which is expected to follow the one specified in Rel-18 for multi-Rx simultaneous reception feature - Option 1: multi-Rx simultaneous reception of UE is in active mode, which is expected to follow the one specified in Rel-18 for multi-Rx simultaneous reception feature. - Option 1a: The UE is in multi-Rx operation if following condition is met: UE is configured with group-based beam reporting (GBBR) report - Option 1b: The UE is considered activated in multi-Rx simultaneous reception mode when the UE is configured with group-based beam reporting. The UE is considered activated for L3 reporting when the GBBR is configured not long prior to the expected L3 reporting. - Option 2: The L3 measurement delay enhancement requirements for UE supporting multi-Rx simultaneous reception can apply if multiple panels are activate and SSBs in a SMTC window can be measured with multiple beams. - Option 3: the conditions for UE to apply L3 measurement delay reduction by optimizing Rx BSF is that multi-Rx simultaneous reception of UE is in active mode, as for whether the condition is same as that for Rel-18 multi-Rx simultaneous reception can be further discussed. - Option 4: Do not reuse the same applicable conditions specified in Rel-18 multi-Rx - Option 4a : R19 Rx BSF is independent to R18 multi-Rx simultaneous reception feature includes conditions and requirement. - Option 5: The conditions in R18 multi-Rx WI do not limit the discussion on having further conditions in R19 for UE supporting multi-Rx to enhance FR2-1 SSB based L3 measurement delay

Rel-18 multi-Rx reception is for L1 measurement of intra-cell based multi-TRP scenario, but in Rel-19, the objective is to reduce L3 measurement delay by optimizing Rx BSF. The proposal of some companies in the previous meeting is that multi-Rx reception is when group-based beam reporting (GBBR) is configured. However, GBBR is for CSI / L1 measurement reporting, so it would not be directly related to L3 measurement reporting. Therefore, Rel-19 UE behaviour for multi-Rx reception does not have to depend on the condition for active mode of multi-Rx reception in Rel-18. The detailed condition could be further discussed when target scenario to use L3 measurement dealy reduction by Rx BSF is decided.

- Proposal 4: Rel-19 UE behaviour for multi-Rx reception does not have to depend on the condition for active mode of multi-Rx reception in Rel-18, and detailed condition could be further discuss based on agreed target scenario to use L3 measurement dealy reduction by Rx BSF.

For mobility status, there were proposals on mobility condition for multi-Rx reception. The L3 measurement delay reduction by optimizing Rx BSF can be applicable to both low and high mobility condition, so RAN4 does not need to consider mobility condition. For HST as UE power class 6, as mentioned above, power class 3 can be first priority.

- Proposal 5: Do not consider mobility condition for L3 measurement delay reduction by optimizing Rx BSF, and power class 3 can be first priority (i.e., power class 6 as HST can be further discussed later)

For power consumption issue, Rel-18 UAI ‘multiRx-PreferenceFR2’ for power saving can be considered as starting point.

- Proposal 6: For power consumption of multi-Rx operation, Rel-18 UAI ‘multiRx-PreferenceFR2’ for power saving can be considered as starting point.

Scenarios to use L3 measurement delay reduction by optimizing Rx BSF

WF for next meeting: [Moderator note]: The scenarios here means which UE behavior/activity/procedure(s) would be improved with this feature or which corresponding requirements in the existing RRM spec will be enhanced to accommodate this feature. FFS on the justifications to decide which of the following scenarios shall be considered. - Encourage companies to discuss the expected benefit to considering BSF reduction for the following scenarios. FFS on following scenarios to decide which ones are considered to use L3 measurement delay reduction by optimizing Rx BSF: - Scenario 1: SSB based Intra-frequency measurement without MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra - Scenario 2: SSB based Intra-frequency measurement with MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra - Scenario 3: SSB based Inter-frequency measurement without MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter - o Scenario 4: SSB based Inter-frequency measurement with MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter - Scenario 5: Handover - Scenario 6: PSCell addition - Scenario 7: RRC Re-establishment/RRC Connection Release with Redirection - Scenario 8: SCell activation - Scenario 9: SCG activation - Scenario 10: CGI identification - Scenario 11: CSI-RS based intra-/inter-frequency measurements, the CSI-RS is configured associatedSSB. The discussion on CSI-RS configured with associatedSSB could be revisited if SSB based L3 measurement delay reduction is concluded.

Many scenarios to apply L3 measurement delay reduction were proposed. Even if it is not clear which RRM requirements should be considered in WID, at least SSB based intra- / inter-frequency measurement with and without measurement gap in Scenario 1 to 4 should be considered as first priority, then other RRM requirements enhancement and related conditions could be investigated. The FR2-1 L3 measurement delay reduction would be required for supporting UE mobility when UE is experiencing lower signal quality to serving cell (i.e., cell edge region). Therefore, if UE supports multi-Rx reception simultaneously for L3 measurement and signal quality for serving cell is low, L3 measurement delay reduction by optimizing Rx BSF could be applied.

- Proposal 7: Scenario 1 to 4 as SSB based intra- / inter-frequency measurement should be considred as first priority.

- Proposal 8: If UE supports multi-Rx reception simultaneously for L3 measurement and signal quality for serving cell is low, L3 measurement delay reduction by optimizing Rx BSF could be applied to scenario 1 to 4.

The SSB based intra-frequency measurement requirements without gaps for FR2-1 in Table 1 has been defined considering UE Rx beam sweeping.

Table 1 Measurement period for intra-frequency measurements without gaps (FR2)

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | max(400ms, ceil(Mmeas_period_w/o_gaps x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 320ms | max(400ms, ceil(1.5x Mmeas_period_w/o_gaps x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | ceil(Mmeas_period_w/o_gaps xKp x Klayer1_measurement ) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

M values depending on power class are below.

Mmeas_period_w/o_gaps: For a UE supporting FR2-1 power class 1 or 5, Mmeas_period_w/o_gaps =40. For a UE supporting FR2-1 power class 2, Mmeas_period_w/o_gaps =24. For a UE supporting FR2-1 power class 3, Mmeas_period_w/o_gaps =24. For a UE supporting power class 4, Mmeas_period_w/o_gaps =24. For a UE supporting FR2-2 power class 1, Mmeas_period_w/o_gaps = 60. For a UE supporting FR2-2 power class 2, Mmeas_period_w/o_gaps = 36. For a UE supporting FR2-2 power class 3, Mmeas_period_w/o_gaps = 36.

For UE supporting FR2-1 power class 3, M=24, and it means that 4 measurement samples with 6 Rx beam sweeping or 3 measurement samples with 8 Rx beam sweeping. For example, for UE supporting multi-Rx simultaneous reception, the Rx beam sweeping can be smaller value such as 3 or 4, so M can be reduced as 12. RAN4 needs to discuss how to reduce M values for SSB based intra- / inter-frequency measurements for UE supporting multi-Rx simultaneous reception.

- Proposal 8-1: RAN4 to discuss how to reduce M values for SSB based intra- / inter-frequency measurements for UE supporting multi-Rx simultaneous reception

Common aspects for L3 measurement delay reduction

There is one issue whether previous release features shall be considered in FR2-1 SSB based L3 measurement delay reduction. If RAN4 considers combination of other features/previous release features and Rel-19 feature, there would be very complicated scenarios and conditions, and these could lead to unnecessary discussion such as corner cases. So, we prefer to focus on SSB based L3 measurement delay enhancement without considering other features and previous release features.

- Proposal 9: RAN4 not to consider SSB based L3 measurement delay enhancement with other features and previous release features.

Conclusion

In this contribution, we provide views on issues for FR2-1 SSB based L3 measurement delay reduction for UE supporting multiple-Rx simultaneous reception, and we propose

- Proposal 1: For the applicable scenario based on agreement of the last meeting,

Baseline: L3 delay enhancements in Rel-19 by optimizing Rx BSF for UE supporting multi-rx simultaneous reception are applicable provided that:

- the target carrier(s) to be measured: only one carrier in the single FR2-1 band is configured for L3 SSB measurement and

- UE serving carrier(s): UE is configured with single carrier on FR2-1 band, i.e. FR2-1 PCell without CA/DC.

Note: Target and serving carrier frequency can be the same or different.

Note: The ‘other UE CA/DC modes (e.g., 1 or 2 FR2-1 bands CA, or FR1+FR2 CA/DC, or EN-DC)’ and/or the ‘other number of target to-be-measured carrier(s) on FR2-1 band’ can be FFS after concluding the baseline above. These extra FFS parts will NOT delay the WI completion.

- Proposal 2: RAN4 to focus on SSB based intra- / inter-frequency L3 measurement to reduce measurement delay by optiminzing Rx BSF as first priority.

- Proposal 3: The supporting power class for L3 measurement delay enhancement with multi-Rx simultaneous reception is PC3 as first priority, but RAN4 should consider if other power classes could apply the outcome of the WI discussion.

- Proposal 4: Rel-19 UE behaviour for multi-Rx reception does not have to depend on the condition for active mode of multi-Rx reception in Rel-18, and detailed condition could be further discuss based on agreed target scenario to use L3 measurement dealy reduction by Rx BSF.

- Proposal 5: Do not consider mobility condition for L3 measurement delay reduction by optimizing Rx BSF, and power class 3 can be first priority (i.e., power class 6 as HST can be further discussed later)

- Proposal 6: For power consumption of multi-Rx operation, Rel-18 UAI ‘multiRx-PreferenceFR2’ for power saving can be considered as starting point.

- Proposal 7: Scenario 1 to 4 as SSB based intra- / inter-frequency measurement should be considred as first priority.

- Proposal 8: If UE supports multi-Rx reception simultaneously for L3 measurement and signal quality for serving cell is low, L3 measurement delay reduction by optimizing Rx BSF could be applied to scenario 1 to 4.

- Proposal 8-1: RAN4 to discuss how to reduce M values for SSB based intra- / inter-frequency measurements for UE supporting multi-Rx simultaneous reception

- Proposal 9: RAN4 not to consider SSB based L3 measurement delay enhancement with other features and previous release features.

Reference

[1] R4-2410260, “Way Forward for [111][228] NR_RRM_Ph5,” Apple.