3GPP TSG-RAN WG4 Meeting #112                                      R4-2411691

Masstricht, Neitherland, August 19st – August 25th, 2023

Source: 	Huawei, HiSilicon

Title: 	On general aspects of fragmented carriers

Agenda Item:	8.6.2

Document for:	Discussion

Introduction

The SI of NR FR1 DL Fragmented Carriers was approved at RAN#104 [1] with following objectives.

The objective of this study is as follows:

-	Identify methods for reducing the number of UE Rx chains (e.g. from separate RF chains per CC to shared RF chains) needed for single DL band with frequency span ≤ 100 MHz, containing two non-contiguous CCs within a CA combination for the inter-operator co-located scenario, considering:

-	Which RF requirements could be adjusted for the inter-operator co-located BS scenario, e.g. existing UE RF requirements such as ΔRIBNC, ACS and in-band blocking [RAN4];

- Other requirements are not precluded, if identified

-	The ability to semi-statically switch hardware resources (i.e. Rx chains) [RAN4, RAN2 – See note 2];

-	Up to 6 dB DL received power spectral density imbalance between the two non-contiguous CCs [RAN4];

-	Determine a reasonable level for the power spectral density difference between carriers of co-located adjacent channel operators for study [RAN4]

-	Impacts on DL performance [RAN4];

-	Means for a UE to inform the network of appropriate CA configuration it can support with adjusted RF requirements [RAN4, RAN2 – See note 2].

NOTE 1:	No RAN1 impact is foreseen

NOTE 2:	RAN2 work, if necessary, will be triggered by RAN4 LS

NOTE 3:	This study starts from single DL band. Sharing RF chain is not considered among inter-band DL carriers.

- Whether and how to apply the conclusions to intra-band component of inter-band CA will be decided after the study on 2CC in a single DL band is completed.

In this contribution, we would like to discuss some general aspects related to this feature.

Discussion

Reference RF architecture

The motivation of current study item is to reduce the number of UE Rx chains that are used to receive the fragmented carriers. To serve this motivation, different RF architectures were mentioned during early discussions, e.g. from separate RF chains per CC to shared RF chains.

The reference RF architecture of non-contiguous CA was initially introduced in LTE Rel-11, and captured in TR 36.823 as below.

Figure 1	Reference RF architecture of non-contiguous CA (Figure 6.2.3.1-1 of TR 36.823)

According to this reference RF architecture, the LNA is shared by two non-contiguous carriers, while the mixer, LPF and A/D are independent for each carrier. This reference RF architecture continues to be used in deriving the NR requirements for non-contiguous CA. Therefore, the current RF receiving requirements in TS 38.101-1 for non-contiguous CA are based on this partially shared architecture.

Observation 1: The current RF receiving requirements in TS 38.101-1 for non-contiguous CA are based on the partially shared architecture. i.e. the antenna and LNA are shared by both carriers.

Comparing with the fully separated RF architecture, the partially shared architecture saves the cost and on-chip space of antenna and LNA, therefore could be considered as one option of handling the fragmented carriers. The existing requirements could be referred to and no extra requirements need to be studied.

To further reduce the Rx chains, the fully shared architecture is worth thorough study. Below figure is the comparison of fully shared, partially shared and separated architectures. We could tell from the figure that the fully shared architecture could handle the non-contiguous CCs with less Rx chain, or provide additional DL MIMO layer capability with the same number of Rx chains.

Figure 2	Comparison of Fully Shared, Partially Shared and Separated RF architectures

Observation 2: Comparing with partially shared or separated architecture, the fully shared architecture could handle the non-contiguous CCs with less Rx chain, or provide additional DL MIMO layer capability with the same number of Rx chains.

On the other hand, the fully shared architecture will face extra challenges due to shared LNA, LPF and A/D. The pass-band of LPF increases to receive the non-contiguous carriers simultaneously, therefore the out-band rejection might be worse. In FDD bands, it means the Tx leakage will have more impact on the receiver performance, and leads to larger REFSENS relaxation. In addition, the ACS and IBB blocker falling in between the non-contiguous carries can’t be filtered out by the LPF, which might cause the switching of AGC state, and lead to performance degradation.

Observation 3: The RF receiving performance might be degraded with the fully shared architecture.

Proposal 1: Current study item focuses on the fully shared architecture to identify what the major impacts on the RF requirements are, and how much the impacts could be.

Inter-operator collocated scenario

In the objective of current SI, the inter-operator deployment is limited to co-located scenario, while the PSD difference between operators is for further study.

Currently there is already common assumption for co-located scenario between non-contiguous carriers from single operator, i.e. the PSD difference is less than 6dB, and the MRTD is less than 3us. The assumption is based on the understanding of the non-contiguous carriers are transmitted from the same gNB.

Regarding the inter-operator deployment, it’s very difficult to ensure different operators are always co-located. Even in some cases the operators share the same site, separate infrastructures could still be used, e.g. separate RRUs are used. As a result, the coverage of different operators is not fully aligned.

Observation 4: Regarding inter-operator deployment scenario, it’s not clear whether the SI objective is limited to shared RRU case, or separate RRU case is also included.

If the SI objective is limited to shared RRU case, the applicable scenario will be severely constrained. The separate RRU case is more realistic scenario and should be considered.

Proposal 2: For inter-operator deployment scenario, both shared RRU case and separate RRU case will be studied.

Conclusion

In this contribution we discussed the general aspects of the fragmented carriers. According to the analysis, we have the following observations and proposals:

Observation 1: The current RF receiving requirements in TS 38.101-1 for non-contiguous CA are based on the partially shared architecture. i.e. the antenna and LNA are shared by both carriers.

Observation 2: Comparing with partially shared or separated architecture, the fully shared architecture could handle the non-contiguous CCs with less Rx chain, or provide additional DL MIMO layer capability with the same number of Rx chains.

Observation 3: The RF receiving performance might be degraded with the fully shared architecture.

Proposal 1: Current study item focuses on the fully shared architecture to identify what the major impacts on the RF requirements are, and how much the impacts could be.

Observation 4: Regarding inter-operator deployment scenario, it’s not clear whether the SI objective is limited to shared RRU case, or separate RRU case is also included.

Proposal 2: For inter-operator deployment scenario, both shared RRU case and separate RRU case are considered.

- References

- RP-241669, “New SID: Study on NR FR1 DL Fragmented Carriers”, MediaTek Inc, RAN#104 Meeting, June 2024