3GPP TSG-RAN WG4 Meeting #112	R4-2411693

# Maastricht, Netherlands, 19th – 23rd August, 2024

Title:	Discussion on MPR reduction for FR2 CA

Source:	Samsung

Agenda item:	8.1.1.2.3

Document for:	Discussion

1.	Introduction

According to the approved WF [1, R4-2410749], most of work on MPR enhancement for FR2 CA has been done:

2.3. Applicable MPR for FR2 single carrier UL with DL intra band CA Way forward: - For the case of single carrier UL with DL intra band CA, the MPR requirements of single carrier case in clause 6.2.2 of TS 38.101-2 applies with UE indication of new capability for MPR improvement. 2.4. Applicable MPR for FR2 UL CA with DL intra band CA Way forward: - MPR based on UL BWchannel_CA applies instead that based on cumulative aggregated channel BW (CABW) with UE indication of new capability for MPR improvement - If only 1 UL CC is activated, the MPR requirements of single carrier could be reused - FFS whether new MPR requirement could be defined for CABW < 400 MHz (e.g., 200 MHz).

Next we can focus on how to change the spec and design the new UE capability.

One more remained issue is FFS whether new MPR requirement could be defined for CABW < 400 MHz (e.g., 200 MHz).

In this contribution, we further discuss the above issues.

2. 	Discussion

For the case of single carrier UL with DL intra band CA, it is noticed that in the WID [2, RP-240828] the single uplink case is limited to intra-band DL contiguous CA only.

  - Include intra-band UL contiguous CA and intra-band DL contiguous CA with single UL for FR2

In previous RAN4 discussion, it seems that the FR2 MPR enhancement for the single UL case is assumed to be also applicable for intra-band DL non-contiguous CA with single UL.

Proposal 1:	RAN4 to clarify whether FR2 MPR enhancement for the single UL case is also applicable for intra-band DL non-contiguous CA with single UL.

For the case of intra-band UL CA with DL intra band CA, it is agreed to apply the MPR requirements based on UL BWchannel_CA instead of legacy CABW, with UE indication of new capability for MPR improvement. However, it is still not very clear the new MPR applicability is configuration based or CC activation based, as mentioned in [3, R4-2406584]

  - intra-band UL CA with DL intra band CA (configuration based)

  - intra-band UL CA with DL intra band CA (CC activation based)

It is not very clear what the difference is in architecture or UE design between ‘configuration based’ and ‘CC activation based’ assuming the UE is already equipped with distinct LO between RX and TX. In previous meeting, there is proposal to enhance FR2 MPR with two levels, 1st level is for UE supporting MPR reduction based on CC configuration, 2nd level is for more advanced UE supporting MPR reduction based on CC activation. Assuming UE is equipped with distinct LO between RX and TX, it is necessary to make clear what the difficulty is to support ‘CC activation based’ MPR directly.

Observation 1:	if MPR is to be enhanced with two separate levels for ‘configuration based’ and ‘CC activation based’ respectively, clarification is needed in terms of UE architecture or functionality difference in design

Moreover, in the WID it is explicitly mentioned that the MPR applicability is activation based than configuration based:

- Specify MPR applicability based on the UL CCs with activated cells for NR intra-band UL CA configuration - Include both intra-band UL contiguous CA and intra-band non-contiguous UL CA for FR1 - Include intra-band UL contiguous CA and intra-band DL contiguous CA with single UL for FR2 - MPR requirement is not applicable until the SCell is activated

Proposal 2:	RAN4 to focus on activation based MPR applicability. If configuration based MPR applicability is also considered, more clarification is needed.

For the case of intra-band UL CA with DL intra band CA, one more issue for further study is about the BW granularity. Now the granularity for MPR table is at least 400MHz, as indicated in Table 6.2A.2.4-1 of TS 38.101-2

Table 6.2A.2.4-1: Maximum power reduction (MPRC_CA) for UE power class 3 in FR2-1

|  |  | Cumulative aggregated channel bandwidth (CABW) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | ≤ 400 MHz | > 400 MHz and < 800 MHz | ≥ 800 MHz and ≤ 1400 MHz | > 1400 MHz and ≤ 2400 MHz |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 5.01 | ≤ 7.7 | ≤ 8.2 | ≤ 8.7 |
|  | QPSK | ≤ 5.01 | ≤ 7.7 | ≤ 8.2 | ≤ 9.7 |
|  | 16 QAM | ≤ 6.5 | ≤ 8.7 | ≤ 9.3 | ≤ 9.7 |
|  | 64 QAM | ≤ 9.0 | ≤ 10.7 | ≤ 11.2 | ≤ 11.7 |
| CP-OFDM | QPSK | ≤ 5.0 | ≤ 7.5 | ≤ 8.0 | ≤ 9.7 |
|  | 16 QAM | ≤ 6.5 | ≤ 8.7 | ≤ 9.2 | ≤ 9.7 |
|  | 64 QAM | ≤ 9.0 | ≤ 10.7 | ≤ 11.2 | ≤ 11.7 |
| NOTE 1: (Void). |  |  |  |  |  |

But for BW   400MHz, it is further divided into 200MHz granularity for DFT-s-BPSK or DFT-s-QPSK

In case of a contiguous RB, DFT-s-BPSK or DFT-s-QPSK UL allocation in a single CC of a CA configuration with contiguous CCs, and whose cumulative aggregated BW 400 MHz, MPRC_CA shall be derived instead as MAX(MPR1, MPR2), where: MPR1 shall be determined from Table 6.2.2.3-1 if CABW 200 MHz, from Table 6.2.2.3-2 if CABW > 200 MHz. MPR2 shall be determined from Table 6.2.2.3-1 if UL BWchannel_CA 200 MHz, from Table 6.2.2.3-2 if UL BWchannel_CA > 200 MHz.

Observation 2:	the BW granularity of MPR applicability is at least 400MHz for BW ≥ 400MHz, and 200MHz granularity is already considered in existing spec for DFT-s-BPSK or DFT-s-QPSK where MPRC_CA = MAX(MPR1, MPR2)

Given 200MHz BW granularity is already considered, it seems not necessary to further define new MPR for CABW <400MHz. What we need to do is to change the MPR calculation from MPRC_CA = MAX(MPR1, MPR2) to MPRC_CA = MPR2, for UE with UE indication of new capability for MPR improvement.

Proposal 3:	RAN4 not to specify new column for <400MHz CABW in the MPR table, but to change the MPR calculation for DFT-s-BPSK or DFT-s-QPSK from “MPRC_CA = MAX(MPR1, MPR2)” to “MPRC_CA = MPR2”, for UE with UE indication of new capability for MPR improvement..

3. 	Conclusion

Observation 1:	Enhanced UEs with new FR2 MPR reduction UE capability is assumed with distinct LO between RX and TX.

Proposal 1:	For the case of single carrier UL with DL intra band CA, the MPR requirements of single carrier case in clause 6.2.2 of TS 38.101-2 applies for UE supporting the new capability.

Proposal 2:	RAN4 to clarify whether FR2 MPR enhancement for the single UL case is also applicable for intra-band DL non-contiguous CA with single UL.

Observation 2:	if MPR is to be enhanced with two separate levels for ‘configuration based’ and ‘CC activation based’ respectively, clarification is needed in terms of UE architecture or functionality difference in design

4. 	References

- R4-2410749     “WF on power domain enhancements”, Huawei, HiSilicon

- RP-240828    “New WID: UE RF enhancements for NR FR1 and FR2, Phase 4”, RAN4 chair (Huawei)

- R4-2406584    “WF on power domain enhancements”, Huawei, HiSilicon