3GPP TSG-RAN WG4 Meeting #112	R4-2411694

# Maastricht, Netherlands, 19th – 23rd August, 2024

Title:	Discussion on UE RF requirements for LP-WUR

Source:	Samsung

Agenda item:	8.22.2.2

Document for:	Discussion

1.	Introduction

In RAN4#111 meeting, the performance metric for LP-WUR RX RF requirements has been agreed as MDR (FAR will be considered separately in demodulation) [1, R4-2410569].

Issue 1-1-11: Metric for LP_WUR requirements Agreements: - MDR is used as the metric for LP_WUR Rx RF requirements - FFS MDR values. - FAR will be defined as demodulation requirements - FFS FAR values.

In this contribution, we share our views on MDR values and further discuss some aspects on specifying REFSENS requirements considering both coverage target and UE implementation.

2. 	Discussion

Instead of 95% throughput, X% missed detection rate is agreed as performance metric for LP-WUS UE RF requirements. Now in RAN4 simulation both 1% and 5% are considered, while in RAN1 simulation only 1% is adopted. Missed detection is critical for the whole communication system. The 5% option was proposed for the sake of saving test time. However, 5% missed detection rate is not affordable, and it is difficult to be considered in-coverage when the MDR is up to 5%. It is better to align with RAN1 to adopt 1% MDR as performance metric.

Proposal 1:	RAN4 to adopt 1% MDR as performance metric for LP-WUR RX RF requirements

Regarding REFSENS, it was agreed to derive REFSENS reusing legacy approach based on SNR, NF, IM of receiver, and further study whether to design REFSENS requirements or other requirements to ensure LP-WUR meet the coverage target [2, R4-2406619].

Issue 2-2-2: How to specify REFSENS requirements Agreement: - Reuse legacy approach to derive REFSENS, further discuss SNR, NF, IM - FFS whether to design REFSENS requirements or other requirements to ensure LP-WUR meet the coverage target - Side condition for REFSENS test: DL test signal will only have LP-WUS signal.

Coverage target is one of the WID objective, i.e., the target coverage of LP-WUS and LP-SS shall be the coverage of PUSCH for message3. Now RAN1 is evaluating the SNR value to achieve the coverage target. Note that the coverage target is not only achieved by REFSENS performance but also signal repetition schemes, because the REFSENS test condition has no signal repetition. On the other hand, NF and IM are supposed to reflect UE architecture and implementation.

Proposal 2:	The SNR value corresponding to 1% MDR for REFSENS derivation is supposed to partially reflect coverage target, while NF and IM are supposed to reflect UE architecture and implementation.

For NF, the performance gap between envelop based architecture and OFDM based architecture are quite different. If there is only one set of REFSENS requirements which is minimum based on envelop receiver, such requirement would almost have no restriction to OFDM based receiver. Given the LP-WUS coverage for LR is worse than NR coverage for NR, it is beneficial to specify separated REFSENS requirements between envelop based receiver and OFDM based receiver.

Proposal 3:	Better to consider different NF for envelop based receiver and OFDM based receiver, i.e., two sets of REFSENS requirements.

According to the outcome of Rel-18 study on LP-WUS/WUR in TR 38.869, there are three kinds of receive architectures with envelope detection:

-	Architecture with RF envelope detection

-	Heterodyne architecture with IF envelope detection

-	Homodyne/zero-IF architecture with baseband envelope detection

Architecture with RF envelope detection could not satisfy the WID objectives in terms of multi-band support and target coverage, IF envelop detection has good performance but requires more components in hardware, zero-IF architecture with baseband envelope detection seems the best choice as baseline for envelop based receiver.

Proposal 4:	RAN4 to take zero-IF architecture with baseband envelope detection as baseline for envelop based receiver.

For IM, it is related with detailed implementation. But considering the UE type supporting LP-WUS is usually of small size, it is highly possible that the antenna and RF front end of LP-WUR are shared from MR (Main Radio). The additional insertion loss caused by the antenna sharing and switching between LR and MR should be considered in the IM.

Proposal 5:	RAN4 to take antenna sharing and switching as baseline for RF front end, and additional insertion loss should be considered in the IM value for REFSENS derivation.

3. 	Conclusion

Proposal 1:	RAN4 to adopt 1% MDR as performance metric for LP-WUR RX RF requirements

Proposal 2:	The SNR value corresponding to 1% MDR for REFSENS derivation is supposed to partially reflect coverage target, while NF and IM are supposed to reflect UE architecture and implementation.

Proposal 3:	Better to consider different NF for envelop based receiver and OFDM based receiver, i.e., two sets of REFSENS requirements.

Proposal 4:	RAN4 to take zero-IF architecture with baseband envelope detection as baseline for envelop based receiver.

Proposal 5:	RAN4 to take antenna sharing and switching as baseline for RF front end, and additional insertion loss should be considered in the IM value for REFSENS derivation.

4. 	References

- R4-2410569    “WF for [111][136] NR_LPWUS_UERF”, vivo

- R4-2406619    “WF on NR LP-WUS UE requirements”, vivo