3GPP TSG-RAN WG4 Meeting #112	R4-2411695

# Maastricht, Netherlands, 19th – 23rd August, 2024

Title:	On XR UE OTA test

Source:	Samsung

Agenda item:	8.12.2.1

Document for:	Discussion

1.	Introduction

In last RAN4 meeting, the XR device type was discussed and further study is expected [1, R4-2409931]:

Issue 2-1-1: XR device type Agreements FFS The defined test methodologies of TRP/TRS for FR1 non-Redcap XR devices shall be applied to all possible 5G XR device types except the XR device form-factors based on smartphone UE i.e. XR5G-P1. RAN4 further discuss prioritization of XR devices type, e.g., - Whether 1Tx XR device should be prioritized in Rel-19 - whether head-worn XR devices with external 5G (e.g. XR5G-V1/V2/V3, XR5G-A1/A2/A5) should be considered.

In this contribution, we share our views on the XR device types.

2. 	Discussion

Regarding the prioritization between 1Tx and 2Tx XR devices, though 2Tx XR devices are in scope of Rel-19 OTA WI objectives, it is better to take a step by step way for the test method of new device types. So it is proposed to prioritize 1Tx XR devices firstly.

Proposal 1:	Prioritize 1Tx XR devices than 2Tx in Rel-19

In our previous contribution [2, R4-2407896], it is noticed that some 5G XR device types are actually with external 5G modem. In our view the DUT of head-worn XR should be the helmet or eyeglasses. So we should focus on the head-worn XR devices with 5G modem inside.

It can be also noticed that some 5G XR types can use 5G sidelink as tethering. For such XR device types, there are 5G sidelink inside the helmet or eyeglasses, though the 5G modem is external. However, the OTA performance of 5G sidelink as tethering is not so critical compared with XR devices with internal 5G modem.

So we propose to at least prioritize the head-worn XR devices with 5G modem inside, i.e. XR5G-V4, XR5G-A3 and XR5G-A4.

Proposal 2:	prioritize the head-worn XR devices with 5G modem inside, i.e. XR5G-V4, XR5G-A3 and XR5G-A4. FFS if head-worn XR devices with 5G sidelink for tethering should be considered.

Table 1: XR Device Types (Table 4.8-1 of TR26.928)

| XR Type Number | XR Device Type Name | Tethering Examples | 5G Uu Modem | XR Engine Localization | Power Supply | Typical Max Avail Power |
| --- | --- | --- | --- | --- | --- | --- |
| XR5G-P1 | Phone | n/a | XR device | XR device or split | Internal | 3-5 W |
| XR5G-V1 | Simple VR Viewer wired tethering | USB-C | External | External | External | 2-5 W |
| XR5G-V2 | Simple VR Viewer wireless tethering | 802.11ad/y, 5G sidelink, etc. | External | External | Internal | 2-3 W |
| XR5G-V3 | Smart VR Viewer wireless tethering | 802.11ad/y, 5G sidelink, etc. | External | XR device or Split | Internal | 2-3 W |
| XR5G-V4 | VR HMD Standalone | n/a | XR device | XR device or Split | Internal | 3-7 W |
| XR5G-A1 | Simple AR Wearable Glass wired tethering | USB-C | External | External | External | 1-3 W |
| XR5G-A2 | Simple AR Wearable Glass wireless tethering | 802.11ad/y, 5G sidelink. etc. | External | External | Internal | 0.5 – 2 W |
| XR5G-A3 | Smart AR HMD see-through standalone | n/a | XR device | XR device or Split | Internal | 3-7 W |
| XR5G-A4 | AR Wearable Glass standalone | n/a | XR device | XR device or Split | Internal | 2 - 4 W |
| XR5G-A5 | Smart AR Wearable Glass wireless tethering | 802.11ad/y, 5G sidelink. etc. | External | XR device or Split | Internal | 0.5 – 2 W |

3. 	Conclusion

Proposal 1:	Prioritize 1Tx XR devices than 2Tx in Rel-19

Proposal 2:	prioritize the head-worn XR devices with 5G modem inside, i.e. XR5G-V4, XR5G-A3 and XR5G-A4. FFS if head-worn XR devices with 5G sidelink for tethering should be considered.

4. 	References

- R4-2409931    “WF for [111][338] TRP_TRS_MIMO_OTA”, vivo

- R4-2407896    “Discussion on XR UE OTA test”, Samsung