3GPP TSG-RAN WG4 Meeting #112	R4-2411696

# Maastricht, Netherlands, 19th – 23rd August, 2024

Title:	On NTN UE OTA test

Source:	Samsung

Agenda item:	8.12.2.2

Document for:	Discussion

1.	Introduction

In last RAN4 meeting, the usage scenarios and performance metric for NTN UE OTA test have been extensively discussed but no conclusion yet. In this contribution, we further discuss usage scenarios and performance metric.

2. 	Discussion

For usage scenarios, the pure free space option was excluded and other options on the table are as following [1, R4-2409931]

Issue 3-1-6: Usage scenarios for NR-NTN and IoT-NTN handheld UE Agreements - RAN4 further discuss prioritized usage scenarios. Free space only is not considered. - Option 1: Prioritize the browsing mode (with hand phantom) - Option 2: Prioritize the browsing mode (with hand phantom) and Free Space - Option 3: Prioritize browsing mode (with hand phantom) and talk mode (Head+Hand) - Option 4: Prioritize Head+Hand talk mode, hand only browsing mode and talk mode (new positioning guideline), and Free Space - Option 5: Prioritize Free Space mode only

It is common understanding that NTN communication is important for emergency and life critical scenarios, users tend to avoid talk mode for better satellite signal, instead, to perform voice call under free space or browsing mode via loud speaker. For many commercial mobile phones supporting NTN communications, there is often software assistant function to guide user how to hold the UE, obviously talk mode is not applicable for such cases.

Moreover, there would be trade-off between different scenarios in antenna design. If antenna for NTN is designed for balanced performance for both talk mode and browse mode, the OTA performance for browse mode which is the most important scenario would not be optimal.

Based on above observations, the browsing mode (with hand phantom) should be prioritized for NTN UE OTA testing.

Proposal 1:	the browsing mode (with hand phantom) should be prioritized for NTN UE OTA testing.

For performance metrics, the single point metric i.e. peak EIRP/EIS was excluded and other options on the table are as following [1, R4-2409931]

Issue 3-2-1: Categorized performance metric for NR-NTN handheld Agreements - RAN4 should not adopt direction 1, other directions are FFS - Direction 1: Only single point performance metric, i.e., Peak EIRP/EIS - Direction 2: Single point + measured full sphere (integrated or CDF), e.g., - D2a, Peak EIRP/EIS+TRP/TRS, - D2b, Peak EIRP/EIS+ full Spherical coverage CDF - Direction 3: Single point + measured partial sphere (integrated or CDF), e.g., - D3a, Peak EIRP/EIS+ partial TRP/TRS (e.g., within selected 30, 60, 90, 180 degree range of angles), - D3b, Peak EIRP/EIS+ partial Spherical coverage CDF (e.g., within selected 30, 60, 90, 180 degree range of angles) - Direction 4: Only Full sphere (integrated TRP/TRS or Spherical coverage CDF) with/without weighting, - D4a, Spherical Coverage with EIRP-CDF and EIS-CCDF at [>50%]-ile - D4b, TRP/TRS or Weighted metric, e.g., weighted TRP/TRS (with larger weight for declared hemisphere) - Direction 5: Only Partial sphere (integrated or CDF) (e.g., within selected 30, 60, 90, 180 degree range of angles),

The peak EIRP/EIS metric is usually used for UE with high antenna gain, e.g. FR2 UE with beamforming. For FR1 UE, the antenna radiation pattern is not so directional, and the antenna gain is not so high. It is not appropriate to adopt peak EIRP/EIS metric for FR1 UE.

For FR1 UE antenna, it often receives signals from different directions at the same time. Even for the lower hemisphere, there is possible incident satellite signals due to signal reflection and scattering. So the performance metric should consider all directions in full sphere. It is probably the upper hemisphere includes more incident satellite signal than the lower sphere, different weights for upper hemisphere and lower hemisphere can be considered.

Based on above considerations, we support the Direction 4 with TRP TRS metric from full sphere, and larger weight can be put on the declared hemisphere, e.g., upper hemisphere.

Proposal 2:	Adopt full sphere TRP TRS as performance metric for NTN handheld UE, and larger weights and be put on the declared hemisphere, e.g., upper hemisphere, in the TRP TRS integration.

3. 	Conclusion

Proposal 1:	the browsing mode (with hand phantom) should be prioritized for NTN UE OTA testing.

Proposal 2:	Adopt full sphere TRP TRS as performance metric for NTN handheld UE, and larger weights and be put on the declared hemisphere, e.g., upper hemisphere, in the TRP TRS integration.

4. 	References

- R4-2409931    “WF for [111][338] TRP_TRS_MIMO_OTA”, vivo