3GPP TSG-RAN WG4 Meeting #112	R4-2411697

# Maastricht, Netherlands, 19th – 23rd August, 2024

Title:	On FR1 dynamic MIMO OTA

Source:	Samsung

Agenda item:	8.12.2.3

Document for:	Discussion

1.	Introduction

In last RAN4 meeting, RAN4 has agreed to adopt dynamic link adaption for dynamic MIMO OTA [1, R4-2409931]:

Issue 4-2-2: Link adaption configuration for Dynamic FR1 MIMO OTA Agreements - RAN4 adopt dynamic link adaptation. FFS details, e.g., - mapping tables between CQI and MCS for available rank values

In this contribution, further discussion on detailed configurations e.g. mapping table between CQI and MCS are provided.

2. 	Discussion

Dynamic link adaption includes both MCS adaption and rank adaption.

For MCS adaption, network dynamically change the MCS configuration based on UE’s CQI reporting. In order to enable dynamic MIMO OTA testing, the mapping rule between UE reported CQI and TE configured MCS is needed. On this aspect, the mapping table used for demodulation test in TS 38.101-4 can be a reference.

E.g., the mapping between CQI and MCS for MCS table 64QAM and MCS table 256QAM from A.4 of TS 38.101-4 are following respectively:

| CQI index | Spectral efficiency | MCS index | Modulation |
| --- | --- | --- | --- |
| 0 | OOR | OOR | OOR |
| 1 | 0.2344 | 0 | QPSK |
| 2 | 0.2344 | 0 |  |
| 3 | 0.3770 | 2 |  |
| 4 | 0.6016 | 4 |  |
| 5 | 0.8770 | 6 |  |
| 6 | 1.1758 | 8 |  |
| 7 | 1.4766 | 11 | 16QAM |
| 8 | 1.9141 | 13 |  |
| 9 | 2.4063 | 15 |  |
| 10 | 2.7305 | 18 | 64QAM |
| 11 | 3.3223 | 20 |  |
| 12 | 3.9023 | 22 |  |
| 13 | 4.5234 | 24 |  |
| 14 | 5.1152 | 26 |  |
| 15 | 5.5547 | 28 |  |

| CQI index | Spectral efficiency | MCS index | Modulation |
| --- | --- | --- | --- |
| 0 | OOR | OOR | OOR |
| 1 | 0.2344 | 0 | QPSK |
| 2 | 0.3770 | 1 |  |
| 3 | 0.8770 | 3 |  |
| 4 | 1.4766 | 5 | 16QAM |
| 5 | 1.9141 | 7 |  |
| 6 | 2.4063 | 9 |  |
| 7 | 2.7305 | 11 | 64QAM |
| 8 | 3.3223 | 13 |  |
| 9 | 3.9023 | 15 |  |
| 10 | 4.5234 | 17 |  |
| 11 | 5.1152 | 19 |  |
| 12 | 5.5547 | 21 | 256QAM |
| 13 | 6.2266 | 23 |  |
| 14 | 6.9141 | 25 |  |
| 15 | 7.4063 | 27 |  |

Proposal 1:	take the mapping table between CQI and MCS used for demodulation in TS 38.101-4 as reference for dynamic MCS adaption.

For rank adaption, TE can directly follow the UE reported Rank Indicator (RI) for dynamic MIMO OTA testing.

Proposal 2:	TE directly follow the UE reported Rank Indicator (RI) for rank adaption in dynamic MIMO OTA testing.

3. 	Conclusion

Proposal 1:	take the mapping table between CQI and MCS used for demodulation in TS 38.101-4 as reference for dynamic MCS adaption.

Proposal 2:	TE directly follow the UE reported Rank Indicator (RI) for rank adaption in dynamic MIMO OTA testing.

4. 	References

- R4-2409931    “WF for [111][338] TRP_TRS_MIMO_OTA”, vivo