3GPP TSG-RAN WG4 Meeting #112	R4-2411698

# Maastricht, Netherlands, 19th – 23rd August, 2024

Title:	On FR2 OTA testing of STxMP

Source:	Samsung

Agenda item:	8.13.2

Document for:	Discussion

1.	Introduction

In this contribution, discussions on how to measure/distinguish EIRP per TCI, AoA separation and UE orientation, test skipping rules, etc. are presented based on the progress of last meeting.

2. 	Discussion

Regarding how to measure/distinguish EIRP per TCI, it was agreed to conclude in this meeting [1, R4-2409921]

Issue 1-1-1: Signal level condition for measuring/distinguishing EIRP per TCI - Agreement: - RAN4 assume TE has the capability of distinguishing EIRP per TCI with the configuration of rank 2 PUSCH transmission in EIRP measurement of STxMP. under SDM scheme to further study the test methodology - Signal level conditions (e.g., SNR, power imbalance) and test procedure need to be further checked. - RAN4 to conclude the feasibility issue in next meeting.

The SDM scheme generally makes the EIRP measurement of STxMP doable. For some directions where large power imbalance may occur, in such case the actual Tx power of wanted signal is much lower than the beam peak power, so EIRP measurement results of such directions does not matter (too small to affect final results).

Based on plentiful discussion in previous meetings and further discussion above, we see the feasibility is no problem to measure/distinguish EIRP per TCI.

Proposal 1:	confirm the feasibility and agree on the configuration of rank 2 PUSCH transmission in EIRP measurement of STxMP under SDM scheme.

Regarding the AoA separation and UE orientation, according to the Multi-RX discussion in Rel-18, there are too many combinations of various AoA separation and UE orientations for multi-AoA measurement, it is not possible to test all of them. In Multi-RX work item, the declaration approach is adopted, i.e., UE declare only one AoA separation and only one UE orientation. Based on beam correspondence principle, in our view the UE declared AoA separation and orientation is also applicable for Multi-Tx (STxMP).

The configured transmitted power requirement corresponds to three measured values: PUMAX,f,c,k (peak EIRP per TCI), PUMAX,f,c (aggregated EIRP), PTMAX,f,c (total radiated power).

At least for PUMAX,f,c,k (peak EIRP per TCI), it is applicable to re-use the UE declared AoA separation and orientation of Multi-RX.

Proposal 2:	for PUMAX,f,c,k (peak EIRP per TCI), re-use the UE declared AoA separation and orientation of Multi-RX.

Based on proposal 1 and 2, the Multi-Tx (STxMP) testing would be much similar as that for Multi-Rx in the simplest way, i.e., completing the EIRP measurements for both TCI by only one time 3D scan.

As discussed in our previous contribution [2, R4-2407899], EIRPmax can be obtained by post-processing and TRPmax can be measured after beam peak is found and beam lock is enabled.

However, it is also observed in last meeting that this simple method requires a pre-condition: make sure the mapping between panels and probes are kept unchanged during the whole 3D scan process, in order to avoid swapping during test (as indicated in the highlighted sub-bullet for option 1).

Issue 1-4-1: AoA separation and UE orientation for EIRP PUMAX,f,c,k testing - Agreements: - FFS on AoA separation and UE orientation for EIRP PUMAX,f,c,k testing considering the following options - Option 1: Consider similar as Multi-Rx UE declaration approach for one AoA pair selection from the set of {30deg, 60deg, 90deg, 120deg, 150deg} and one proper UE orientation listed in TS38.101-2 for EIRP per TCI measurement - How to process the measurement data is FFS - Data processing should avoid repeated selection of peak EIRP per TCI. - Option 2: For the selection of AoA pair, one of AoA is at the beam peak direction of single CC operation without STxMP enable (from Beam Peak Search for MOP in 6.2 of TS 38.101-2), another AoA is decided by UE declared orientation (listed in TS38.101-2) and AoA pair from set of {30deg, 60deg, 90deg, 120deg, 150deg}.

Observation 1:	the mapping relationship “panel1-TCI1-probe1, panel2-TCI2-probe2” need to be kept unchanged and no swap during the whole 3D scan

Further investigation is needed on how to keep the mapping relationship during 3D scan testing.

Proposal 3:	further study how to keep the mapping relationship between panels and probes unchanged (no swap) during 3D scan

In last meeting the test skipping rule for EIRPmax and TRPmax was agreed when single CC results is smaller than  (EIRPmax - 3dB –additional margin).

Issue 1-5-1: Skipping rule for EIRPmax testing - Agreements: - RAN4 agree on the skipping rule proposal for EIRPmax, i.e., “If the peak EIRP of single CC operation without STxMP (from Peak EIRP for MOP in 6.2 of TS 38.101-2) is smaller than EIRPmax - 3dB –additional margin , the corresponding EIRPmax verification can be skipped”. - Further check on the handling of the test case skip as part of conformance test. FFS on additional margin, such as systematic error of beam peak search, etc. - The slightly change of antenna radiation pattern of one panel when the other panel is activated if any, which might varry the test results of EIRPmax, can be compensated by addition margin. Issue 1-6-1: Skipping rule for TRPmax testing - Agreements: - RAN4 agree on the skipping rule proposal for TRPmax, i.e., “If the peak TRP of single CC operation without STxMP (from TRP for MOP in 6.2 of TS 38.101-2) is smaller than TRPmax - 3dB – additional margin[], the corresponding TRPmax verification can be skipped”. - Further check on the handling of the test case skip as part of conformance test. FFS on additional margin, such as systematic error of beam peak search, etc. - The slightly change of antenna radiation pattern of one panel when the other panel is activated if any, which might vary the test results of TRPmax, can be compensated by addition margin.

For the additional margin, two aspects are listed above for further check, one is the error due to finite beam peak search grid, another is the potential slightly change of antenna pattern between Multi-Tx and single-Tx. Actually the error due to finite beam peak search grid is the same in single CC case where the EIRPmax and TRPmax criteria does not adopt additional margin for such error. Considering another aspect about potential slightly change of antenna radiation pattern, we are open to consider 0.5dB margin for the skipping rule.

Proposal 4:	consider 0.5dB as additional margin for EIRPmax and TRPmax test skipping rule, i.e., “If the peak EIRP and TRP of single carrier is no greater than EIRPmax – 3.5dB or TRPmax – 3.5dB, the corresponding EIRPmax or TRPmax verification can be skipped”

3. 	Conclusion

Proposal 1:	confirm the feasibility and agree on the configuration of rank 2 PUSCH transmission in EIRP measurement of STxMP under SDM scheme.

Proposal 2:	for PUMAX,f,c,k (peak EIRP per TCI), re-use the UE declared AoA separation and orientation of Multi-RX.

Observation 1:	the mapping relationship “panel1-TCI1-probe1, panel2-TCI2-probe2” need to be kept unchanged and no swap during the whole 3D scan

Proposal 3:	further study how to keep the mapping relationship between panels and probes unchanged (no swap) during 3D scan

Proposal 4:	consider 0.5dB as additional margin for EIRPmax and TRPmax test skipping rule, i.e., “If the peak EIRP and TRP of single carrier is no greater than EIRPmax – 3.5dB or TRPmax – 3.5dB, the corresponding EIRPmax or TRPmax verification can be skipped”

4. 	References

- R4-2409921    “Way Forward for [111][339] NR_FR2_OTA”, Qualcomm Incorporated

- R4-2407899    “Discussion on FR2 OTA testing of STxMP”, Samsung