3GPP TSG-RAN WG4 Meeting #112	R4-2411700

# Maastricht, Netherlands, 19th – 23rd August, 2024

Title:	Discussion on RC lab alignment and RC harmonization with AC

Source:	Samsung

Agenda item:	6.2.1

Document for:	Discussion

1.	Introduction

Since Samsung RC lab gets the n28 LADs in late stage and in addition the test is not smooth, the n28 measurement results from Samsung lab are not available in last meeting, and RAN4 agreed to allow us to submit data in maintenance phase [1, R4-2410765]

Issue 2-1-2: Maintenance work for RC unavailable results Conclusions: Allow to update RC harmonization data in maintenance phase when the unfinished n28 testing from lab 9 is done.

We have finished the n28 measurements before this meeting and the test results are submitted in [2, R4-2411699].

In this contribution, we further analyze the RC lab alignment and RC harmonization by taking Samsung measurements into account.

2. 	Discussion

The TRP results of talk mode and browse mode from all 6 labs are show as below, and all are well aligned.

The TRS results of talk mode and browse mode from all 6 labs are show as below, the following yellow highlighted results are not within lab alignment pass/fail limit (BHH TRS 1.82dB, BH TRS 1.71dB)

Observation 1:	all 6 RC labs are aligned well except for low band TRS

It is also observed that the deviation between Samsung results and RC reference value is large (around 2dB), however, Samsung results are actually close to AC reference value.

Observation 2:	Samsung RC results for low band TRS are actually close to AC reference value

Note that these measurement results are for both RC lab alignment and RC harmonization with AC, so we should investigate RC lab alignment and RC harmonization together.

However, now the RC lab alignment conclusion and RC harmonization conclusion are as follows:

E.2.4 Conclusions For RC lab alignment: Based on the analysis in Annex E.2.2 and pass fail limits defined in Annex E.2.3, 3GPP Rel-18 RC lab alignment activity can be successfully concluded as following: - All the 6 labs with Reverb chamber system are well aligned at FR1 high band - All 5 labs with available n28 measurement results from Reverb chamber system are well aligned at FR1 low band For RC harmonization: Based on the analysis in Annex E.2.2 and harmonization pass fail limits defined in Annex E.2.3, 3GPP Rel-18 RC harmonization activity can be successfully concluded as following: - RC test method can be harmonized with reference AC test method at FR1 band >3GHz for TRP and TRS measurements. - RC test method harmonization with reference AC test method at FR1 FDD bands below 3GHz is not confirmed.

For FR1 low band, the conclusion is based on 5 labs. Now 6th lab test results have been submitted. It is proposed to update the lab alignment conclusion based on the measurement results of all 6 labs.

Proposal 1:	It is proposed to update the RC lab alignment and RC harmonization outcome by taking Samsung RC lab’s results into account

The corresponding CR to reflect the update can be found in [3, R4-2412051].

These measurement data indicates that more work needs to be done for both lab alignment and RC harmonization for low band TRS. It will be beneficial to the industry to further study RC lab alignment and RC harmonization at low band especially for TRS in the future.

Proposal 2:	RAN4 to further study RC lab alignment and RC harmonization at FR1 low band especially for TRS.

3. 	Conclusion

Observation 1:	all 6 RC labs are aligned well except for low band TRS

Observation 2:	Samsung RC results for low band TRS are actually close to AC reference value

Proposal 1:	It is proposed to update the RC lab alignment and RC harmonization outcome by taking Samsung RC lab’s results into account

The corresponding CR to reflect the update can be found in R4-2412051

Proposal 2:	RAN4 to further study RC lab alignment and RC harmonization at FR1 low band especially for TRS.

4. 	References

- R4-2410765    “WF for [111][336] NR_FR1_TRP_TRS_enh”, vivo

- R4-2411699    “Measurement results for 3GPP Rel-18 TRP TRS RC lab alignment and harmonization activity - Samsung”, Samsung

- R4-2402051    “CR to TR 38.870 on updating RC harmonization results”, vivo, Samsung