3GPP TSG-RAN WG4 Meeting # 112	R4-2411701

Maastricht, Netherlands, Aug. 19 – Aug. 23, 2024

Agenda Item: 5.24.1

Source: MediaTek Inc.

Title: Discussion on R18 L1/L2 mobility core part requirements

Document for: Discussion

- Introduction

In last meeting, RAN4 had some discussion on R18 L1/L2 mobility. There are still some issues open [1]. We will discuss related open issues in this contribution.

- Discussion

2.1 Early TCI state activation

Whether to support unknown TCI state in FR2

One main purpose for early TCI state activation is to skip T/F tracking during cell switch delay. As L1-RSRP measurement delay in FR2 is very likely larger than 160ms, to skip T/F tracking during cell switch delay, NW may have to trigger cell switch soon after TCI state activation. But if the TCI state is unknown, even cell switch is triggered soon after TCI state activation, T/F tracking is still needed. As shown below, to support early TCI state activation for unknown TCI state, UE needs to perform L1-RSRP measurement during the activation delay. As the measurement occasions shall be shared among different cells or with L3 measurement and beam sweeping is also needed, it is uncertain at both UE and NW when UE performs T/F tracking on the target TCI state with a good Rx beam. The gap between cell switch command and the time occasion that UE performs T/F tracking may be far more than 160ms. In other words, even early TCI state activation for unknown TCI state in FR2 is supported, it is difficult to skip T/F tracking during cell switch delay. We see no benefit to support it.

Proposal 1: Not to consider unknown TCI state in FR2 for early TCI state activation.

Whether to consider early TCI state activation for multiple cells at the same time

According to the structure of Candidate Cell TCI States Activation/Deactivation MAC CE (in 38.321 6.1.3.76) designed in RAN2 as below, a single candidate cell TCI states activation/deactivation MAC CE can only activate/deactivate TCI states of a single candidate cell. With this understanding, early TCI state activation for multiple cells at the same time only comes from the scenario with multiple time-overlapping MAC CE commands. We don’t think we need to handle this scenario. It will lead to a huge number of cases because one procedure may be affected by another.

Proposal 2: Not to handle the collision of multiple MAC CE commands for early TCI state activation. Specifically, not to consider early TCI state activation for multiple cells at the same time.

2.2 Cell switch delay requirements

2.2.1. Applicable conditions of the cell switch delay requirements

Last meeting, RAN4 agreed that the cell switch delay requirement is applicable for the case without L1 measurement and report in FR1. RAN4 is supposed to further discuss the applicable conditions.

< Agreement>: - For FR1, the cell switch delay requirement is applicable for the case without L1 measurement and report, under certain conditions. - Further discuss the how the requirement applies together with the condition.

Based on the proposals and discussions last meeting, there are several options:

<Way Forward >: Further discuss the following options in FR1: - Option 1: change known TCI state conditions to make cell switch delay requirements applicable to the case without L1 measurement and report - Option 2: Allow cell switch delay requirements applicable to unknown TCI state case with conditions. - Regarding the conditions, further discuss the following options when SNR of the TCI state≥ -3dB - Condition 1: TCI state has been activated and - Condition 1A: the TCI state activation command was received not more than 1280 ms before the cell switch command - Condition 1B: Beam-level L3 measurement results have been reported in 1280 ms - Condition 1C: The RS associated to the target TCI state is available at least every 1280 ms after TCI state activation command - Condition 2: UE has reported L3-RSRP measurements for the SSB associated to the target TCI state in [1280]ms before the cell switch command

First of all, there should be some conditions to discuss to ensure the activated TCI state is really known at UE side. So, at least, one condition to add is “SNR of the SSB associated to TCI state should be ≥ -3dB”.

The main divergence between Condition 1 and Condition 2 lies in that whether corresponding TCI state is activated or not. The condition that TCI state is activated is to determine whether T/F tracking during cell switch delay is needed or not. Here we are discussing the applicable conditions of cell switch delay requirement. Even TCI state is not activated, the cell switch delay requirements can still apply, and T/F tracking time may be still needed. So we don’t think activated TCI state is one of the applicable conditions of the cell switch delay requirements.

In general, NW will not indicate the TCI state blindly. Thus we propose cell switch delay requirements apply if beam-level L3 measurement results have been reported in 1280ms.

Proposal 3: In FR1, without L1 measurement, cell switch delay requirements are applicable, provided:

- SNR of the SSB associated to TCI state ≥ -3dB

- Beam-level L3 measurement results have been reported in 1280ms

Last meeting, there was also some discussion but with no consensus on how to capture the conditions as shown in Option 1 and Option 2 above. We totally understand the concern on each option. We are thinking maybe RAN4 does not need to define known TCI state conditions here as long as the applicable conditions are made clear.

Proposal 4: Capture the applicable conditions for cell switch delay requirements directly in spec instead of defining known TCI state conditions.

In FR2, it was agreed that the RAN4 Rel-18 related LTM requirements are not applicable if L1 measurement of LTM candidate cell is not supported or configured. There was some proposal that L1 report may be not necessary for cell switch delay especially when TCI state is activated. One of the motivations comes that periodic L1 report leads to much overhead. As well known, R19 mobility is designing event triggered L1 report. In R18, aperiodic report can also be used to reduce report overhead. In our understanding, the intention of configuring L1 measurement is to switch to target cell’s best beam. If cell switch decision is made based on L3 report, we don’t see the necessary and the benefit to configure L1 measurement in FR2. So we prefer to keep the conditions for FR2 as it is.

Proposal 5: Not to extend cell switch delay requirements to FR2 without L1 report.

2.2.2 Issue related to PL-RS

Issue 3-1-2-1: Extra time for PL-RS measurement Ad hoc agreement < Agreement>: - No additional PL-RS measurement time is needed, provided - Option 1: L3-RSRP or L1-RSRP on the SSB associated with PL-RS has been measured/reported - Option 2: Target PL-RS is maintained.

Last meeting, RAN4 reached the consensus that no additional PL-RS measurement time is needed in cell switch delay and the applicable conditions need further discussion. Now there are two options on table. Recalling the legacy requirements, the number of maintained PL-RS per serving cell is limited to 4. For option 1, we don’t think UE shall maintain all the RS that UE has performed L3 or L1 measurement. Originally, Option 2 is our preference, but we found it cannot cover all the scenarios. Following legacy, PL-RS is maintained only if the target PL-RS is associated with or included in the UL or joint TCI states in the active TCI list. However, early TCI state activation is an optional feature. If we go with Option 2, it will make early TCI state activation mandatory for LTM.

We think this issue can be analysed for different cases at first and then discuss whether a simple condition can be defined.

- In some cases, i.e., UE does not support early sync or early sync not triggered or early sync is triggered but the conditions to skip T/F tracking are not met, UE would perform SSB based T/F tracking during cell switch. Due to beam correspondence, the PL-RS should be associated with the SSB used for T/F tracking. With this, the time provided to UE for T/F tracking also already provides UE opportunity for PL estimation, meaning that no additional time for PL-RS measurement is needed. Thus for these cases, the simple condition we need is that PL-RS is associated with the SSB used for T/F tracking during cell switch.

- If PDCCH-order RACH has been triggered in 160ms, UE will not perform T/F tracking during cell switch. In this case, before early RACH, UE already performed T/F tracking on the indicated SSB in PDCCH-order. The condition to skip T/F tracking is that the SSB for tracking indicated in cell switch command is the same as the one indicated in PDCCH-order. If the PL-RS is associated with the SSB used for T/F tracking, then as long as UE maintains the SSB indicated in PDCCH order, no additional time for PL-RS measurement is needed. So the condition is that PL-RS is associated with the SSB indicated for T/F tracking in cell switch command.

- If the TCI state indicated in cell switch command has been activated and other conditions to skip T/F tracking are also satisfied, UE will not perform T/F tracking during cell switch. In this case, the SSB indicated for tracking in the cell switch command has to be one of SSB UE has performed early tracking on. In our understanding, tracking a RS is far more complex than maintain the RSRP of a RS. UE shall maintain the SSBs of the activated TCI states (not exceeds the max number UE can support). Then no addition time for PL-RS measurement is needed. Due to beam correspondence, we don’t think it is a strong limitation to network that the PL-RS is associated with the SSB indicated for T/F tracking in cell switch command.

In summary, the condition of no additional time for PL-RS measurement in cell switch delay is that the PL-RS is associated with the SSB indicated for T/F tracking in cell switch command.

Proposal 6: The condition of no additional time for PL-RS measurement in cell switch delay is that the PL-RS is associated with the SSB indicated for T/F tracking in cell switch command.

2.2.3. Conditions for early ASN.1 decoding and validity check

In previous meetings, RAN4 agreed that both early TCI state activation command and PDCCH-order can be used to trigger early RRC decoding and validity check. This is further optimization. We prefer to define a simple and clear rule to determine the cell(s) to perform early RRC decoding on. During offline discussion, we think some general rules shall be defined:

- NW will not trigger TCI state activation or PDCCH-order RACH on different candidate cells at the same occasion.

- If NW deactivates all the TCI states of a candidate, this cell will be removed from the early RRC decoding list until any of its TCI states is added back again.

With these rules, the concept of most recent cell will become clear as shown below.

Proposal 7: When the configured candidate cells are more than number of candidates that UE supports early RRC decoding and validity check, UE will perform early RRC decoding on the last X cells which TCI state activation MAC-CE or PDCCH order command is sent for and

- NW will not trigger TCI state activation or PDCCH-order RACH on different candidate cells at the same occasion.

- If NW deactivates all the TCI states of a candidate, this cell will be removed from the early RRC decoding list until any of its TCI states is added back again.

2.3 UE feature

For L1-RSRP measurement, 39-1 is introduced to indicate whether UE supports RTD>CP case. RAN1 introduced the capabilities for early TCI state activation (45-3a, 45-4a) and PDCCH order RACH on candidate cell (45-5). If UE reports supporting RTD>CP case and supporting early TCI state activation, then UE also supports early TCI state activation for RTD>CP case. The same logic applies to PDCCH order RACH on neighbor cell (T/F tracking is needed before PACH on neighbor cell in FR1 without L1 measurement) too.

| Index | Feature group | Components | Prerequisite feature groups |
| --- | --- | --- | --- |
| 39-1 | SSB based L1-RSRP measurements for multiple cells with RTD > CP | Capability of simultaneous L1-RSRP measurements for more than one cell when the max RTD among the cells on the same frequency layer or in the same active BWP is larger than CP length of the cell on the frequency layer or in the same active BWP. | 45-1 from RAN1 Rel-18 feature list or 39-2 or 39-2a |
| 45-3a | MAC-CE activated joint LTM TCI states | 1. Supported QCL source RS for MAC-CE activated joint LTM TCI states 2. Maximum number of MAC-CE activated joint LTM TCI states per candidate cell 3. Maximum number of MAC-CE activated joint LTM TCI states across candidate cells and serving cells | 45-3 |
| 45-4a | MAC-CE activated DL/UL LTM TCI states | 1. Supported QCL source RS for MAC-CE activated DL/UL LTM TCI states 2. Maximum number K1 of MAC-CE activated DL TCI states per candidate cell 3. Maximum number K2 of MAC-CE activated UL TCI states per candidate cell 4. Maximum number of MAC-CE activated DL TCI states across all candidate cells and serving cells 5. Maximum number of MAC-CE activated UL TCI states across all candidate cells and serving cells | 45-4 |
| 45-5 | RACH-based early TA acquisition | 1. Maximum number of candidate cells for TA acquisition based on PDCCH ordered CFRA procedure before receiving cell switch command MAC-CE 2. Power ramping for PRACH retransmission based on PDCCH order indication 3. Support of dropping the serving cell UL to handle the overlap between UL transmission on serving cell(s) and PRACH on candidate cell(s) | 45-7, RAN2 FG for LTM |

Last meeting, RAN4 reached consensus that LTM requirements are applicable for the case without L1 measurement and report in FR1. It is possible that UE does not support L1-RSRP measurement on neighbor cell while supports early TCI state activation and PDCCH order RACH on neighbor cell. If UE does not support L1-RSRP measurement on neighbor cell, then UE will not report supporting 39-1. Similar as L1-RSRP measurement, an optional capability is needed to indicate whether UE supports early TCI state activation and/or PDCCH order RACH for RTD>CP case. As RAN2 has not implemented 39-1 yet, we can revise 39-1 to indicate whether UE supports L1-RSRP measurement and/or early T/F tracking and/or PDCCH order RACH when RTD of cells >CP. During offline discussion, the motivation was questioned. We would like to further clarify the motivation here. There may be some UE that does not support RTD>CP. That is why 39-1 is introduced. For those UEs, if the actual RTD is larger than CP, then NW may fail to receive early RACH from this kind of UE as the estimated timing of neighbor cell is not reliable. After cell switch, this kind of UE may fail to receive the first several DCIs or NW may fail to receive the first UL. To avoid the above, it is better to introduce a UE capability, and NW can differentiate different kinds of UEs and disable early TCI state activation and early RACH for this kind of UE if the condition RTD>CP cannot always be met.

Proposal 8: Revise 39-1 to indicate whether UE supports L1-RSRP measurement and/or early T/F tracking and/or PDCCH order RACH when RTD of cells >CP.

- Summary

In this paper, we provide some views on the open issues of R18 L1/L2 triggered mobility. We have the following proposals:

Proposal 1: Not to consider unknown TCI state in FR2 for early TCI state activation.

Proposal 2: Not to handle the collision of multiple MAC CE commands for early TCI state activation. Specifically, not to consider early TCI state activation for multiple cells at the same time.

Proposal 3: In FR1, without L1 measurement, cell switch delay requirements are applicable, provided:

- SNR of the SSB associated to TCI state ≥ -3dB

- Beam-level L3 measurement results have been reported in 1280ms

Proposal 4: Capture the applicable conditions for cell switch delay requirements directly in spec instead of defining known TCI state conditions.

Proposal 5: Not to extend cell switch delay requirements to FR2 without L1 report.

Proposal 6: The condition of no additional time for PL-RS measurement in cell switch delay is that the PL-RS is associated with the SSB indicated for T/F tracking in cell switch command.

Proposal 7: When the configured candidate cells are more than number of candidates that UE supports early RRC decoding and validity check, UE will perform early RRC decoding on the last X cells which TCI state activation MAC-CE or PDCCH order command is sent for and

- NW will not trigger TCI state activation or PDCCH-order RACH on different candidate cells at the same occasion.

- If NW deactivates all the TCI states of a candidate, this cell will be removed from the early RRC decoding list until any of its TCI states is added back again.

Proposal 8: Revise 39-1 to indicate whether UE supports L1-RSRP measurement and/or early T/F tracking and/or PDCCH order RACH when RTD of cells >CP.

4 Reference

R4-2410302, “WF on NR mobility enhancements (part 1)”, MediaTek Inc., RAN4#111 meeting, May 2024