3GPP TSG-RAN WG4 Meeting # 112	R4-2411703

Maastricht, Netherlands, Aug. 19 – Aug. 23, 2024

Agenda Item: 5.24.2

Source: MediaTek Inc.

Title: Discussion on RRM performance requirements for R18 mobility

Document for: Discussion

1 Introduction

Last meeting, a big CR which captures LTM L1-RSRP measurement accuracy requirements and test cases was endorsed [1]. We found the definition of relative accuracy can be further optimized. This contribution provides related discussions.

2. Discussion

In current spec, relative accuracy is defined as

- Intra-f: the L1-RSRP measured from one SSB compared to the largest measured value of L1-RSRP among all SSBs of the cell(s) on the same frequency.

- Inter-f: the L1-RSRP measured from one SSB from one cell on a frequency compared to the largest measured value of L1-RSRP among all SSBs measured from another cell on another frequency.

Here it is always the strongest SSB which is the reference to compare to, which follows legacy definition of relative accuracy L1-RSRP measurement. In legacy L1 measurement report, the largest L1-RSRP value is reported as an absolute value with 7 bits and the others are reported as differential L1-RSRP with 4-bits. As the largest L1-RSRP of serving cell will be always included in the L1 measurement report, we think it is reasonable to use the strongest SSB as a reference. When it comes to LTM, due to the limitation of UCI size, it depends on NW configuration to include which cell and which beam in a measurement report, so the strongest SSB may not always be configured in a particular L1 measurement report. We think it is more reasonable to define L1-RSRP relative accuracy similar as L3 measurement, that is the relative accuracy between any two SSBs.

More specifically, the definition is

- The intra-frequency relative accuracy of SSB based L1-RSRP is defined as the L1-RSRP measured from one SSB compared to the value of L1-RSRP measured from another SSB of the cell(s) on the same frequency, or between any two SSBs measured on the same cell.

- The inter-frequency relative accuracy of SSB based L1-RSRP is defined as the L1-RSRP measured from one SSB compared to the value of L1-RSRP measured from another SSB of the cell(s) on a different frequency.

Proposal 1: The intra-frequency relative accuracy of SSB based L1-RSRP is defined as the L1-RSRP measured from one SSB compared to the value of L1-RSRP measured from another SSB of the cell(s) on the same frequency, or between any two SSBs measured on the same cell.

The inter-frequency relative accuracy of SSB based L1-RSRP is defined as the L1-RSRP measured from one SSB compared to the value of L1-RSRP measured from another SSB of the cell(s) on a different frequency.

3 Summary

In this paper, we provide some view on performance requirements for R18 mobility. We have the following proposal:

Proposal 1: The intra-frequency relative accuracy of SSB based L1-RSRP is defined as the L1-RSRP measured from one SSB compared to the value of L1-RSRP measured from another SSB of the cell(s) on the same frequency, or between any two SSBs measured on the same cell.

The inter-frequency relative accuracy of SSB based L1-RSRP is defined as the L1-RSRP measured from one SSB compared to the value of L1-RSRP measured from another SSB of the cell(s) on a different frequency.

4 Reference

R4-2410400, Big CR to TS 38.133 on performance requirements for Further NR mobility enhancements, MediaTek Inc., Apple, RAN4#111 meeting, May, 2024