3GPP TSG-RAN WG4 Meeting # 112	R4-2411705

Maastricht, Netherlands, Aug. 19 – Aug. 23, 2024

Agenda Item:8.23.2

Source: MediaTek Inc.

Title: Discussion on R19 mobility

Document for: Discussion

- Introduction

RAN#104 agreed on the revised WID “NR mobility enhancements Phase 4” [1]. In this WI, there are three objectives in total: inter-CU LTM, Measurements related enhancements for LTM and Conditional LTM. As there is a checkpoint in Sept. for condition LTM, we would like to focus on potential RRM impacts of the first two objectives in this contribution.

- Discussion

2.1 Inter-CU LTM

In R18, intra-CU LTM is supported and RAN4 defined the requirements for PCell/PSCell cell switch. When defining the requirements in R18, RAN4 didn’t differentiate intra-CU or inter-CU case, the requirements defined are also applicable to inter-CU case.

Proposal 1: LTM requirements defined in R18 are applicable to inter-CU case.

PCell/PSCell switch with scell(s) is also typical scenario. But due to limited time in R18, RAN4 didn’t define the related cell switch delay requirements. We understand that this is not in the WID, if RAN4 would like to do further optimization on top of R18 LTM requirements, more discussion in RAN plenary is necessary. But as this scenario is typical and important, we propose hopefully to see if consensus can be made in RAN4.

Proposal 2: Define cell switch delay requirements for PCell/PSCell switch with scell(s).

2.2 Measurements related enhancements for LTM

According to WID, RAN4 is supposed to define new measurement requirements for three parts:

- Event triggered L1 report,

- CSI-RS measurements,

- Any other measurement related enhancements RAN1/2 to introduce for LTM.

In the next, we would like to provide our views on the above three parts one by one.

2.2.1 Event triggered L1 report

RAN2 assumes filtering of the L1 measure results is needed for LTM event. In R18, L1 measurement delay requirements were defined without filtering. At least, RAN4 needs to discuss how to define SSB based L1 measurement delay requirements with filtering. As this is also the first meeting in RAN1 and there are limited agreements in RAN2, RAN4 can wait for more progress in RAN1 and RAN2 before discussing the detailed requirements.

Proposal 3: Define additional SSB based L1 measurement delay requirements with filtering for event triggered L1 report. Wait for more RAN1/2 progress to discuss the detailed requirements.

2.2.2 CSI-RS measurements

In our understanding, CSI-RS measurements include at least CSI-RS based L1 measurement on neighbor cells which is also our focus in this part.

Before discussing how to define CSI-RS L1 measurement requirements, we would like to recall the past discussions on CSI-RS based L3 measurement. For CSI-RS based L3 measurement, some information is listed here:

- associatedSSB is configured in CSI-RS-Resource-Mobility and detectable,

- Define requirements for Density=3 with (≥) 48 PRBs. No tighter requirements for more 48 PRBs,

- Single FFT is assumed for multiple cell measurements per frequency layer for both intra- and inter-frequency measurements,

- The requirements for intra-f without gap and inter-f with MG are specified.

- Intra-frequency and inter-frequency layer are defined. A measurement is defined as a CSI-RS based intra-frequency measurement provided that:

  - the SCS is the same as the SCS of CSI-RS on the serving cell indicated for measurement, and

  - the CP is the same as the CP type of the CSI-RS resource of the serving cell indicated for measurement,

  - the centre frequency is the same as the centre frequency of the CSI-RS resource of the serving cell indicated for measurement.

- Only one MO is configured per CSI-RS frequency layer

  - all CSI-RS resources in the same MO are configured with the same csi-rs-MeasurementBW,

  - all CSI-RS resources in the same MO are configured with the same periodicity,

  - All CSI-RS in the same MO are confined in the same 5ms window.

For CSI-RS based L1 measurement on neighbor cells, we think RAN4 also needs to discuss whether each of the above items is applicable. In the next, we would like to discuss the above items one by one.

Whether measurement on associated SSB is performed before measuring CSI-RS

Similar as SSB based L1-RSRP measurement, CSI-RS based L1-RSRP measurement should be performed only on known cells. If following CSI-RS configuration used for L1-RSRP measurement in serving cell, higher layer parameter repetition may be set to OFF or on. If the parameter repetition is set to OFF, UE is supposed to use the same Rx beam as the associated SSB to measure the CSI-RS. So L1-RSRP measurement on the associated SSB shall be performed at first. If the parameter repetition is set to on, UE is supposed to sweep Rx beams to find the best Rx beam. In this case, at least L3 measurement shall be performed at first.

Proposal 4: CSI-RS based L1-RSRP measurement is performed only on known cells.

Proposal 5: For CSI-RS resources with repetition OFF, L1-RSRP measurement is performed only after UE has performed L1-RSRP measurement on the associated SSB.

Define requirements for Density=3 with (≥) 48 PRBs

For CSI-RS based L1-RSRP measurement on serving cell and CSI-RS based L3 measurement, RAN4 only defined the requirements for Density=3 with (≥) 48 PRBs. There are no tighter requirements for more 48 PRBs. To avoid introducing too much complexity, for CSI-RS based L1-RSRP measurement on neighbor cell, we propose to define the requirements for Density=3 with (≥) 48 PRBs for CSI-RS based L1-RSRP measurement on neighbor cell and not to define tighter requirements for more 48 PRBs too.

Proposal 6: For CSI-RS based L1-RSRP measurement on neighbor cell, define the requirements for Density=3 with (≥) 48 PRBs and not to define tighter requirements for more 48 PRBs.

Whether to support RTD>CP case?

Single FFT is assumed for multiple cell measurements for CSI-RS based L3 measurement. For SSB based L1 measurement, RTD>CP case is supported with UE capability. If RAN4 can reach consensus on Proposal 6 above, to support RTD>CP case for CSI-RS based L1 measurement, another FFT is needed to cover as wide as 48PRBs, while SSB only occupies 20PRBs. The cost will be much higher to support RTD>CP for CSI-RS compared to SSB. In addition, compared to SSB based L1 measurement, we don’t see much gain to support CSI-RS based L1 measurement. It is sufficient to use SSB based L1 measurement. We suggest to study whether to support RTD>CP case for CSI-RS based L1-RSRP measurement further.

Proposal 7: Further study whether to support RTD>CP case for CSI-RS based L1-RSRP measurement.

Whether to support CSI-RS based inter-f L1-RSRP measurement on neighbor cell?

To discuss this issue, the definition of intra-f and inter-f shall be discussed at first. Different from L3 CSI-RS measurement, there is no concept of “Serving MO” that can be used as a reference to define intra-f and inter-f. Different from SSB based L1-RSRP measurement, different CSI-RS resources of the serving cell may have varied center frequency and BW, which means there is no specific CSI-RS resource to be used as a reference to define intra-f and inter-f. For SSB based L1-RSRP measurement, RAN4 defined the requirements for intra-f without gap, inter-f without gap and inter-f with MG. As well known, the requirements for SSB within active BWP, i.e., inter-f without gap and intra-f without gap, are exactly the same. But we have to capture the same requirements in two different chapters. Similarly, even we define intra-f and inter-f for CSI-RS based L1-RSRP measurement, the requirements of intra-f without gap and inter-f without gap will be the same. It may be better to categorize CSI-RS based L1-RSRP measurement into CSI-RS based L1-RSRP measurement within active BWP and outside active BWP for further discussion, instead of defining intra-f and inter-f.

Proposal 8: Categorize CSI-RS based L1-RSRP measurement into CSI-RS based L1-RSRP measurement within active BWP and outside active BWP for further discussion.

To perform CSI-RS based L1-RSRP measurement outside active BWP, gap shall be configured. Typical MGL are 6ms, 3ms, 4ms, 5.5ms and 1.5ms. CSI-RS resources for L1-RSRP measurement on neighbor cell for a UE would also be the resources for L1 measurement of serving cell for some other UEs. For serving cell, there is no specific restriction in time domain on configurations of CSI-RS resources for L1-RSRP measurement. It makes difficult to ensure all the CSI-RS of the same frequency layer are within a single gap window. To solve this issue, for CSI-RS based L3 measurement, all CSI-RS resources in the same MO are confined in the same 5ms window. If to support CSI-RS based L1-RSRP measurement outside active BWP, similar restriction is also necessary. Also, it is necessary to define what a frequency layer is.

Observation 1: If to support CSI-RS based L1-RSRP measurement outside active BWP, all CSI-RS in the same frequency layer shall be confined in the same 5ms window.

Observation 2: If to support CSI-RS based L1-RSRP measurement outside active BWP, the definition of frequency layer should be discussed at first.

Generally, MG is configured for SSB based L3 measurement. SSB of each cell are overlapped or partially overlapped with MG. Due to measurement restriction in FR2, UE is not supposed to perform SSB based L3 measurement and CSI-RS based L1 measurement simultaneously. So the reasonable configuration is that SSB and CSI-RS for L1 measurement in FR2 are not overlapping. It may be even difficult to confine SSB and CSI-RS of the same frequency layer in the same 5ms window. In other words, it is difficult to use one gap to cover both SSB based L3 or L1 measurement and CSI-RS based L1 measurement, so concurrent gap is needed.

Observation 3: If to support CSI-RS based L1-RSRP measurement outside active BWP, concurrent gap shall be used.

In summary, to support CSI-RS based L1-RSRP measurement outside active BWP, there would be much more restriction on CSI-RS configurations, and an optional feature concurrent gap should be supported at first. In addition, compared to SSB based L1 measurement, we don’t see much gain to support CSI-RS based L1 measurement. It is sufficient to use SSB based L1 measurement for inter-f. We propose to deprioritize CSI-RS based L1-RSRP measurement outside active BWP in R19.

Proposal 9: Deprioritize CSI-RS based L1-RSRP measurement outside active BWP in R19.

Whether and how to define “frequency layer”?

If RAN4 concludes not to define requirements for CSI-RS based L1-RSRP measurement outside active BWP and also RTD>CP case, maybe it is not necessary to define “frequency layer”.

In legacy, UE only performs L1-RSRP measurement on CSI-RS resources within active BWP in serving cell and RAN4 didn’t define any frequency layer concept for CSI-RS based L1 measurement. If only to consider CSI-RS based L1 measurement within active BWP and within CP, it will be similar as CSI-RS based L1 measurement in serving cell. It is not necessary to define frequency layer.

If RAN4 will define requirements for RTD>CP case or outside active BWP case, frequency layer shall be first defined. It is straight forward for CSI-RS based L1-RSRP measurement outside active BWP. In the next, we would like to discuss why frequency layer shall be first defined if to support RTD>CP case. Our proposal is not to define tighter requirements for more 48 PRBs, so another FFT supporting 48 PRBs is needed to support RTD>CP. As shown in Fig. 1, CSI-RS #1,2,3 are three resources from the same neighbor cell. It is obvious that CSI-RS #1 and #2 should be measured TDM-ly as they can not be covered by a single FFT window. If not defining the concept frequency layer, there will be ambiguity on how to measure CSI-RS#3. Or another much wider FFT is needed.

Fig. 1

Observation 4: If to define requirements for RTD>CP case, the definition of frequency layer should be discussed at first.

Although we prefer not to define requirements for RTD>CP case in R19, we still suggest to define frequency layer considering potential support of RTD>CP in future releases. Regarding how to define, we can follow CSI-RS based L3 measurement, i.e., for CSI-RS resources of the same frequency layer, SCS, CP type, center frequency, BW and periodicity are the same.

Proposal 10: Define the concept frequency layer for CSI-RS based L1-RSRP measurement.

Proposal 11: Following CSI-RS based L3 measurement, for CSI-RS resources of the same L1 frequency layer, SCS, CP type, center frequency, BW and periodicity are the same.

2.2.3 Other LTM measurement enhancements

Due to limited time in R18, RAN4 didn’t define SSB based L1-RSRP measurement requirements for deactivated SCell. We understand that RAN4 is supposed to discuss the requirements for the measurements related enhancements introduced in RAN1 and RAN2 according to the WID. RAN1 and RAN2 may not introduce any enhancements for SSB based L1-RSRP measurement requirements for deactivated SCell or intra-f neighbor cell of deactivated SCell in R19. But it is also a typical case that the target cell may be deactivated SCell or its intra-f neighbor cell in LTM. We propose to define SSB based L1-RSRP measurement requirements for deactivated SCell or intra-f neighbor cell of deactivated SCell in the hope to see if consensus can be made in RAN4.

- Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - RAN1 and RAN2 to progress independently on the event triggered measurements objectives of their respective MIMO and Mobility enhancement WIs. Review progress at RAN#105 to see if any modification of objectives is required to avoid/manage any overlap in the work - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management, and/or other necessary physical layer operations on candidate cells before LTM [RAN1]

Proposal 12: Define SSB based L1-RSRP measurement requirements for deactivated SCell or intra-f neighbor cell of deactivated SCell in R19 mobility.

- Summary

In this paper, we provide some views on the R19 mobility. We have the following observation and proposals:

Proposal 1: LTM requirements defined in R18 are applicable to inter-CU case.

Proposal 2: Define cell switch delay requirements for PCell/PSCell switch with scell(s).

Proposal 3: Define additional SSB based L1 measurement delay requirements with filtering for event triggered L1 report. Wait for more RAN1/2 progress to discuss the detailed requirements.

Proposal 4: CSI-RS based L1-RSRP measurement is performed only on known cells.

Proposal 5: For CSI-RS resources with repetition OFF, L1-RSRP measurement is performed only after UE has performed L1-RSRP measurement on the associated SSB.

Proposal 6: For CSI-RS based L1-RSRP measurement on neighbor cell, define the requirements for Density=3 with (≥) 48 PRBs and not to define tighter requirements for more 48 PRBs.

Proposal 7: Further study whether to support RTD>CP case for CSI-RS based L1-RSRP measurement.

Proposal 8: Categorize CSI-RS based L1-RSRP measurement into CSI-RS based L1-RSRP measurement within active BWP and outside active BWP for further discussion.

Observation 1: If to support CSI-RS based L1-RSRP measurement outside active BWP, all CSI-RS in the same frequency layer shall be confined in the same 5ms window.

Observation 2: If to support CSI-RS based L1-RSRP measurement outside active BWP, the definition of frequency layer should be discussed at first.

Observation 3: If to support CSI-RS based L1-RSRP measurement outside active BWP, concurrent gap shall be used.

Proposal 9: Deprioritize CSI-RS based L1-RSRP measurement outside active BWP in R19.

Observation 4: If to define requirements for RTD>CP case, the definition of frequency layer should be discussed at first.

Proposal 10: Define the concept frequency layer for CSI-RS based L1-RSRP measurement.

Proposal 11: Following CSI-RS based L3 measurement, for CSI-RS resources of the same L1 frequency layer, SCS, CP type, center frequency, BW and periodicity are the same.

Proposal 12: Define SSB based L1-RSRP measurement requirements for deactivated SCell or intra-f neighbor cell of deactivated SCell in R19 mobility.

4 Reference

RP-241515, Revised Work Item: NR mobility enhancements Phase 4, Apple Inc, China Telecom