3GPP TSG-RAN WG4 Meeting # 112	                                                          R4-2411706

Maastricht, Netherlands, Aug. 19 – Aug. 23, 2024

Agenda item:	8.17.2

Source:	MediaTek Inc.

Title:	Discussion on AI/ML RAN4 BM use case

Document for:	Discussion

- Introduction

In this contribution, we provide our views about AI/ML RAN4 BM use case.

- Discussion

      - 2.1 Test setup for AI/ML BM

In the past few meetings, there were extensive discussion on testability and test setup for AI/ML BM. Last meeting, RAN4 reached the following agreements. In the next, we would like to further discuss the testability and test setup for AI/ML BM.

Issue 2-3: Reported measurements and ground truth Agreement: - The ground truth for the predicted RSRP is the ideal measurement of RSRP on the predicted Tx beam - In RAN4, the ground truth is the approximate as the reported RSRP measurement under the certain SNR on the predicted Tx beam - FFS on SNR level to ensure that SNR is high enough for sufficient accuracy of reported RSRP - FFS on impact of multiple-AoA test setup - FFS on the channel condition - FFS on whether the ground truth will be changing or not - Other solutions are not precluded Issue 2-5: Test setup needs Agreement: maximum number of set B Tx beams that test system should be able to emulate: [8-16] maximum number of set A Tx beams that test system should be able to emulate: [64-128] FFS on AoAs UE rotation during the test: FFS UE rotation/repositioning between different tests: Yes Issue 2-6: Datasets for training/testing Agreement: - Vendors may take into account the test environment/conditions defined by RAN4 when training the UE - sufficient test environment/conditions should be defined to enable vendors to create the data needed for training - FFS on proposals to augment training data to avoid overfitting of UE models to the test environment RAN4 to strive to make the test conditions similar to field deployment conditions

2.1.1 Test setup needs

Last meeting, there were lots of discussion on which channel model to use in AI/ML BM test. Channel models which are specified include CDL, TDL, AWGN and channel models in 38.901. As well known, CDL channel and channel models in 38.901 are much closer to real field channels. TDL channels are overly simplified and cannot reflect critical beamforming effects at all. As agreed last meeting, RAN4 should strive to make the test conditions similar to field deployment. Based on this basic rule, CDL channel or channel models in 38.901 shall be used.

Proposal 1: CDL channel or channel models in 38.901 shall be used in AI/ML BM test.

The number of gNB beam candidates has an impact on test system complexity, specifically related to the system simulator and channel emulator. Last meeting, RAN4 agreed maximum number of set B Tx beams should be [8-16] and maximum number of set A Tx beams should be [64-128]. In the test, to get the ground truth, i.e., approximate as the reported RSRP measurement under the certain SNR on the predicted Tx beam, TE is supposed to transmit all the Tx beams in set A. As set A Tx beams are far more than set B, number of Tx beams that test system should be able to emulate is equal to number of set A Tx beams, that is [64-128].

Proposal 2: The number of Tx beams that test system should be able to emulate is equal to number of set A Tx beams, that is [64-128].

2.1.2 Potential issues to use existing OTA test systems

R4-2409770 gave a detailed summary on the capabilities of existing OTA test systems. We copy the related information here. It can be observed that none of the existing OTA test systems are able to emulate [64-128] Tx beams with CDL channel. As pointed in R4-2409770, the cost and complexity will be large to update FR2 MIMO OTA test system to support multiple Tx beams with CDL channel. Different from other OTA tests, which require 3 or 4 AoAs at most, here, at least 64Tx beams are required. We are not experts on TE realization, but we doubt it is possible. At least, increase so many supported AoAs will cause large cost in chamber room.

| Applicability Criteria | IFF | Enhanced IFF | FR2 MIMO OTA |
| --- | --- | --- | --- |
| CDL Channel Models | No | No | Yes |
| TDL Channel Models | Yes | Yes | Yes |
| True FF Conditions | Yes | Yes | Not necessarily |
| Multiple TRPs with CDL channel models presented simultaneously | No | No | Requires upgrade whose cost & complexity might be not negligible |
| Multiple TRPs with CDL channel models presented sequentially at t0 and t0 + DT (using positioner movement) | No | No | Yes, but DT≥1s might be excessive |

Observation 1: None of the existing OTA test systems are able to emulate [64-128] Tx beams with CDL channel. It is costly and complex to use multiple probes to emulate multiple AoDs and AoAs of [64-128] Tx beams with CDL channel.

As the exact Rx beamforming gain is unknown at TE and the range of Rx antenna gain is from -10 to 20dB, RAN4#111 agreed that the ground truth is the approximate as the reported RSRP measurement under the certain SNR on the predicted Tx beam. A possible way to get the “ground truth” is separating the test into two steps. In the first step, TE generates the channel and transmits Tx beams in Set B only. UE reports prediction results. In the next step, TE tries to generate the same channel as in the first step and transmits all the Set A Tx beams. The order of the above steps can also be reversed. As the measurement results may be 10dB lower or 20dB higher the transmitted RSRP at TE, it is difficult to check whether UE reports the real measurement results of Set A or reports fake values according to the previous predicted results to pass the test.  Another issue is how to ensure the channels in these two steps are the same. Even setting the random seeds the same, when to perform measurement with a specific beam is uncertain. It is difficult to make sure the channels are the same at the prediction phase and the phase of measuring all Set A.

Observation 2: It is difficult to ensure reference RSRP is measured under similar channel conditions as in prediction phase.

Proposal 3: How to avoid UE cheating in the test, such as reports fake values according to the previous predicted results, shall be further discussed to use the reported RSRP measurement as ground truth.

2.1.3 Solution

To emulate real multiple AoDs and AoAs of [64-128] Tx beams with CDL channel, there will be TE implementation issues. Whether it is possible to get a reliable “ground truth” is still questionable. We wonder whether it is it necessary to emulate real multiple AoDs and AoAs of [64-128] Tx beams with CDL channel or what assumptions can be made to simply the test.

We understand that the intention to discuss how to update existing OTA test systems is to emulate the practical operating scenario. But no matter how, there will still be a gap between the test system and real field. E.g., synthetic channel cannot be the same as the real channel, BS antenna radiation pattern defined for the test or realized by TE may not be exactly the same as any used in the field, and the chamber and beam patterns from different TEs may lead to different test results. As captured in TR38.843, there is significant generalization performance degradation with unseen various gNB setting (i.e., different gNB antenna array dimensions, and/or DL Tx beam codebook) or unseen various Set B of beam(pairs). AI/ML BM models are quite scenario/configuration specific. To past the test, it is very likely to train a specific model based on the defined test environment and configurations. Due to the gap between test system and real field, we may have to admit that the exact model tested will not be any model used in the field.

As captured in TR38.843, measurements based on Set B of beams are used as model input. In addition, beam ID information may be also provided as input to the AI/ML model (e.g. indicate beam ID as position within input or ensure the input beam ID is fixed between training and inference). In our understanding, the model input “measurements” refers to L1 measurement results. All the spatial characteristics will be finally reflected on RSRP. There may be AI model that use spatial characters as model input and learn the correlation between each Tx beam. Basically, better performance can be achieved with more information as input. As mentioned above, the model to pass the test may not actually be used in the field. It is better to focus on the basic model in the test and it is enough to use L1 measurement results of Set B and beam ID information as input.

Proposal 4: In AI/ML BM test, consider the basic model which uses L1 measurement results of Set B as input. Beam ID information may be used as input too.

For the model with L1 measurement results of Set B and beam ID information (optional) as input, it is only the final L1-RSRP value that matters. All the spatial characteristics will be reflected on RSRP. Instead of emulating CDL channels and [64-128] Tx beams through multiple AoAs and AoDs, the randomness and variations in propagation conditions across both time and spatial domains can be finally reflected on received RSRP at the test UE through adjusting Tx power at TE on each beam at different time. This is also the method which is widely used in legacy RAN4 RRM tests.

Proposal 5: In AI/ML BM test, emulate the randomness and variations in propagation conditions across both time and spatial domains through adjusting Tx power at TE on each beam at different time.

In FR2, different test UEs would have specific UE parameters, E.g., different radiation pattern and different Rx beamforming gains on each Tx beam. This is another reason to emulate real multiple AoAs of [64-128] Tx beams with CDL channel. As shown in TR38.843, at least for BMCase-1, AI/ML (without considering model switching) has some performance degradation with various UE parameters.

Last meeting, RAN4 agreed that sufficient test environment/conditions should be defined to enable vendors to create the data needed for training. It means the training data can be created through simulation but not be collected by put test UE in the chamber before test. Then the way to consider specific UE parameters is to simulate the radiation pattern in the simulator when creating training data. However, no matter how to simulate, there will be some mismatch between the simulated radiation pattern and the real one. Again, there is little possibility that the model used in the test is one of the models used in the field.  We have simulated AI/ML model performance with different Rx beam selection. The simulation results are as shown in the table below. In the simulation, UE is set with single panel and 4Rx beams in total. It can be observed that beam prediction accuracy is similar for models “training and inference with best Rx beam” and “training and inference with fixed Rx beam”. In addition, fixed Rx beamforming gain can be viewed as a special radiation pattern. Based on the above, we think it is sufficient to test AI/ML BM with fixed Rx beamforming gain.

| Training | Inference | KPIs |
| --- | --- | --- |
| Rx beam selection | Rx beam selection | Top-1/3/5 accuracy [%] |
| Best Rx | Best Rx | 56.24/86.72/94.18 |
|  | Fixed Rx | 57.35/86.72/93.70 |
| Fixed Rx | Best Rx | 53.44/86.14/93.81 |
|  | Fixed Rx | 54.23/86.4./93.70 |

With fixed Rx beamforming gain, we can use fixed and single AoA in the test. Existing IFF and enhance IFF test systems can be used for AI/ML BM test.

With fixed Rx beamforming gain, although the exact RSRP of each beam received at UE is not known at TE, genie-aided relative strength of Tx beams is available at TE. If RSRP difference is used to test AI/ML BM model performance finally, it is also possible to derive the Rx beamforming gain as it is fixed.

Proposal 6: Only to test AI/ML BM with fixed Rx beamforming gain.

Observation 3: With single AoA, genie-aided relative strength of Tx beams is available at TE.

Proposal 7: Use existing IFF or enhance IFF test systems with single AoA for AI/ML BM test.

## 2.2 Test Metric

For the test metric of AI/ML based beam management, there are still several options. RAN4#111 further discussed the definition of RSRP prediction accuracy.

- Option 1: RSRP accuracy - Option 2: Beam prediction accuracy - Top-1 (%) : the percentage of "the Top-1 strongest beam is Top-1 predicted beam" - Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams" - Top-1/K (%) : the percentage of "the Top-1 predicted beam is one of the Top-K strongest beams" - Option 3: The successful rate for the correct prediction which is considered as maximum RSRP among top-K predicted beams is larger than the RSRP of the strongest beam – x dB, - Related measurement accuracy can be considered to determine x - Option 4: combinations of above options

Issue 2-2: RSRP Prediction accuracy - Proposals - Option 1: Difference between the predicted RSRP and the measured RSRP of the same Tx beam - Option 2: Difference between the predicted RSRP and the ideal RSRP of the same Tx beam - Option 3: Difference between the predicted RSRP(of the predicted beam) and the ideal RSRP of the beam that should be predicted (genie-aided predicted beam) - Option 4: Difference between the predicted RSRP(of the predicted beam) and the measured RSRP of the beam that should be predicted (genie-aided predicted beam) - Option 5: RSRP difference between the RSRP of the genie-aided Top-1 beam and the measured RSRP of the predicted Top-K - Option 6: others

In our understanding, issue 2-2 above was to clarify the candidate metrics Option 1. The difference between Option 1 and Option 2 is what the reference RSRP is, measured or ideal RSRP of the same Tx beam, which is also the difference between Option 3 and Option 4. It depends on which test setup finally adopted. As discussed in section 2.1, if single AoA is used, at least genie-aided relative strength of Tx beams is available at TE and a near ideal RSRP is also possible to be obtained at TE. If multiple AoAs are used in the test, we have to use the measured RSRP as reference as it is not possible to get ideal RSRP or genie-aided relative strength of Tx beams.

Proposal 8: What the reference RSRP is, measured or ideal RSRP, depends on the test setup finally adopted in RAN4.

      - 2.3 RRM core requirements

As AI/ML beam prediction is for the purpose of beam management, there may be some impact on beam management related requirements. For instance, after UE reports the predicted beams or RSRP to NW, NW may activate corresponding reported best-predicted beam. In RAN4, we defined TCI state switching delay requirements. Take the requirements of MAC CE based TCI state activation as an example. The TCI state switch delay is THARQ + 3ms + TL1-RSRP +TOuk*(Tfirst-SSB+ TSSB-proc) / NR slot length, where TL1-RSRP = 0 for known TCI state and TL1-RSRP equals to L1 measurement delay in FR2 for unknown TCI state. If the TCI state is unknown, the switching delay would be long. One of the conditions of known TCI state is that UE has sent at least 1 L1-RSRP report for the target TCI state before the TCI state switch command in 1280ms. The question here is whether the predicted beam is known or unknown if a prediction report is sent recently. Next, we would like to provide our views.

As UE is the one who performs TCI state switching, when talking about known condition, the corresponding TCI state should be actually known from UE side. At least two conditions shall be met in FR2. One is that SINR of the corresponding RS cannot be too low. Another is that UE knows its Rx beam for reception (for DL TCI state activation). To meet the 2nd condition, a TCI state is known only if UE has measured the corresponding RS and reported the measured result in last 1280ms before TCI state switching command. Here if the beam to switch to does not belong to set B, even there is a prediction report but UE may not know which Rx beam to use for DL reception. Note that in the WID only DL Tx beam prediction is in scope and Rx beam prediction is not mandated as shown below.

From WID: - Beam management - DL Tx beam prediction for both UE-sided model and NW-sided model, encompassing [RAN1/RAN2]: - Spatial-domain DL Tx beam prediction for Set A of beams based on measurement results of Set B of beams (“BM-Case1”) - Temporal DL Tx beam prediction for Set A of beams based on the historic measurement results of Set B of beams (“BM-Case2”) - Specify necessary signalling/mechanism(s) to facilitate LCM operations specific to the Beam Management use cases, if any - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE NOTE: Strive for common framework design to support both BM-Case1 and BM-Case2

Observation 4: Only DL Tx beam prediction is in scope and Rx beam prediction is not for both spatial-domain and temporal beam prediction.

Proposal 9: The TCI state QCL to an RS that is not in Set B is unknown if no L1 measurement is performed by UE within 1280ms before TCI state activation.

For the TCI state whose QCL source RS belongs to Set B, if it is spatial-domain beam prediction, the TCI state can be viewed as known if the corresponding predicted beam is reported in 1280ms. If it is temporal beam prediction, we should use the last observation occasion (T3) instead of the reported occasion as shown in Figure 3. The reason lies in that the time gap between T3 and T4 may be too long to ensure Rx beam unchanged. The report occasion may be close to T4.

Figure 3. Illustration for AI/ML temporal beam prediction

Proposal 10: The TCI state which is QCL to an RS in Set B is known:

- if the corresponding predicted beam is reported in 1280ms before the TCI state switch command and SNR of the RS is above -3dB for spatial-domain beam prediction.

- if the last observation occasion is within 1280ms before the TCI state switch command and SNR of the RS is above -3dB for temporal beam prediction.

- Conclusion

In this contribution, we provide our views for AI/ML BM use case. Observations and proposals are summarized as follows.

Proposal 1: CDL channel or channel models in 38.901 shall be used in AI/ML BM test.

Proposal 2: The number of Tx beams that test system should be able to emulate is equal to number of set A Tx beams, that is [64-128].

Observation 1: None of the existing OTA test systems are able to emulate [64-128] Tx beams with CDL channel. It is costly and complex to use multiple probes to emulate multiple AoDs and AoAs of [64-128] Tx beams with CDL channel.

Observation 2: It is difficult to ensure reference RSRP is measured under similar channel conditions as in prediction phase.

Proposal 3: How to avoid UE cheating in the test, such as reports fake values according to the previous predicted results, shall be further discussed to use the reported RSRP measurement as ground truth.

Proposal 4: In AI/ML BM test, consider the basic model which uses L1 measurement results of Set B as input. Beam ID information may be used as input too.

Proposal 5: In AI/ML BM test, emulate the randomness and variations in propagation conditions across both time and spatial domains through adjusting Tx power at TE on each beam at different time.

Proposal 6: Only to test AI/ML BM with fixed Rx beamforming gain.

Observation 3: With single AoA, genie-aided relative strength of Tx beams is available at TE.

Proposal 7: Use existing IFF or enhance IFF test systems with single AoA for AI/ML BM test.

Proposal 8: What the reference RSRP is, measured or ideal RSRP, depends on the test setup finally adopted in RAN4.

Observation 4: Only DL Tx beam prediction is in scope and Rx beam prediction is not for both spatial-domain and temporal beam prediction.

Proposal 9: The TCI state QCL to an RS that is not in Set B is unknown if no L1 measurement is performed by UE within 1280ms before TCI state activation.

Proposal 10: The TCI state which is QCL to an RS in Set B is known:

- if the corresponding predicted beam is reported in 1280ms before the TCI state switch command and SNR of the RS is above -3dB for spatial-domain beam prediction.

- if the last observation occasion is within 1280ms before the TCI state switch command and SNR of the RS is above -3dB for temporal beam prediction.

- Reference

- 3GPP TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface