3GPP TSG-RAN WG4 Meeting # 112	R4-2411707

Maastricht City, Maastricht, NL, Aug 19 – Aug 23, 2024

Agenda Item: 5.27.1

Source: MediaTek Inc.

Title: Discussion on R18 MIMO for RRM core part requirement

Document for: Discussion

- Introduction

According to WF [1], RAN4 had some issues were discussed without conclusion yet. In this paper, we provided our view on the core part requirement, for the following topics:

- UL TCI state switching requirements for mDCI

- The requirement of RLM, BFD and CBD when RTD > CP

- Discussion

2.1 UL TCI state switching requirements for mDCI

Regarding mDCI MAC-CE UL TCI state switching, there’s one open issue, i.e., whether to consider additional time to track DL reference RS for 2TAs for UL TCI state switching.

WF [1] on RAN4 111 Issue 1-2-2: For mDCI mTRP, how to specify UL TCI state switching requirements for eUTCI if UE supporting two TAs (RTD<CP and RTD>CP)? < Way forward > - Option 1 (Apple, Samsung) - For joint TCI state, no additional DL RS tracking time for UL TCI state switching. - For separate UL TCI state, If the DL beams are changed as well and DL TCI is not in the active list, the previous DL timing cannot be used. Additional DL RS tracking time for UL TCI state switching is needed as: - Known case: THARQ + 3Nslotsubframe,µme,µ) + TOk-ref (Tfirst-SSB-DLRef + OL*T SSB-DLRef + 2ms)+NM*( Tfirst-PL-RS + 4*Ttarget_PL-RS + 2ms) - Unknown case: THARQ + 3Nslotsubframe,µ + TL1-RSRP + TOuk-ref (Tfirst-SSB-DLRef + OL*T SSB-DLRef + 2ms)+ Tfirst-PL-RS + 4*Ttarget_PL-RS + 2ms - TOk-ref = 1 if there is no active DL TCI-State for DL timing reference associated with the same coresetPoolIndex - For other cases of separate UL TCI state, no additional DL tracking is needed. - Option 2: (Huawei, MediaTek, Ericsson) - No additional DL RS tracking time for UL TCI state switchin

This is not a new issue. When defining UL spatial relation switch delay, similar issue had been discussed. The final conclusion is: RAN4 did not consider additional time tracking for UL spatial relation switch.

TS 38.133 7.1 UE transmit timing For multi-DCI based multi-TRP operation with two TAs, for each TAG, the uplink transmission timing takes place (NTA+NTA offset)×Tc before the reception of the first detected path (in time) of [one of] the corresponding downlink reference signal(s) of the cell associated with a coresetPoolIndex having same TAG as the uplink signal, where NTA is commanded by the network independently for each TAG [TS 38.331].

From the definition of DL reference timing in 7.1 UE transmit timing, DL reference timing could be any DL RS associated to the same coresetPoolIndex for mDCI. Although adding addition DL reference timing seems relaxing the requirements but implies to switch DL reference timing for UL. We don’t think it is necessary.

Proposal 1: For mDCI mTRP, if UE supports two TAs, do not consider additional DL RS tracking time for UL TCI state switching delay requirement.

2.2 The requirement of RLM, BFD and CBD when RTD > CP

WF [1] on RAN4 111 Issue 1-2-4: RLM/BFD/CBD requirements for mTRP? < Way forward > Option 1 (MediaTek) - The legacy evaluation delay of RLM/BFD/CBD is applicable to RTD>CP case in FR1. The legacy RLM, BFD and CBD requirements are not applicable to RTD>CP case in FR2.

In our understanding, the requirement of RLM, BFD and CBD when RTD > CP is still unclear in current spec. There’s no statement for RTD > CP applicability defined in the requirement of RLM, BFD and CBD. Similar as L1-RSRP measurement, the measurement restriction of RLM/BFD/CBD when RTD>CP is the same as the requirements when RTD<CP in FR1. We provided corresponding CR and open to discuss how to address this in spec.

Regarding FR2, RAN4 agreed not to consider RTD>CP for RLM/BFD/CBD in FR2 in R18 MIMO. Take BFD for instance, UE is required to measure SSB for L1 measurement on the BFD evaluation period in FR2. But when BFD SSB is overlapping or adjacent to the L1-RSRP SSB of the other TRP, we never discussed how to handle this case in legacy requirement of L1 measurement (RLM, BFD and CBD). Therefore, we need to make it clear that the current RLM, BFD and CBD requirements are not applicable to RTD > CP in FR2.

Proposal 2: The legacy measurement restriction of RLM/BFD/CBD is applicable to RTD>CP case in FR1. The legacy RLM, BFD and CBD requirements are not applicable to RTD>CP case in FR2.

- Summary

In this paper, the discussion of R18 MIMO is provided. We have the following proposals:

Proposal 1: For mDCI mTRP, if UE supports two TAs, do not consider additional DL RS tracking time for UL TCI state switching delay requirement.

Proposal 2: The legacy measurement restriction of RLM/BFD/CBD is applicable to RTD>CP case in FR1. The legacy RLM, BFD and CBD requirements are not applicable to RTD>CP case in FR2.

- Reference

[1] R4-2410317, WF on RRM requirements for NR_MIMO_evo_DL_UL, Samsung, RAN#111