3GPP TSG-RAN WG4 Meeting #112	R4-2411710

Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

Agenda item:	8.8.4

Source:	MediaTek Inc.

Title:	Discussion on NTN testing for NGSO

Document for:	Discussion

# Introduction

According to the WID [1], RAN4 will discuss test methodology for NR and IoT NTN by considering channel model with varying Doppler and delay shifts in Rel-19. We provide our views in this contribution.

- Specify the TE-emulated channel model with varying Doppler and delay shifts for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands - Match the satellite motion trajectory based on the ephemeris for NGSO (Non-Geostationary Orbit) scenarios - Inform RAN5 to assist specifying RF frequency error tests, if needed - Checking if the existing frequency error requirement, can be met under the TE-emulated channel model without intention of modifying the existing core requirement - Specify UE RRM performance test of uplink timing for NGSO for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands based on the above channel model - Checking if the existing RRM uplink timing requirements, can be met under the TE-emulated channel model without intention of modifying the existing core requirement - Specify UE demodulation performance requirement(s) of PDSCH for NGSO for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands based on the above channel model - Minimize the number of demodulation performance requirements - Specify the applicability of the tests under the TE-emulated channel model

# Discussion

This WI involves RF/RRM/Demod requirements by considering channel model with varying Doppler and delay shifts. Based on the Clause 5.3.4.3 in TR38.811, the Doppler shift can be derived if the trajectory of satellite is known. Therefore, RAN4 should first focus on determining the trajectory of satellite.

Observation 1: The value of varying Doppler and delay shifts depends on the trajectory of satellite.

In real field, UE will use the ephemeris from satellite to perform the compensation. Also, the ephemeris is signalled by TE in testing. As UE will leverage the ephemeris for signal processing, the ephemeris should match the trajectory of satellite.

Observation 2: The ephemeris should match the trajectory of satellite.

There are two possible options to determine the trajectory of satellite.

Option 1: Assuming the satellite moves in an orbit with some shape around the Earth, e.g., a circular orbit. Then we can derive Doppler and delay at every instance mathematically. Ephemeris can also be derived from the trajectory.

Option 2: Use GMAT mentioned in [2] to generate Ephemeris and the trajectory of satellite.

Proposal 1: RAN4 to discuss use which way to determine the trajectory of satellite

- Option 1: Assuming satellite moves in an orbit with some shape around the earth, e.g., a circular orbit. Then derive Doppler and delay at every instance mathematically. Ephemeris can also be derived from the trajectory

- Option 2: Use GMAT to generate Ephemeris and the trajectory of satellite

In RAN5 conformance test, the position of UE is assumed in a fixed position on Earth. Therefore, the Doppler and delay should be based on the trajectory of satellite and the position of UE assumed in RAN5.

## 4.13 UE position requirements for NTN testing Clauses 4.14 and 8.1.5B define the common test environment which applies to all test cases executed for eMTC and NB-IoT NTN UEs, unless otherwise specified. - UE's positioning engine (e.g., standalone GNSS receiver) shall be enabled to allow it to acquire the position. UE position during the test is stationary. Below position shall be used to pre-configure UE position: - Latitude: 25.08439333 - Longitude: 121.56076999 - Altitude: 0

Proposal 2: The Doppler and delay should consider the trajectory of satellite and the position of UE assumed in RAN5.

# Conclusion

In this contribution, we provide our views on NTN testing for NGSO. Observations and proposals are summarized as below.

Observation 1: The value of varying Doppler and delay shifts depends on the trajectory of satellite.

Observation 2: The ephemeris should match the trajectory of satellite.

Proposal 1: RAN4 to discuss use which way to determine the trajectory of satellite

- Option 1: Assuming satellite moves in an orbit with some shape around the earth, e.g., a circular orbit. Then derive Doppler and delay at every instance mathematically. Ephemeris can also be derived from the trajectory

- Option 2: Use GMAT to generate Ephemeris and the trajectory of satellite

Proposal 2: The Doppler and delay should consider the trajectory of satellite and the position of UE assumed in RAN5.

# Reference

- RP-241281, Revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN, Samsung

- R5-237216, NGSO satellite Ephemeris file generation methodology for NTN NR UE testing, THALES