3GPP TSG-RAN WG4 Meeting #112	R4-2411711

Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

Agenda item:	8.17.4

Source:	MediaTek Inc.

Title:	Discussion on AIML RAN4 CSI Interoperability and testability

Document for:	Discussion

# Introduction

In RAN4#111, RAN4 agreed to bring simulation results in RAN4#112 to check the feasibility of testing for AI/ML CSI compression [1]. Furthermore, with two rounds of email discussion after RAN4#111, companies agreed to use residual convolutional neural networks (ResCNN) as a starting point for alignment. In this contribution, we provide our simulation results based on the agreed data generation assumptions and AI/ML model.

# Discussion

For training and testing data, we follow RAN4’s agreement to reuse system level parameters in TR38.843. Here, we consider the baseline case and ideal channel estimation.

Table 1. Parameters for system level simulation

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD, OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Dense Urban (Macro only) |
| Frequency Range |  | FR1 only, [2GHz, 4GHz] |
| Inter-BS distance |  | 200m |
| Channel model |  | According to TR 38.901 |
| Antenna setup and port layouts at gNB |  | Companies need to report which option(s) are used between - 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ |
| Antenna setup and port layouts at UE |  | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for (rank 1-4) |
| BS Tx power |  | 44dBm for 20MHz |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | Baseline: 15kHz for 2GHz; Optional: 30kHz for 4GHz |
| Simulation bandwidth |  | Baseline: 10 MHz for 15kHz Optional: 20 MHz for 30kHz |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | SU-MIMO |
| MIMO layers |  | Baseline: 1 Optional: 2 |
| CSI feedback |  | Feedback assumption at least for baseline scheme -CSI feedback periodicity (full CSI feedback): 5 ms (baseline) -Scheduling delay (from CSI feedback to time to apply in scheduling): 4 ms |
| Traffic load (Resource utilization) |  | Baseline: 50% Optional: 20/70% |
| UE distribution |  | CSI compression: 80% indoor (3 km/h), 20% outdoor (30 km/h) |
| UE receiver |  | MMSE-IRC as the baseline receiver |
| Feedback assumption |  | Realistic |
| Channel estimation |  | Realistic or ideal channel estimation |

The detailed structures of ResCNN encoder and decoder for two-sided CSI compression AI/ML model are shown in Figure 1 and 2.

| Fig 1. Detailed model diagrams of encoder | Fig 2. Detailed model diagrams of decoder |
| --- | --- |

We generate training and testing data based on assumptions in Tabel 1 and use the above encoder and decoder pair for training with the hyperparameters in Table 2.

Table 2. Hyperparameters used for ResCNN model

| Hyperparameters |  |
| --- | --- |
| Number of samples in training dataset | 700k |
| Number of samples in testing dataset | 160k |
| Optimizer | Adam |
| Learning rate | 0.001 |
| Batch size | 128 |
| Number of epochs | 300 |
| Loss function | SCGS |
| Drop rate | 0.05 |

We provide our results for performance and complexity in Table 3 and 4. The SGCS of AI CSI feedback and eTypeII CSI feedback are both relative to ideal CSI. For the case of 2GHz and 1 layer, it shows that the SCGS of eTypeII and ResCNN are very similar.

Table 3. SCGS for eTypeII and ResCNN

|  | Overhead (bits) | SCGS |
| --- | --- | --- |
| eTypeII PC1 | 62 bits | 0.719 |
| ResCNN model | 64 bits | 0.722 |

Table 4. Complexity of ResCNN AI/ML model

|  | Parameters [M] | FLOPs [M] |
| --- | --- | --- |
| Encoder | 0.127 | 93 |
| Decoder | 1.808 | 1480 |

Observation 1: For the case of 2GHz and 1 layer, the SCGS of ResCNN model is 0.722 with 64 bits feedback. Also, the SCGS of eTypeII PC1 with 62 bits feedback and ResCNN model with 64 bits feedback is very close.

Observation 2: FLOPs of ResCNN encoder is 93M and FLOPs of ResCNN decoder is 1480M.

Observation 3: Parameters of ResCNN encoder is 0.127M and parameters of ResCNN decoder is 1.808M.

# Conclusion

In this contribution, simulation results for alignment are provided. Considering ResCNN AI/ML model, we have the following observations:

Observation 1: For the case of 2GHz and 1 layer, the SCGS of ResCNN model is 0.722 with 64 bits feedback. Also, the SCGS of eTypeII PC1 with 62 bits feedback and ResCNN model with 64 bits feedback is very close.

Observation 2: FLOPs of ResCNN encoder is 93M and FLOPs of ResCNN decoder is 1480M.

Observation 3: Parameters of ResCNN encoder is 0.127M and parameters of ResCNN decoder is 1.808M.

# Reference

- R4-2401570, WF on AI/ML, Qualcomm