3GPP TSG-RAN4 Meeting #112	R4-2411714

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.115-1 | CR | 0039 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS 38.115-1 Editorial Correction on Clause number of Unwanted emissions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Murata Manufacturing Co Ltd. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_netcon_repeater-Perf |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | D |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Clause number 6.5.3.4 is duplicated as follows. 6.5.3.4 Method of test 6.5.3.4 Test requirements 6.5.4 Transmitter spurious emissions It is reasonable to understand that the correct clause number of “Test requirements” is 6.5.3.5, 6.5.3.4 is incorrect |  |  |  |  |  |  |  |  |
| Summary of change: |  | In Clause 6.5.3, clause number of “Test requirements” is corrected. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Duplicated clause number in requirement is remained. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changes >

6.5.3.4	Method of test

6.5.3.4.1	Initial conditions

Test environment: Normal; see annex B.2.

RF channels to be tested for single carrier: B, M and T; see clause 4.9.1.

6.5.3.4.2	Procedure

1)	Connect the single-band connector or multi-band connector under test to measurement equipment as shown in annex D.1.1. All connectors not under test shall be terminated.

As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity, efficiency and avoiding e.g. carrier leakage, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

The measurement device characteristics shall be:

Detection mode: True RMS.

2)	Set the input signal to the representative connectors under test according to the applicable test configuration in clause 4.8 using the corresponding test models NR-FR1-TM 1.1 in clause 4.9.2 at the input power intended to produce the maximum rated output power, Pin,p,AC + 10dB.

3)	Step the centre frequency of the measurement filter in contiguous steps and measure the emission within the specified frequency ranges with the specified measurement bandwidth. For connector under test declared to operate in multiple bands or non-contiguous spectrum, the emission within the Inter passband Bandwidth or sub-block gap shall be measured using the specified measurement bandwidth from the closest RF Bandwidth or sub block edge.

4)	Repeat the test for the remaining test cases, with the channel set-up according to NR-FR1-TM 1.2.

In addition, for multi-band connectors, the following steps shall apply:

5)	For a multi-band connectors and single band tests, repeat the steps above per involved operating band where single band test configurations and test models shall apply with no carrier activated in the other operating band.

6.5.3.5	Test requirements

6.5.3.5.1	Minimum requirements for Wide Area repeater type 1-C (Category A)

For repeater operating in Bands n5, n8, n12, n13, n14, n18, n26, n28, n29, n71, n85, minimum requirements are specified in table 6.5.3.5.1-1.

Table 6.5.3.5.1-1: Wide Area repeater type 1-C operating band unwanted emission minimum requirements (NR bands below 1 GHz) for Category A

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirements (Note 1, 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz |  | 100 kHz |
| 5 MHz f < min(10 MHz, fmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | -12.5 dBm | 100 kHz |
| 10 MHz f fmax | 10.05 MHz f_offset < f_offsetmax | -13 dBm (Note 3) | 100 kHz |
| NOTE 1: For a repeater type 1-C supporting non-contiguous spectrum operation within any operating band, the emission limits within gaps between passbands is calculated as a cumulative sum of contributions from adjacent sub-blocks on each side of the gap between passband. Exception is f ≥ 10MHz from both adjacent sub-blocks on each side of the gap between passband, where the emission limits within gaps between passbands shall be -13 dBm/1 MHz. NOTE 2: For a multi-band connector with inter-passband gap < 2*ΔfOBUE the emission limits within the inter-passband gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or passband on each side of the inter-passband gap, where the contribution from the far-end sub-block or passband shall be scaled according to the measurement bandwidth of the near-end sub-block or passband. NOTE 3: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

For repeater type 1-C operating in Bands n1, n2, n3, n7, n24, n25, n30, n34, n38, n39, n40, n41, n50, n65, n66, n70, n74, n75, n92, n94 minimum requirements are specified in table 6.5.3.5.1-2:

Table 6.5.3.5.1-2: Wide Area repeater type 1-C operating band unwanted emission minimum requirements (1GHz < NR bands ≤ 3GHz) for Category A

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirement (Note 1, 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz |  | 100 kHz |
| 5 MHz f < min(10 MHz, fmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | -12.5 dBm | 100 kHz |
| 10 MHz f fmax | 10.5 MHz f_offset < f_offsetmax | -13 dBm (Note 3) | 1MHz |
| NOTE 1: For a repeater supporting non-contiguous spectrum operation within any operating band, the emission limits within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is f ≥ 10MHz from both adjacent sub blocks on each side of the sub-block gap, where the emission limits within sub-block gaps shall be -13 dBm/1 MHz. NOTE 2: For a multi-band connector with Inter RF Bandwidth gap < 2*ΔfOBUE the emission limits within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or RF Bandwidth. NOTE 3: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

For repeater type 1-C operating in Bands n48, n77, n78, n79, minimum requirements are specified in table 6.5.3.5.1-3:

Table 6.5.3.5.1-3: Wide Area repeater type 1-C operating band unwanted emission limits 
(NR bands >3GHz) for Category A

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirement (Note 1, 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz |  | 100 kHz |
| 5 MHz f < min(10 MHz, fmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | -12.2 dBm | 100 kHz |
| 10 MHz f fmax | 10.5 MHz f_offset < f_offsetmax | -13 dBm (Note 3) | 1MHz |
| NOTE 1: For a repeater supporting non-contiguous spectrum operation within any operating band, the emission limits within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is f ≥ 10MHz from both adjacent sub blocks on each side of the sub-block gap, where the emission limits within sub-block gaps shall be -13 dBm/1 MHz. NOTE 2: For a multi-band connector with Inter RF Bandwidth gap < 2*ΔfOBUE the emission limits within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or RF Bandwidth. NOTE 3: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

6.5.3.5.2	Minimum requirements for Wide Area repeater type 1-C (Category B)

For Category B Operating band unwanted emissions, there are two options for the minimum requirements that may be applied regionally. Either the minimum requirements in clause 6.5.3.5.2.1 or clause 6.5.3.5.2.2 shall be applied.

6.5.3.5.2.1	Category B requirements (Option 1)

For repeater type 1-C operating in Bands n5, n8, n12, n20, n26, n28, n29, n67, n71, n85, the minimum requirements are specified in table 6.5.3.5.2.1-1:

Table 6.5.3.5.2.1-1: Wide Area repeater type 1-C operating band unwanted emission minimum requirements (NR bands below 1 GHz) for Category B

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirement (Note 1, 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz |  | 100 kHz |
| 5 MHz f < min(10 MHz, fmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | -12.5 dBm | 100 kHz |
| 10 MHz f fmax | 10.05 MHz f_offset < f_offsetmax | -16 dBm (Note 3) | 100 kHz |
| NOTE 1: For a repeater type 1-C supporting non-contiguous spectrum operation within any operating band, the emission limits within gaps between passbands is calculated as a cumulative sum of contributions from adjacent sub-blocks on each side of the gap between passband. Exception is f ≥ 10MHz from both adjacent sub-blocks on each side of the gap between passband, where the emission limits within gaps between passbands shall be -15 dBm/1 MHz. NOTE 2: For a multi-band connector with inter-passband gap < 2*ΔfOBUE the emission limits within the inter-passband gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or passband on each side of the inter-passband gap. NOTE 3: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

For repeater type 1-C operating in Bands n1, n2, n3, n7, n25, n34, n38, n39, n40, n41, n48, n50, n65, n66, n70, n75, n92, n94, minimum requirements are specified in table 6.5.3.5.2.1-2:

Table 6.5.3.5.2.1-2: Wide Area repeater type 1-C operating band unwanted emission limits 
(1GHz < NR bands ≤ 3GHz) for Category B

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirements (Note 1, 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz |  | 100 kHz |
| 5 MHz f < min(10 MHz, fmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | -12.5 dBm | 100 kHz |
| 10 MHz f fmax | 10.5 MHz f_offset < f_offsetmax | -15 dBm (Note 3) | 1MHz |
| NOTE 1: For a repeater type 1-C supporting non-contiguous spectrum operation within any operating band, the emission limits within gaps between passbands is calculated as a cumulative sum of contributions from adjacent sub-blocks on each side of the gap between passband, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is f ≥ 10MHz from both adjacent sub-blocks on each side of the gap between passband, where the emission limits within gaps between passbands shall be -15 dBm/1 MHz. NOTE 2: For a multi-band connector with inter-passband gap < 2*ΔfOBUE the emission limits within the inter-passband gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or passband on each side of the inter-passband gap, where the contribution from the far-end sub-block or passband shall be scaled according to the measurement bandwidth of the near-end sub-block or passband. NOTE 3: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

For repeater type 1-C operating in Bands n48, n77, n78, n79, minimum requirements are specified in tables 6.5.3.5.2.1-3:

Table 6.5.3.5.2.1-3: Wide Area repeater operating band unwanted emission limits 
(NR bands >3GHz) for Category B

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirement (Note 1, 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz |  | 100 kHz |
| 5 MHz f < min(10 MHz, fmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | -12.2 dBm | 100 kHz |
| 10 MHz f fmax | 10.5 MHz f_offset < f_offsetmax | -15 dBm (Note 3) | 1MHz |
| NOTE 1: For a repeater supporting non-contiguous spectrum operation within any operating band, the emission limits within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is f ≥ 10MHz from both adjacent sub blocks on each side of the sub-block gap, where the emission limits within sub-block gaps shall be -15 dBm/1 MHz. NOTE 2: For a multi-band connector with Inter RF Bandwidth gap < 2*ΔfOBUE the emission limits within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or RF Bandwidth. NOTE 3: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

6.5.3.5.2.2	Category B requirements (Option 2)

The limits in this clause are intended for Europe and may be applied regionally for repeater type 1-C operating in bands n1, n3, n7, n8, n38, n65.

For a repeater type 1-C operating in bands n1, n3, n7, n8, n38 or n65, minimum requirements are specified in Table 6.5.3.5.2.2-1:

Table 6.5.3.5.2.2-1: Regional Wide Area repeater type 1-C operating band unwanted emission minimum requirements for Category B

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirements (Note 1, 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 0.2 MHz | 0.015 MHz f_offset < 0.215 MHz | -12.5 dBm | 30 kHz |
| 0.2 MHz f < 1 MHz | 0.215 MHz f_offset < 1.015 MHz |  | 30 kHz |
| (Note 4) | 1.015 MHz f_offset < 1.5 MHz | -24.5 dBm | 30 kHz |
| 1 MHz f min(10 MHz, fmax) | 1.5 MHz f_offset < min(10.5 MHz, f_offsetmax) | -11.5 dBm | 1 MHz |
| 10 MHz f fmax | 10.5 MHz f_offset < f_offsetmax | -15 dBm (Note 3) | 1 MHz |
| NOTE 1: For a repeater type 1-C supporting non-contiguous spectrum operation within any operating band, the emission limits within gaps between passbands is calculated as a cumulative sum of contributions from adjacent sub-blocks on each side of the gap between passband, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is f ≥ 10MHz from both adjacent sub-blocks on each side of the gap between passband, where the emission limits within gaps between passbands shall be -15 dBm/1 MHz. NOTE 2: For a multi-band connector with inter-passband gap < 2*ΔfOBUE the emission limits within the inter-passband gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or passband on each side of the inter-passband gap, where the contribution from the far-end sub-block or passband shall be scaled according to the measurement bandwidth of the near-end sub-block or passband. NOTE 3: The requirement is not applicable when fmax < 10 MHz. NOTE 4: This frequency range ensures that the range of values of f_offset is continuous. |  |  |  |

6.5.3.5.3	Minimum requirements for Medium Range repeater type 1-C (Category A and B) for DL

For Medium Range repeater type 1-C for DL, minimum requirements are specified in table 6.5.3.5.3-1 to table 6.5.3.5.3-4.

For the tables in this clause for repeater type 1-C, Prated,x = Prated,p,AC - 10*log (ceil (BWPassband/20MHz))

Table 6.5.3.5.3-1: Medium Range repeater type 1-C operating band unwanted emission minimum requirements, 31< Prated,x  38 dBm (NR bands ≤ 3 GHz)

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirements (Note 1, 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz |  | 100 kHz |
| 5 MHz f < min(10 MHz, Δfmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | Prated,x - 58.5dB | 100 kHz |
| 10 MHz f fmax | 10.05 MHz f_offset < f_offsetmax | Min(Prated,x - 60dB, -25dBm) (Note 3) | 100 kHz |
| NOTE 1: For a repeater type 1-C DL supporting non-contiguous spectrum operation within any operating band the emission limits within gaps between passbands is calculated as a cumulative sum of contributions from adjacent sub-blocks on each side of the gap between passband. Exception is f ≥ 10MHz from both adjacent sub-blocks on each side of the gap between passband, where the emission limits within gaps between passbands shall be Min(Prated,x -60dB, -25dBm)/100kHz. NOTE 2: For a multi-band connector with inter-passband gap < 2*ΔfOBUE the emission limits within the inter-passband gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or passband on each side of the inter-passband gap. NOTE 3: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

Table 6.5.3.5.3-2: Medium Range repeater type 1-C operating band unwanted emission minimum requirements, Prated,x  31 dBm (NR bands ≤ 3 GHz)

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirements (Note 1, 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz |  | 100 kHz |
| 5 MHz f < min(10 MHz, Δfmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | -27.5 dBm | 100 kHz |
| 10 MHz f fmax | 10.05 MHz f_offset < f_offsetmax | -29 dBm (Note 3) | 100 kHz |
| NOTE 1: For a repeater type 1-C DL supporting non-contiguous spectrum operation within any operating band the emission limits within gaps between passbands is calculated as a cumulative sum of contributions from adjacent sub-blocks on each side of the gap between passband. Exception is f ≥ 10MHz from both adjacent sub-blocks on each side of the gap between passband, where the emission limits within gaps between passbands shall be -29dBm/100kHz. NOTE 2: For a multi-band connector with inter-passband gap < 2*ΔfOBUE the emission limits within the inter-passband gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or passband on each side of the inter-passband gap. NOTE 3: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

Table 6.5.3.5.3-3: Medium Range repeater operating band unwanted emission limits, 31< Prated,x  38 dBm (NR bands >3GHz)

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirements (Note 1, 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz |  | 100 kHz |
| 5 MHz f < min(10 MHz, Δfmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | Prated,x - 58.2dB | 100 kHz |
| 10 MHz f fmax | 10.05 MHz f_offset < f_offsetmax | Min(Prated,x - 60dB, -25dBm) (Note 3) | 100 kHz |
| NOTE 1: For a repeater supporting non-contiguous spectrum operation within any operating band the emission limits within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10MHz from both adjacent sub blocks on each side of the sub-block gap, where the emission limits within sub-block gaps shall be Min(Prated,x -60dB, -25dBm)/100kHz. NOTE 2: For a multi-band connector with Inter RF Bandwidth gap < 2*ΔfOBUE the emission limits within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap. NOTE 3: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

Table 6.5.3.5.3-4: Medium Range repeater operating band unwanted emission limits, Prated,x  31 dBm (NR bands >3GHz)

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirements (Note 1, 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz |  | 100 kHz |
| 5 MHz f < min(10 MHz, Δfmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | -27.2 dBm | 100 kHz |
| 10 MHz f fmax | 10.05 MHz f_offset < f_offsetmax | -29 dBm (Note 3) | 100 kHz |
| NOTE 1: For a repeater supporting non-contiguous spectrum operation within any operating band the emission limits within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10MHz from both adjacent sub blocks on each side of the sub-block gap, where the emission limits within sub-block gaps shall be -29dBm/100kHz. NOTE 2: For a multi-band connector with Inter RF Bandwidth gap < 2*ΔfOBUE the emission limits within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap. NOTE 3: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

6.5.3.5.4	Minimum requirements for Local Area repeater type 1-C (Category A and B)

For Local Area repeater type 1-C in NR bands ≤ 3 GHz, minimum requirements are specified in table 6.5.3.5.4-1.

For Local Area repeater type 1-C in NR bands > 3 GHz, minimum requirements are specified in table 6.5.3.5.4-2.

Table 6.5.3.5.4-1: Local Area repeater type 1-C operating band unwanted emission limits (NR bands ≤3GHz)

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirements (Note 1, 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz |  | 100 kHz |
| 5 MHz f < min(10 MHz, Δfmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | -35.5 dBm | 100 kHz |
| 10 MHz f fmax | 10.05 MHz f_offset < f_offsetmax | -37 dBm (Note 10) | 100 kHz |
| NOTE 1: For a repeater type 1-C supporting non-contiguous spectrum operation within any operating band the emission limits within gaps between passbands is calculated as a cumulative sum of contributions from adjacent sub-blocks on each side of the gap between passband. Exception is f ≥ 10MHz from both adjacent sub-blocks on each side of the gap between passband, where the emission limits within gaps between passbands shall be -37dBm/100kHz. NOTE 2: For a multi-band connector with inter-passband gap < 2*ΔfOBUE the emission limits within the inter-passband gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or passband on each side of the inter-passband gap NOTE 3: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

Table 6.5.3.5.4-2: Local Area repeater operating band unwanted emission limits (NR bands >3GHz)

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirements (Note 1, 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz |  |  |
| 5 MHz f < min(10 MHz, Δfmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | -35.2 dBm | 100 kHz |
| 10 MHz f fmax | 10.05 MHz f_offset < f_offsetmax | -37 dBm (Note 3) |  |
| NOTE 1: For a repeater supporting non-contiguous spectrum operation within any operating band the emission limits within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10MHz from both adjacent sub blocks on each side of the sub-block gap, where the emission limits within sub-block gaps shall be -37dBm/100kHz. NOTE 2: For a multi-band connector with Inter RF Bandwidth gap < 2*ΔfOBUE the emission limits within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap NOTE 3: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

6.5.3.5.5	Minimum requirements for additional requirements

6.5.3.5.5.1	Limits in FCC Title 47

In addition to the requirements in clauses 6.5.3.5.1, 6.5.3.5.2, 6.5.3.5.3 and 6.5.3.5.4, the repeater type 1-C may have to comply with the applicable emission limits established by FCC Title 47 [13], when deployed in regions where those limits are applied, and under the conditions declared by the manufacturer.

6.5.3.5.5.2	Protection of DTT

In certain regions the following requirement may apply for protection of DTT. For repeater type 1-C operating in Band n20, the level of emissions in the band 470-790 MHz, measured in an 8 MHz filter bandwidth on centre frequencies Ffilter according to table 6.5.3.5.5.2-1, a minimum requirements PEM,N is declared by the manufacturer. This requirement applies in the frequency range 470-790 MHz even though part of the range falls in the spurious domain.

Table 6.5.3.5.5.2-1: Declared emissions minimum requirement for protection of DTT

| Filter centre frequency, Ffilter | Measurement bandwidth | Declared emission minimum requirement (dBm) |
| --- | --- | --- |
| Ffilter = 8*N + 306 (MHz); 21 ≤ N ≤ 60 | 8 MHz | PEM,N |

Note:	The regional requirement is defined in terms of EIRP (effective isotropic radiated power), which is dependent on both the repeater emissions at the antenna connector and the deployment (including antenna gain and feeder loss). The requirement defined above provides the characteristics of the repeater needed to verify compliance with the regional requirement. Compliance with the regional requirement can be determined using the method outlined in TS 36.104 [5], annex E.

<End of Changes >