# 3GPP TSG-RAN WG4 Meeting #112	R4-2411717

Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda Item:	4.2

Source: 	Murata Manufacturing Co Ltd.

Title: 	Discussion on Measurement Bandwidth for FR2 UE Tx

Document for:	Discussion

1	Introduction

This contribution discusses the issue on Measurement bandwidth of FR2 UE and proposes to correct it.

2	Discussion

Measurement bandwidth is specified for the requirement of minimum output power and transmit OFF power. From Rel-17, PC5, PC6 and PC7 were introduced for FR2 UE RF requirement.

For the minimum output power requirement, measurement bandwidth of PC5 and PC6 is different from that of other power class. But on the other hand, for transmit OFF power requirement, the same measurement bandwidth is specified for the all power class. The details are follows [1],[2].

Table 2-1: Minimum output power for power class 1 (Table 6.3.1.1-1 in TS 38.101-2)

| Operating band | Channel bandwidth (MHz) | Minimum output power (dBm) | Measurement bandwidth (MHz) |
| --- | --- | --- | --- |
| n257, n258, n260, n261, n262 | 50 | 4 | 47.58 |
|  | 100 | 4 | 95.16 |
|  | 200 | 4 | 190.20 |
|  | 400 | 4 | 380.28 |
| n263 | 100 | 4 | 95.16 |
|  | 400 | 4 | 381.12 |
|  | 800 | 4 | 715.20 |
|  | 1600 | 4 | 1429.44 |
|  | 2000 | 4 | 1705.92 |

Table 2-2: Minimum output power for power class 2, 3, and 4 (Table 6.3.1.2-1 in TS 38.101-2)

| Operating band | Channel bandwidth (MHz) | Minimum output power (dBm) | Measurement bandwidth (MHz) |
| --- | --- | --- | --- |
| n257, n258, n259, n260, n261, n262 | 50 | -13 | 47.58 |
|  | 100 | -13 | 95.16 |
|  | 200 | -13 | 190.20 |
|  | 400 | -13 | 380.28 |
| n263 | 100 | -13 | 95.16 |
|  | 400 | -13 | 381.12 |
|  | 800 | -13 | 715.20 |
|  | 1600 | -13 | 1429.44 |
|  | 2000 | -13 | 1705.92 |
| NOTE 1: n260 is not applied for power class 2. NOTE 2: n259 is not applied for power class 4. NOTE 3: power class 4 is not applicable to n263 |  |  |  |

Table 2-3: Minimum output power for power class 5 and 6 (Table 6.3.1.3-1 in TS 38.101-2)

| Operating band | Channel bandwidth (MHz) | Minimum output power (dBm) | Measurement bandwidth (MHz) |
| --- | --- | --- | --- |
| n257, n258, n259, n261 | 50 | -6 | 47.52 |
|  | 100 | -6 | 95.04 |
|  | 200 | -6 | 190.08 |
|  | 400 | -6 | 380.16 |

Table 2-4: Minimum output power for power class 7 (Table 6.3.1.4-1 in TS 38.101-2)

| Operating band | Channel bandwidth (MHz) | Minimum output power (dBm) | Measurement bandwidth (MHz) |
| --- | --- | --- | --- |
| n257, n258, n261 | 50 | -13 | 47.58 |
|  | 100 | -13 | 95.16 |

As shown in the above tables, the measurement bandwidth of PC5 and PC6 is specified to be 47.52 MHz for 50 MHz channel bandwidth in the minimum output power requirement. However, in the other power classes (PC1 to PC4 and PC7), 47.58 MHz is specified for the same 50 MHz channel bandwidth. And similar differences exist for other channel bandwidths.

On the other hand, in the transmit OFF power requirement, measurement bandwidth is specified as table 2-5, and there is no difference depending on power class.

Table 2-5: Transmit OFF power for FR2-1  (Table 6.3.2-1 in TS 38.101-2)

| Operating band | Channel bandwidth / Transmit OFF power (dBm) / measurement bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| n257, n258, n259, n260, n261, n262 | -35 | -35 | -35 | -35 |
|  | 47.58 MHz | 95.16 MHz | 190.20 MHz | 380.28 MHz |

The above differences might be due to the following factors. In case of PC5 and PC6, measurement bandwidth of minimum output power requirement is defined as the widest bandwidth which determined by the following equation.

Measurement bandwidth = 12 * SCS * NRB	(1)

Table 2-6 shows the results of calculating the measurement bandwidth for each SCS and channel bandwidth using the above equation. The widest bandwidth in each channel bandwidth is colored in yellow. These values are consistent with the measurement bandwidth values applied to PC5 and PC6 for the minimum output power requirement (Table 2-3).

Table 2-6: Measurement bandwidth calculated by equation (1)

| SCS | Channel bandwidth / measurement bandwidth (MHz) |  |  |  |
| --- | --- | --- | --- | --- |
| (kHz) | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| 60 | 47.52 | 95.04 | 190.08 | N/A |
| 120 | 46.08 | 95.04 | 190.08 | 380.16 |

In contrast, for other power class and the transmit OFF power requirement, measurement bandwidth of minimum output power requirement is defined as the widest bandwidth which determined by the following equation.

Measurement bandwidth = 12 * SCS * NRB + SCS		(2)

Table 2-7 shows the results of calculating the measurement bandwidth for each SCS and channel bandwidth using equation (2). The widest bandwidth in each channel bandwidth is colored in yellow. These values are consistent with the measurement bandwidth values applied to PC1 ~ PC4, PC7 for the minimum output power requirement (Table 2-1, 2-2, 2-4).

Table 2-7: Measurement bandwidth calculated by equation (2)

| SCS | Channel bandwidth / measurement bandwidth (MHz) |  |  |  |
| --- | --- | --- | --- | --- |
| (kHz) | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| 60 | 47.58 | 95.10 | 190.14 | N/A |
| 120 | 46.20 | 95.16 | 190.20 | 380.28 |

Table 2-8 shows the results of calculating the measurement bandwidth for FR2-2 using equation (2). The widest bandwidth in each channel bandwidth is colored in yellow. These values are consistent with the measurement bandwidth values applied to PC1 to PC4 for the minimum output power requirement (Table 2-1, 2-2).

Table 2-8: Measurement bandwidth for FR2-2 calculated by equation (2)

| SCS | Channel bandwidth / measurement bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| (kHz) | 100 MHz | 400 MHz | 800 MHz | 1600 MHz | 2000 MHz |
| 120 | 95.16 | 380.28 | N/A | N/A | N/A |
| 480 | N/A | 380.64 | 714.72 | 1428.96 | N/A |
| 960 | N/A | 381.12 | 715.20 | 1429.44 | 1705.92 |

As mentioned above, the measurement bandwidth of PC5 and PC6 is apparently narrower by one SCS than that of the other power classes (PC1 ~ PC4 and 7). Considering the relationship between SCS and the bandwidth, the concept of PC1 ~ PC4 and PC7 is reasonable to understand. Unless there is a specific reason, the measurement bandwidth for PC5 and PC6 should be revised.

Observation 1:	The measurement bandwidth of PC5 and PC6 is apparently narrower by one SCS than that of other Power Classes (PC1~PC4 and 7)

Proposal 1:	To revise the measurement bandwidth of PC5 and PC6 on the minimum output power requirement as follows.

Table 2-9: Minimum output power for power class 5 and 6

| Operating band | Channel bandwidth (MHz) | Minimum output power (dBm) | Measurement bandwidth (MHz) |
| --- | --- | --- | --- |
| n257, n258, n259, n261 | 50 | -6 | 47.58 |
|  | 100 | -6 | 95.16 |
|  | 200 | -6 | 190.20 |
|  | 400 | -6 | 380.28 |

The relevant CRs are R4-240XXXX and R4-240XXXX.

3	Conclusion

This contribution shows our consideration and proposal on measurement bandwidth for FR2 UE Tx.

Observation 1:	The measurement bandwidth of PC5 and PC6 is apparently narrower by one SCS than that of other Power Classes (PC1~PC4 and 7)

Proposal 1:	To revise the measurement bandwidth of PC5 and PC6 on the minimum output power requirement as follows.

Table 2-9: Minimum output power for power class 5 and 6

| Operating band | Channel bandwidth (MHz) | Minimum output power (dBm) | Measurement bandwidth (MHz) |
| --- | --- | --- | --- |
| n257, n258, n259, n261 | 50 | -6 | 47.58 |
|  | 100 | -6 | 95.16 |
|  | 200 | -6 | 190.20 |
|  | 400 | -6 | 380.28 |

4	References

3GPP TS 38.101-2 V17.14.0

3GPP TS 38.101-2 V18.6.0