3GPP TSG-RAN WG4 Meeting #112	R4-2411720

Maastricht, Netherlands, 19 Aug 2024 - 23 Aug 2024

Source:	ISSDU Inc.

Title:	Discussion on ACLR Calculation Procedure for 14800 - 15350 MHz with Beamforming and MIMO

Agenda item:	8.2.4.1

Document for:	Discussion

Introduction

Based on the current specification [1], critical requirements like ACLR and ACS are determined under the simple SISO case, without considering the possible use of MIMO. In case of adaptive antennas the ACLR is adjusted  as follows:

the 3GPP specifications provide a method to compute the array ACLR (albeit in a TRP mode) :

The ACLR (CACLR) absolute basic limits in table 6.6.3.2-2 + X, 6.6.3.2-2a + X (where X = 9 dB) or the ACLR (CACLR) basic limit in table 6.6.3.2-1, 6.6.3.2-2a or 6.6.3.2-3, whichever is less stringent, shall apply.

- And no allowance is made for adjusting this for antenna patterns, antenna numbers, MIMO modes etc.

- Given that MIMO technologies for beamforming have become standard for IMT parameters, the influence of MIMO on the required values of ACLR and ACS needs further discussion. This contribution proposes simulation assumptions to perform the ACLR Calculation Procedure for the 14800 - 15350 MHz frequency range.

Discussion

- It is observed that the use of beamforming can relax the ACLR requirements, as reported in [2] but also send the distortions in un-intended directions that are in turn dependent on SU and MU MIMO. In addition, in the specification of satellite communications, the required values of ACLR are different for different MIMO technologies [3]. These therefore motivate the discussions on whether the required values of ACLR should also be determined by the technologies used for transmissions.

- Our goal is to verify how the introduction of MIMO technology can impact ACLR limits.  Transmit powers could be reduced to ensure the PA operates in linear region but this will impact coverage. Reductions of Tx power could reduce out of band radiations. There is a need to understand the impact of beamforming on ACLR and the relationship with MIMO especially SU and MU MIMO.

There is a need to understand the impact of PA non linearities on out of band emissions.

Both SU MIMO and MU MIMO cases should be considered.

Bandwidth up to 200 MHz should be considered.

ACLR Calculation Procedure

Figure 2.1-1: ACLR Calculation Procedure

Step 1: Set test model, bandwidth, subcarrier spacing and duplex mode

It is recommended that the simulation parameters for the 14.8 - 15.35 GHz range reference the frequency range 10 - 10.5 GHz, according to [1],[4] (Q=273*12=3276, N=4096)

Table 2.1.1-1: IMT parameters for 14.8 - 15.35 GHz

| Parameters | Setting |
| --- | --- |
| Frequency Range | 14.8-15.35GHz |
| Modulation | OFDM-QPSK |
| Bandwidth | 100MHz |
| Subcarrier Spacing | 30kHz |
| Duplex Mode | TDD |

Figure 2.1.1-1: Initial Resource Allocation

Note that beamforming is not required for the CSI-RS, and we consider each time slot to be independent. Then we have

Xk=Ik+iQk(QAM symbol at the k-th subcarrier)

Simulation parameters for the 14.8 to 15.35 GHz range should reference the 10-10.5 GHz frequency range, with the 5G NR waveform used for ACLR evaluation.

Step 2: Filtering (Windowing)

The leakage resulting from the superposition of sinc functions becomes severe. Therefore, we will first filter out the energy radiated outside the band.

Let lk for k=0, 1, …, Q-1 be the k-th sample of the filter weight, then set the filtered symbol

Xk'=lkXk, for k=0,1,...,Q-1

Xk=Xk',0≤k<Q0    ,Q≤k<N

Figure 2.1.2-1: Filtering

Step 3: Beamforming and oversampling

Apply a beamforming weight wk(f) on the symbol, then perform an IFFT to convert the signal from the frequency domain back to the time domain.

IFFTwk(f)Xk=1Nk=0N-1wk(f)Xkei2πmkN=xm,  m=0,1,...,N

Oversampling helps to create a waveform that better approximates the desired signal. This reduces spectral leakage and ensures that the signal stays within its allocated bandwidth, thus improving spectral efficiency. If we oversample by a factor of two, then

xj=xj2             ,j is even or zero12⋅xj+12+xj-12,j is odd, j=0,1,…,2N-1

Whether beamforming should be implemented on resource elements of PDCCH should be further discussed for test model.

Beamforming should not be use on resource elements of CSI-RS.

Step 4: Nonlinear high-power amplifiers

After passing the time-domain signal through a nonlinear amplifier (Modified Rapp model), the signal before the RF beamforming and antenna can be obtained.

xj,out=FAM/AM×e(FAM/PM×i)

FAM/AM(xj)=glin×xj(1+glin×xjVsat2p)1/2pFAM/PM(xj)=A×xjq(1+xjB)q+angle(u),where glin: Linear power gain (dB)Vsat: Output saturation levelp: Magnitude smoothness factorA: Phase gain (rad)B: Phase saturationq: Phase smoothness factor

Frequency dependent nonlinearity model for high-power amplifier should be further considered. As the Pas are distributed, a model for PA non linearity variation across the PAs need to be agreed

Step 5: Antenna pattern and RF beamforming

Finally, by combining the effect of analog beamforming and the antenna pattern in TR 38.803 with the time-domain signal xj,out, the ACLR can be computed.

Channel Matrix Generation

By channel model in TR 38.901, we can generate a raw channel matrix Hraw. Then, the raw channel matrix would be combined with the antenna pattern and RF beamforming, resulting in the effective channel H. Using this the effective channel, the required zero-forcing and MMSE beamforming weights can be calculated.

H[f]=FFTH(τ,θ,φ)*sinc(t)|t=j2N

wZF(f)=H+qκH+q, where H+q is q-th column of H+, and H+=HH(HHH)-1

wMMSE(f)=1κHHHHH+Qσn2PIQ-1, where Qσn2P is a regularization factor

Conclusion

Within this contribution we have discussed and introduced the Adjusting ACLR Requirements for 14.8-15.35 GHz.

In this contribution, the following Proposals were made:

- Both SU MIMO and MU MIMO cases should be considered.

Bandwidth up to 200 MHz should be considered.

Simulation parameters for the 14.8 to 15.35 GHz range should reference the 10-10.5 GHz frequency range, with the 5G NR waveform used for ACLR evaluation.

Beamforming should not be use on resource elements of CSI-RS.

In the previous sections we made the following observations:

- There is a need to understand the impact of PA non linearities on out of band emissions.

Whether beamforming should be implemented on resource elements of PDCCH should be further discussed for test model.

Frequency dependent nonlinearity model for high-power amplifier should be further considered. As the Pas are distributed, a model for PA non linearity variation across the PAs need to be agreed.

- References

- 3GPP TS 38.104 v18.5.0 (2024-05), “Base Station (BS) radio transmission and reception (Release 18)”.

- Christopher Mollén, Erik G. Larsson, Ulf Gustavsson, Thomas Eriksson and Robert W. Heath, Jr., 
“Out-of-Band Radiation from Large Antenna Arrays,”, April 6, 2017

- EN302.574-1 v2.1.1 (2016-06), “Satellite Earth Stations and Systems (SES); Harmonised Standard for Mobile Earth Stations (MES) operating in the 1 980 MHz to 2 010 MHz (earth-to-space) and 2170 MHz to 2200 MHz (space-to-earth) frequency bands covering the essential requirements of article 3.2 of the Directive 2014/53/EU”

- 3GPP TR 38.921 v18.0.1 (2024-05), “Study on International Mobile Telecommunications (IMT) 
parameters for 6.425-7.025 GHz, 7.025-7.125 GHz and 10.0-10.5 GHz (Release 18)”.