3GPP TSG-RAN WG4 #112	R4- 2411721

Maastricht, Netherlands, August 19-23, 2024

Agenda item:	5.29.1		RRM Core Requirements

Source: 	Qualcomm Incorporated

Title: 	On the remaining open issues for SSB-less SCell operation

Document for:	Discussion

# Introduction

In this paper, we provide our input to the remaining open RRM core requirements for SSB-less SCell operation.

# Discussion

In RAN4 #111, two main issues remained open in Rel-18 network energy saving for further discussion in the upcoming meeting [1]. That is the power difference condition and the joint activation of multiple SSB-less SCells.

The following way forward was agreed for the power difference condition [1].

Issue 1-1-1: Power difference conditions

Agreement:

Clarify that EPRE difference is based on the EPRE normalized by SCSs of SSB of reference Cell and A-TRS/P-TRS of SSB-less Cell.

Only change [9] dB to 12 [dB] in this meeting.

Further discuss whether and how to support larger EPRE difference in next meeting:

- Option 1: Clarify that EPRE difference is smaller or equal to [12] dB + |20*log (f1/f2)| - Margin, where f1 and f2 is the center frequency of reference Cell and SSB-less Cell and the value of Margin is FFS.

- Option 2: Further increase [12] dB and the value is FFS

- Option 3: Update the requirements with one more P-TRS sample for larger EPRE difference

- Other options are no precluded.

Issue 1-1-1: Power difference conditions

Agreement:

Clarify that EPRE difference is based on the EPRE normalized by SCSs of SSB of reference Cell and A-TRS/P-TRS of SSB-less Cell.

Only change [9] dB to 12 [dB] in this meeting.

Further discuss whether and how to support larger EPRE difference in next meeting:

- Option 1: Clarify that EPRE difference is smaller or equal to [12] dB + |20*log (f1/f2)| - Margin, where f1 and f2 is the center frequency of reference Cell and SSB-less Cell and the value of Margin is FFS.

- Option 2: Further increase [12] dB and the value is FFS

- Option 3: Update the requirements with one more P-TRS sample for larger EPRE difference

- Other options are no precluded.

In RAN4 #111, it has been agreed that EPRE difference between reference cell and SSB-less is increased from 9 dB to 12 dB. However, for certain deployments there is a desire to increase the power difference conditions even further. The power difference condition should ensure that the signal of the inter-band SCell to be activated is not saturated and fits well into the dynamic range of the AGC. Hence, the power difference condition should be defined in absolute terms, independent on the carrier frequencies. Option 1 in the way forward above does not meet this condition and, hence, does not provide a meaningful definition of the power difference.

Observation 1: The power difference conditions should be defined in absolute terms, independent on the carrier frequencies. Option 1 in the way forward above does not meet this condition.

Our view is that the power difference condition could be further increased beyond EPRE > 12 dB but then also one more P-TRS sample may be needed for AGC convergence. For power difference of 12 dB, two P-TRS occurrences are expected to be sufficient, but this may not be the case for larger power differences anymore. Hence, we propose to adopt option 3 issue 1-1-1: power difference conditions.

Proposal 1: We propose to adopt option 3 for issue 1-1-1: power difference conditions. The power difference condition could be further increased beyond EPRE = 12 dB. In this case one more P-TRS occurrence should be allowed for AGC convergence.

Current text version in [2] for introducing the requirements for SSB-less SCell defines the EPRE difference as the power difference between TRS/A-TRS symbol on the SSB-less SCell and SSB symbol on the reference serving cell [after the compensation for AGC]. In our understanding the EPRE power difference denotes the receive power level at the antenna connector. The reference point at the antenna connector is before AGC operation takes place and takes already the pathloss difference due to different carrier frequencies into account. No additional compensation is needed. Hence, the wording [after the compensation for AGC] should be removed.

Proposal 2: The wording [after the compensation for AGC] in [2] should be removed from the specification in [2].

In RAN4 #111, the following way forward was agreed for the activation of multiple SSB-less SCells.

Issue 1-1-3: Multiple SSB-less SCells activation

Agreement:

- Also define requirements for following scenario:

  - When all to-be-activated SCells are SSBless on the same band and SCells are contiguous.

  - All to-be-activated SCells have same QCL source cell.

  - P-TRS is used.

    - Option 1: The multiple SCell activation delay requirements are based on TRS with the shortest periodicity. The detailed requirements can be discussed in the CR.

    - Option 2: UE activate each to-be-activated SCell based on the TRS on the SCell, and the requirements to be defined accordingly (i.e. single CC delay)

  - FFS A-TRS based requirements.

Issue 1-1-3: Multiple SSB-less SCells activation

Agreement:

- Also define requirements for following scenario:

  - When all to-be-activated SCells are SSBless on the same band and SCells are contiguous.

  - All to-be-activated SCells have same QCL source cell.

  - P-TRS is used.

    - Option 1: The multiple SCell activation delay requirements are based on TRS with the shortest periodicity. The detailed requirements can be discussed in the CR.

    - Option 2: UE activate each to-be-activated SCell based on the TRS on the SCell, and the requirements to be defined accordingly (i.e. single CC delay)

  - FFS A-TRS based requirements.

In multiple SSB-less SCells, the SCells are intra-band. Hence, these SCells can have a power difference of up to 6 dB and a MRTD of up to ms. Due to this power difference, the AGC setting of the first activated SCell may not be the best setting for the other SCells. The same applies for the time synchronization due to the MRTD. Hence, for option 1 it is not obvious that the AGC state and time/frequency tracking states can be directly taken from the first SCell that has been brought up. For the clarification of the requirements in the CR, further lengthy studies may be needed. Hence, our preference is option 2

Observation 2: For option 1, it is not obvious that the AGC state and time/frequency tracking states can be directly taken from the first SCell that has been brought up. For the clarification of the requirements in the CR, further lengthy studies may be needed.

Proposal 3: Requirements should be defined based on option 2 for issue 1-1-3: Multiple SSB-less SCell activation

In Rel-17, no requirements for activation of multiple SCells are defined based on A-TRS. Since there is no reference, also no requirements for multiple SSB-less SCell activation should be defined for A-TRS in Rel-18.

Proposal 4: Since there is no reference, also no requirements for multiple SSB-less SCell activation should be defined for A-TRS in Rel-18.

# Conclusions

In this paper we provided our views for the remaining open issues for RRM core requirements in SSB-less SCell operation. Our observations and proposals are summarized as follows:

Observation 1: The power difference conditions should be defined in absolute terms, independent on the carrier frequencies. Option 1 in the way forward above does not meet this condition.

Proposal 1: We propose to adopt option 3 for issue 1-1-1: power difference conditions. The power difference condition could be further increased beyond EPRE = 12 dB. In this case one more P-TRS occurrence should be allowed for AGC convergence.

Proposal 2: The wording [after the compensation for AGC] in [2] should be removed from the specification in [2].

Observation 2: For option 1, it is not obvious that the AGC state and time/frequency tracking states can be directly taken from the first SCell that has been brought up. For the clarification of the requirements in the CR, further lengthy studies may be needed.

Proposal 3: Requirements should be defined based on option 2 for issue 1-1-3: Multiple SSB-less SCell activation

Proposal 4: Since there is no reference, also no requirements for multiple SSB-less SCell activation should be defined for A-TRS in Rel-18.

# References

- R4-2410269, WF on RRM requirements for NR network energy saving, RAN4 #111, Fukuoka, Japan, May 20-24, 2024

- TS 38.133, Requirements for support of radio resource management (Release 18), v18.6.0, June 2024