# 3GPP TSG RAN WG4 Meeting#112	R4-2411722

# Maastricht, NL, Aug 19 – Aug 23, 2024

Source: 	Qualcomm Incorporated

Title: 	Discussion on NTN testing for NGSO

Agenda item:	8.8.4

Document for:	Discussion

# 1. Introduction

Currently NTN testing for NGSO assumes fixed Doppler or fixed delay shifts which are randomly selected before the test. However, it does not reflect the dynamic conditions that the UE supporting NGSO devices may experience in deployment scenarios. To this end, a more realistic channel model with varying Doppler and delay shifts that matches propagation trajectory of the NGSO satellites may be needed to ensure appropriate test coverage for these devices under dynamic conditions.

- Specify the TE-emulated channel model with varying Doppler and delay shifts for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands

  - Match the satellite motion trajectory based on the ephemeris for NGSO (Non-Geostationary Orbit) scenarios

- Specify UE demodulation performance requirement(s) of PDSCH for NGSO for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands based on the above channel model

  - Minimize the number of demodulation performance requirements

- Specify the applicability of the tests under the TE-emulated channel model

To this end, RAN4 has introduced a work item in Rel-19 to come up with a channel model considering varying Doppler and timing shifts for NR-NTN and IoT-NTN devices [1]. A subset of tasks related to demodulation performance are provided below [1].

2. TE-emulated Channel Model

In order to incorporate realistic time varying Doppler and delay shifts into the NTN channel models, RAN4 may need to consider a satellite propagator model, which is used to predict the future position and velocity of a satellite based on its current state and environmental factors. These models may consider gravitational forces, atmospheric drag, solar radiation pressure, and other perturbations to forecast the satellite's trajectory over time. There are several propagator models widely studied in the literatures, such as Keplerian propagators, Ekstein-Hechler propagators and generic numerical propagators.

Keplerian propagators uses simplified two-body equations of motion, assuming only central gravitational forces. This model is suitable for short-term predictions and simple orbits. On the other hand, the Ekstein-Hechler propagator is specifically designed to integrate the equation of motion of a satellite under various perturbations, such as Earth’s oblateness, atmospheric drag and can handle more complex orbital dynamics. Additionally, numerical propagators can accommodate a wide range of perturbation and complexities through various iterative methods. However, the numerical integration-based propagators typically require significantly higher computational complexity than analytical propagators.

The performance of these propagator models is mostly determined by the prediction window and accuracy. Besides, as the UE vendors may have to implement the propagator model in the channel emulator, the complexity of the propagator model must be considered.

Observation 1: The complexity of the propagator models needs to be considered as the UE vendors may have to implement the chosen model in their channel emulator.

Proposal 1: RAN4 to study the feasibility of introducing a reference propagator model for RAN4 requirement definition.

Considering the implementation complexity mentioned above, we prefer an analytical propagator model over a numerical integration-based model. Between the two widely used analytical propagator models (Keplerian and Eckstein-Hechler), we should consider the Eckstein-Hechler model for TE implementation. For instance, if time-varying time/frequency shift is modeled by TE based on the Keplerian model, a UE compensating the time/frequency shift based on the Eckstein-Hechler model might be unfairly penalized—not because of the performance of the propagator model implemented by the UE, but due to the inferior propagator model used for the satellite channel generation. To avoid discouraging UE vendors from implementing a more accurate propagator model, the satellite channel model should be defined based on the Eckstein-Hechler model.

On the other hand, for the purpose of performance requirement definition and alignment of the performance assessment across companies, we can consider the Keplerian propagator model. Note that this does not prevent UE from implementing a more accurate propagator model than Keplerian.

Proposal 2: RAN4 to consider the following reference propagator models.

- Eckstein-Hechler propagator model as a reference model for satellite mobility projection and ephemeris data generation from TE

- Keplerian propagator model as a reference model for UE performance evaluation in case performance alignment across companies is needed.

As part of test procedure, UE has to be provided with ephemeris of satellites via system information and/or RRC signal. Note that depending on the purpose of test cases, UE may need ephemeris information for more than one satellites, e.g. measurement and mobility test cases in RRM. And The ephemeris information must be updated every X sec for NTN networks, and the value of X cannot be larger than 10.24 due to the ambiguity of epoch time.

Proposal 3: Ephemeris information shall be updated based on the chosen reference propagator model for TE at least every 10.24sec. The exact frequency of ephemeris updates is subject to test configurations. The ephemeris information must be generated and provided to the UE for more than one satellite, as needed by test cases, using the same chosen reference propagator model.

Additionally, there may be a need to model both the service link and feeder link in the new channel model. Note that the time-varying time and frequency shift characteristics would differ if the synchronization reference point is not located at the satellite, i.e., the common timing advance value signaled by the cell is not zero.

Proposal 4: RAN4 to discuss whether to consider a non-zero time-varying feeder link delay as part of the new channel model.

2.1 Fading Channel and Satellite Mobility

In real deployment scenarios, satellite mobility will introduce time and frequency shifts. In the TE testing, such a behaviour can be replicated through the adoption of a reference propagator model at the TE side. To this end, it may be feasible to consider that the deterministic time and frequency shifts due to satellite mobility is decoupled from the fading channel. That is to say, the transmitted signals are passed through the channel fader, which are then shifted and distorted by the emulated time and frequency shifts according to the ephemeris information, UE location, and a reference propagator model. As for the fading channel, our view is that RAN4 can start with the Rel-17 NTN fading models, i.e., NTN TDLA and NTN TDLC fading models.

Proposal 5: RAN4 to decouple the deterministic time and frequency shifts due to satellite mobility from the traditional fading channel.

- RAN4 to strive to use the already adopted fading models, such as NTN TDLA and NTN TDLC fading models.

3. UE Performance Requirements for NGSO

The WID suggests specifying UE Performance Requirements for NGSO with the chosen channel models, one for TE satellite channel model (e.g. Eckstein-Hechler model) and the other for UE (e.g. Keplerian). We think that it is essential to first study the feasibility of introducing such reference propagator models for UE performance requirements. Keeping this in mind, our view is that such discussion can be deferred until after the choice of a propagator model and its potential impact on the performance requirements have been determined.

Proposal 6: Defer any discussion on the UE demodulation performance requirements (for both NR-NTN and IoT-NTN) until after the discussion around the propagator model has been settled and its potential impact on the demodulation performance requirements have been determined.

On the other hand, we acknowledge that UE RRM requirements have been defined in a way that the requirements shall be met by the UE even under realistic satellite mobility scenarios. For instance, the RRM requirements already took account for a certain level of error due to UE satellite projection inaccuracies, e.g. 30m.

Proposal 7: RAN4 should not relax the current RRM requirements (for both NR-NTN and IoT-NTN) for cases where satellite projection error has been taken into account, e.g. UL timing, mobility, etc. RAN4 can revisit some of core requirement and test cases if affected by the new channel model accommodating satellite mobility, e.g. RLM/BFD.

4. Applicability of the Tests

The WID also suggests specifying applicability of the tests under the TE-emulated channel model. In Rel-17, we have the several PDSCH demodulation performance requirements with different HARQ configurations and channel models. To minimize the test burden, it would be prudential to consider test applicability between different HARQ configurations and channel models.

Proposal 8: If new PDSCH demodulation performance requirement is introduced, consider applicability of tests between different HARQ configurations and channel models.

5. Conclusions

In this paper, we share our views on the performance requirements for NTN testing for UE supporting NGSO scenarios. The following have been proposed.

Observation 1: The complexity of the propagator models needs to be considered as the UE vendors may have to implement the chosen model in their channel emulator.

Proposal 1: RAN4 to study the feasibility of introducing a reference propagator model for RAN4 requirement definition.

Proposal 2: RAN4 to consider the following reference propagator models.

- Eckstein-Hechler propagator model as a reference model for satellite mobility projection and ephemeris data generation from TE

- Keplerian propagator model as a reference model for UE performance evaluation in case performance alignment across companies is needed.

Proposal 3: Ephemeris information shall be updated based on the chosen reference propagator model for TE at least every 10.24sec. The exact frequency of ephemeris updates is subject to test configurations. The ephemeris information must be generated and provided to the UE for more than one satellite, as needed by test cases, using the same chosen reference propagator model.

Proposal 4: RAN4 to discuss whether to consider a non-zero time-varying feeder link delay as part of the new channel model.

Proposal 5: RAN4 to decouple the deterministic time and frequency shifts due to satellite mobility from the traditional fading channel.

- RAN4 to strive to use the already adopted fading models, such as NTN TDLA and NTN TDLC fading models.

Proposal 6: Defer any discussion on the UE demodulation performance requirements (for both NR-NTN and IoT-NTN) until after the discussion around the propagator model has been settled and its potential impact on the demodulation performance requirements have been determined.

Proposal 7: RAN4 should not relax the current RRM requirements (for both NR-NTN and IoT-NTN) for cases where satellite projection error has been taken into account, e.g. UL timing, mobility, etc. RAN4 can revisit some of core requirement and test cases if affected by the new channel model accommodating satellite mobility, e.g. RLM/BFD.

Proposal 8: If new PDSCH demodulation performance requirement is introduced, consider applicability of tests between different HARQ configurations and channel models.

6. References

[1] RP-241281, “Revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN,” 3GPP TSG RAN WG Meeting #104, Shanghai, CN June 17 – 20, 2024.