# 3GPP TSG-RAN WG4 Meeting #112	                                 R4-2411723

# Maastricht, Netherland, 19 – 23 August, 2024

Agenda item:	8.19.2.3

Source: 	CableLabs, Charter Communications

Title: 	SBFD/TDD coexistence receiver in-band blocking

Document for:	Discussion

# 1. Introduction

Sub-band full duplex (SBFD) was studied in Release 18. TR 38.858 [1] was published at the end of the Rel-18 Study Item (SI). In Release 19, SBFD is extended as a Work Item (WI). SBFD/TDD coexistence study was presented in clause 11 in TR 38.858, where SINR and throughput degradation due to mutual interference are quantified. However, receiver in-band blocking was not discussed in clause 11 [1]. In the RAN4 #111 meeting in Fukuoka, Japan in May 2024, RAN4 agreed to further study BS receiver in-band blocking [2].

- Discussion

RAN4 only agreed to further study BS receiver in-band blocking [2], however, how to present the study method, assumptions, results, and impact are not discussed. It could be captured in TR 38.858 or a Rel-19 TR.

Proposal 1: the new BS receiver in-band blocking study should be captured in a technical report. We prefer Option 3.

Option 1: merge into clause 11 in TR 38.858.

Option 2: capture it in a new Rel-19 TR.

Option 3: capture it in a new Rel-19 TR, briefly summarize the conclusions in TR 38.858 and cite the Rel-19 TR.

The gNB-to-gNB adjacent channel interference (ACI) significantly impacts system performance in both case 2 (TDD as victim) and case 3 (SBFD as victim). The in-band blocking study currently focuses on case 3, it should be extended to case 2.

Proposal 2: The in-band blocking study should extend to case 2 because both case 2 and case 3 have strong gNB-to-gNB ACI. The results should be captured in clause 11 in TR 38.858.

RAN1 is working on mitigating the gNB-to-gNB co-channel CLI. SBFD co-channel interference at gNB includes self-interference, inter-sector interference, inter-cell UE-to-gNB co-channel interference, and gNB-to-gNB co-channel CLI. No matter how well the RAN1 interference mitigation technique performs, there is still some level of SBFD co-channel interference at gNB. The following statement in RAN4 #111 WF [2] is unreasonable: “Option-1: absolute power level of interference from adjacent channel only, i.e., SBFD network internal interference should not be considered in the simulation, from the perspective of definition of in-band blocking requirement.”

Proposal 3: It is unreasonable to only consider ACI for in-band blocking. Some level of co-channel interference should also be considered. How to consider co-channel interference is FFS.

An issue was listed in RAN4 #111 WF [2]

- FFS how to use the power level probability distributions to derive the in-band blocking levels

  - Option-1: Find the X% tile of the UL SBFD wideband received power CDF to define the blocking requirement in each of the down-selected scenarios.

    - FFS the value of X

    - FFS the impact of CLI handling scheme introduced in RAN1 which can be used for adjacent channel CLI mitigation.

Propose 4: the X% tile of the UL SBFD wideband received power CDF to define the blocking requirement should be X=99.

- In-band blocking simulation results

The BS receiver blocking model is described in section E.2.3.1 in [1]. IACI is the adjacent channel interference from BS transmitting on the DL channel in the aggressor network. The interference level is calculated as:

IACI=PTx,BSCLadjacent network BS→serving BS

Based on the agreement in RAN4 #111 WF [2] only the aggregated IACI is considered as the blocking power.

Based on the agreement in RAN4 #111 WF [2]: “Option-1: after RX subarray beamforming gain and before array gain, i.e, at TAB.” The victim SBFD gNB RX antenna gain in blocking calculation is 0 dBi.

## Simulation assumptions and results for scenario 1 (FR1 UMa-to-UMa)

There are multiple optional assumptions listed in Annex E in [1]. The assumptions selected in the blocking simulation include:

- Grid shift: 10%.

- LOS probability: Option 2: If the 2D distance between two BS is less than or equal to the ISD, set the LOS probability to X; X = 0.75. Otherwise, UMa (h_UT = 25 m), see TR 38.828.

- UE distribution: uniform, 20%indoor and 80% outdoor.

- BS transmit power: TDD 49 dBm and SBFD 49 dBm (baseline).

- Antenna configuration: TDD 8×8, and SBFD config 8×8.

The receiver noise figure models for FR1 wide area BS are:

- Baseline: P1=-43 dBm, P2=-25 dBm, F1=5 dB, F2=14 dB

- Optional (Enhanced NF model): P1=-33 dBm, P2=-15 dBm, F1=5 dB, F2=14 dB

Figure E.2.3.1-1: Receiver blocking model [1]

Observation 1: The CDF of the receiver blocking power for scenario 1 (FR1 UMa-to-UMa) case 3 (TDD DL to SBFD UL subband) is provided in Figure 1. 33% of the blocking power is larger than the P2=-25 dBm in the baseline NF model, and 100% of the blocking power is larger than the P2=-15 dBm in the enhanced NF model.

Figure 1. Scenario 1 (FR1 UMa-UMa), case 3 in-band blocking power.

## Simulation assumptions and results for scenario 5 (FR1 UMi-to-UMi)

There are multiple optional assumptions listed in Annex E in [1]. The assumptions selected in the blocking simulation include:

- Grid shift: 10%.

- LOS probability: Option 2: If the 2D distance between two BS is less than or equal to the ISD, set the LOS probability to X; X = 0.75. Otherwise, UMa (h_UT = 25 m), see TR 38.828.

- UE distribution: uniform, 20%indoor and 80% outdoor.

- BS transmit power: TDD 46 dBm and SBFD 46 dBm.

- Antenna configuration: TDD 2×2, and SBFD config 2×2.

The receiver noise figure model for FR1 medium range BS is P1=-38 dBm, P2=-20 dBm, F1=10 dB, F2=19 dB

Observation 2: The CDF of the receiver blocking power for scenario 5 (FR1 UMi-to-UMi) case 3 (TDD DL to SBFD UL subband) is provided in Figure 2. 35% of the blocking power is larger than the P2=-20 dBm in the NF model.

Figure 2. Scenario 4 (FR1 UMi-UMi), case 3 in-band blocking power.

## Simulation assumptions and results for scenario 6 (FR2-1 UMa-to-UMa)

There are multiple optional assumptions listed in Annex E in [1]. The assumptions selected in the blocking simulation include:

- Grid shift: 10%.

- LOS probability: Option 2: If the 2D distance between two BS is less than or equal to the ISD, set the LOS probability to X; X = 0.75. Otherwise, UMa (h_UT = 25 m), see TR 38.828.

- UE distribution: uniform, 20%indoor and 80% outdoor.

- BS transmit power: TDD 30 dBm and SBFD 30 dBm.

- Antenna configuration: TDD 8×16, and SBFD config 8×16.

The receiver noise figure model for FR2-1 wide area range BS is P1=-58 dBm, P2=-40 dBm, F1=10 dB, F2=19 dB.

Observation 3: The CDF of the receiver blocking power for scenario 6 (FR2-1 UMa-to-UMa) case 3 (TDD DL to SBFD UL subband) is provided in Figure 3. 99% of the blocking power is larger than the P2=-40 dBm in the NF model.

Figure 3. Scenario 4 (FR2 UMa-UMa), case 3 in-band blocking power.

- Conclusions

In this contribution, the SBFD gNB in-band blocking in an SBFD/TDD coexistence scenario simulation considerations are discussed. The simulation results for scenarios 1, 5, and 6 for case 3 are presented.

Proposal 1: the new BS receiver in-band blocking study should be captured in a technical report. We prefer Option 3.

Option 1: merge into clause 11 in TR 38.858.

Option 2: capture it in a new Rel-19 TR.

Option 3: capture it in a new Rel-19 TR, briefly summarize the conclusions in TR 38.858 and cite the Rel-19 TR.

Proposal 2: The in-band blocking study should extend to case 2 because both case 2 and case 3 have strong gNB-to-gNB ACI. The results should be captured in clause 11 in TR 38.858.

Proposal 3: It is unreasonable to only consider ACI for in-band blocking. Some level of co-channel interference should also be considered. How to consider co-channel interference is FFS.

Propose 4: the X% tile of the UL SBFD wideband received power CDF to define the blocking requirement should be X=99.

Observation 1: The CDF of the receiver blocking power for scenario 1 (FR1 UMa-to-UMa) case 3 (TDD DL to SBFD UL subband) is provided in Figure 1. 33% of the blocking power is larger than the P2=-25 dBm in the baseline NF model, and 100% of the blocking power is larger than the P2=-15 dBm in the enhanced NF model.

Observation 2: The CDF of the receiver blocking power for scenario 5 (FR1 UMi-to-UMi) case 3 (TDD DL to SBFD UL subband) is provided in Figure 2. 35% of the blocking power is larger than the P2=-20 dBm in the NF model.

Observation 3: The CDF of the receiver blocking power for scenario 6 (FR2-1 UMa-to-UMa) case 3 (TDD DL to SBFD UL subband) is provided in Figure 3. 99% of the blocking power is larger than the P2=-40 dBm in the NF model.

# 4. Reference

- TR 38.858, Study on Evolution of NR Duplex Operation, 3GPP, Dec. 2023.

- R4-2409889, Way Forward for [110bis][313] NR_duplex_evo, Samsung, May 2024.