3GPP TSG-RAN WG4 #112	R4-2411724

Maastricht, Netherlands, August 19-23, 2024

Agenda item:	8.21.2		RRM Core Requirements

Source: 	Qualcomm Incorporated

Title: 	On the RRM impact of enhanced network energy saving

Document for:	Discussion

# Introduction

In RAN4 #112, RAN4 will start working on the RAN1-led WI for Enhanced NES. The objectives of the updated WI as defined in [1] are as follows:

- Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4]

  - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling)

  - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum.

- Study procedures and signaling method(s) to support on-demand SIB1 for UEs in idle/inactive mode, including: [RAN1/2/3]

  - Triggering method by uplink wake-up-signal using an existing signal/channel.

  - Wake-up-signal configuration provisioning to UE

    - Note: No modification of SSB will be discussed under this objective

  - Information exchange between gNBs at least for the configuration of wake-up signal, if necessary.

  - Checkpoint for normative work in RAN#105

- Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4]

  - Adaptation of SSB in time domain, e.g. adapting periodicity

  - Adaptation of PRACH in time domain

  - Study adaptation of PRACH in spatial domain, e.g., non-uniform PRACH resources per SSB, and specify of found beneficial

    - This study is to be done in 2Q 2024 only

  - Adaptation of paging occasions including confining the paging occasions in the time domain

    - Note: there shall be no paging latency increase

  - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown

- Specify the corresponding core requirements, for the above features [RAN4].

It has been decided in RAN #104 that PRACH adaptation in spatial domain is removed from the WID [1].

The status of the work item is reported in [4] and [5]. In this paper, we share our views on the major topics of Rel-19 enhanced network energy saving and a preliminary assessment of the RRM impact.

# Discussion

The work item has three main topics:

- On-demand SSB SCell operation for connected mode

- On-demand SIB1 for idle/inactive mode

- Adaptation of SSB/PRACH in time domain and adaptation of paging occasion

Adaptation of PRACH in spatial domain has been removed from the WID [1].

In the following we share our views on the RRM work plan and the impact to the RRM core requirements for on-demand SSB and SSB/PRACH adaption in time domain.

## On-Demand SSB

So far, it has not been decided in RAN1 whether the on-demand SSB can be defined for a CD-SSB. Currently, the following options are still FFS in RAN1

- Alt-1: it is up to gNB implementation whether on-demand SSB is cell-defining or not

- Alt-2: On-demand SSB is limited to non-cell-defining SSB

That means that on-demand SSB is at least supported for NCD-SSB but whether on-demand SSB can also be CD-SSB has not yet been agreed in RAN1.

Observation 1: On-demand SSB is supported for NCD-SSB, but it has not yet been agreed in RAN1 that on-demand SSB can also be CD-SSB

Proposal 1: RAN4 should start working on requirements for on-demand SSB that are NCD-SSB. Defining requirements for on-demand SSB that are CD-SSB should be postponed till RAN1 has decided that Alt-1 above is supported.

RAN1 has agreed that on-demand can be triggered at least for the following four cases

- Case #1: SCell without always-on SSB, SCell is configured but deactivated

- Case #1A: SCell without always-on SSB, SCell receives activation command

- Case #2: SCell with always-on SSB, SCell is configured but deactivated

- Case #2A: SCell with always-on SSB, SCell receives activation command

Figure 2-1 and Figure 2-2 illustrate the four cases. In Case #1 and Case #2, on-demand SSB is transmitted when the SCell is still deactivated. In Case #1A and Case #2A, on-demand SSB triggering and SCell activation happens at the same time. The differentiation between Case #1 / #1A and Case #2 / #2A is that in Case #2 / #2A also an always-on SSB is present while in Case #1 / #1A there is no always-on SSB.

Figure 2-1: On-demand SSB in Case #1 and Case #1A

Figure 2-2: On-demand SSB in Case #2 and Case #2A

According to the WID in [1], on-demand SSB is supported only for connected mode when the Scell is deactivated or the SCell activation is ongoing but not in case when the UE is in connected mode and the SCell is already activated or when the UE is in idle/inactive state.

In the previous releases, multiple requirements for SCell activation have been introduced, see in section 8.3 in [2]. It should be analyzed on a case-by-case basis to which of them on-demand SSB can be applied to. For example, Rel-17 fast SCell activation and Rel-19 SCell activation with on-demand SSB seem to serve the same purpose, i.e., the acceleration of SCell activation. The difference is that in Rel-17 A-TRS while in Rel-19 on-demand SSB is used to enhance convergence of AGC and time/frequency synchronization. How to combine both is not obvious. Potentially also additional RAN1/RAN2 work would be needed, e.g., to define new DCI and/or MAC CE commands.

Observation 2: Not all existing cases for SCell activation in [2] may be compatible with Case #1A and Case #2A above.

Proposal 2: RAN4 should study which SCell activation scenarios in [2] can work together with on-demand SSB and define priorities which of them should be enhanced.

As a starting point, but not limited to, Rel-15 SCell activation should be the first focus (chapter 8.3.2 in [2]). RAN4 work should focus on the cases #1A and #2A above first to define SCell activation requirements for on-demand SSB.

Proposal 3: RAN4 should define requirements for SCell activation for on-demand SSB for cases #1A and #2A, i.e., joint triggering of SCell activation and on-demand SSB. First focus should be Rel-15 SCell activation, i.e., start with chapter 8.3.2 in [1] as a baseline.

Depending on whether the deactivated SCell is unknown or known, the requirements for SCell activation based on on-demand SSB will differ. In case #1A, no SSB is presented at all before SCell activation and the SCell is expected to be unknown. In case #2A, however, the SCell can be known or unknown depending on the configuration and detectability of the on-demand SSB. One condition for the cell to be known in FR1 is that is has sent a valid measurement report during the period max(5*measCycleSCell, 5*DRX cycles).

This measurement report is a L3 measurement since the UE is not supposed to send a L1-RSRP measurement report when the SCell is deactivated since the UE is not supposed to send a CSI report as long as the cell is deactivated and L1-RSRP belongs to CSI report framework. That means that in case of L1 measurements, either the SCell is always unknown or a new condition for a cell to be known needs to be discussed and agreed in RAN4. However, this would also imply RAN2 impact since TS 38.321 defines the tasks of a UE when the cell is deactivated [3].

It should also be noted that it is still under discussion in RAN1 whether L3 measurement reports for on-demand SSB will be introduced. So far, only L1 measurement reports are agreed in RAN1.

Observation 3: In case of L1 measurements, either the SCell is always unknown or a new condition for a cell to be known needs to be discussed and agreed in RAN4. However, this would also imply RAN2 impact.

Proposal 4: RAN4 should investigate the conditions for known and unknown SCell depending on the configuration of the on-demand SSB and the configuration of the measurement report and define the SCell activation requirements accordingly.

On-demand SSB has not yet been agreed in RAN1 when SCell is already activated. Hence, RAN4 can wait till RAN1 has discussed this case further.

Proposal 5: Postpone any RAN4 work on on-demand SSB during activated SCell till RAN1 agrees to support this scenario.

## On-Demand SIB1

On-demand SIB1 is a study objective till RAN #105 and not many agreements have been achieved in RAN1. Hence, RAN4 work for on demand SIB1 should be postponed at least till the plenary decision and till RAN1 work has made more progress

Proposal 6: Postpone RAN4 work on on-demand SIB1 till the RAN #105 plenary decision and RAN1/RAN2 work has made more progress.

## Adaptation of Common Signals and Channels

So far, not many agreements have been for adaptation of SSB and PRACH. For SSB, it has been agreed that adapting the periodicity is applicable for cells with both legacy UEs and Rel-19 NES-capable UEs. For the Rel-19 NES-capable UE, it is still open whether SSB adaptation applies to

- PCell (connected mode) or SCell

- CD-SSB or NCD-SSB

- SSB is on the sync-raster or not

It has also not yet been agreed whether SSB adaptation can be applied to Rel-19 NES-capable UEs that are in idle/inactive mode. That implies that SSB adaptation is applicable to Rel-19 NES-capable UEs in connected mode, but also other conditions are still open in RAN1. RAN4 should start looking at such requirements for SSB adaptation that apply for UE in connected mode and do not imply that the SSB is cell-defining.

Proposal 7: RAN4 should start investigating requirements for SSB adaptation for UEs in connected mode that do not imply that the SSB is a CD-SSB.

For PRACH, it has been agreed in RAN1 that additional PRACH resources are provided to the Rel-19 NES-capable UE in addition to PRACH resources to legacy UEs. Also, PRACH adaptation is supported for UEs both in connected as well as in idle/inactive mode. But beyond that not many agreements have been made in RAN1 for PRACH adaptation

Observation 4: PRACH adaptation applies for UE in connected as well as in idle/inactive mode. Not many additional agreements have been made for PRACH adaptation.

RAN4 already defines minimum requirements for the measurement procedures at transitions, e.g., in case that the intra-frequency measurement object transits from measurements outside gaps to measurements inside gaps or when the DRX cycle periodicity changes, see section 9.1.6 in [2]. Change of the periodicity of SSB and PRACH is a similar transition. Hence, RAN4 should investigate what requirements are impacted by those periodicity transitions by SSB and PRACH adaptation in time domain, e.g., the measurement procedure for SSB based measurements.

Observation 5: RAN4 already defines minimum requirements for the measurement procedures at transitions in the configurations. Change of the periodicity of SSB and PRACH is a similar transition.

Proposal 8: RAN4 should investigate what requirements are impacted by those periodicity transitions by SSB and PRACH adaptation in time domain.

No agreements for paging occasion adaptation have yet been made in RAN1. Also, not many agreements in RAN2 have yet been achieved that allow to start RAN4 work. Hence, it is proposed to postpone RAN4 work till the agreements in RAN1 and RAN2 have become more mature.

Proposal 9: Postpone RAN4 work on paging occasion adaptation in time domain till more progress in RAN1 and RAN2 has been made.

# Conclusions

In this paper we provided our views on the major topics of enhanced network energy saving and a preliminary assessment of its RRM impact. Our observations and proposals are summarized as follows:

On-Demand SSB

Observation 1: On-demand SSB is supported for NCD-SSB, but it has not yet been agreed in RAN1 that on-demand SSB can also be CD-SSB

Proposal 1: RAN4 should start working on requirements for on-demand SSB that are NCD-SSB. Defining requirements for on-demand SSB that are CD-SSB should be postponed till RAN1 has decided that Alt-1 above is supported.

Observation 2: Not all existing cases for SCell activation in [2] may be compatible with Case #1A and Case #2A above.

Proposal 2: RAN4 should study which SCell activation scenarios in [2] can work together with on-demand SSB and define priorities which of them should be enhanced.

Proposal 3: RAN4 should define requirements for SCell activation for on-demand SSB for cases #1A and #2A, i.e., joint triggering of SCell activation and on-demand SSB. First focus should be Rel-15 SCell activation, i.e., start with chapter 8.3.2 in [1] as a baseline.

Observation 3: In case of L1 measurements, either the SCell is always unknown or a new condition for a cell to be known needs to be discussed and agreed in RAN4. However, this would also imply RAN2 impact.

Proposal 4: RAN4 should investigate the conditions for known and unknown SCell depending on the configuration of the on-demand SSB and the configuration of the measurement report and define the SCell activation requirements accordingly.

Proposal 5: Postpone any RAN4 work on on-demand SSB during activated SCell till RAN1 agrees to support this scenario.

On-Demand SIB1

Proposal 6: Postpone RAN4 work on on-demand SIB1 till the RAN #105 plenary decision and RAN1/RAN2 work has made more progress.

Adaptation of Common Signals and Channels

Proposal 7: RAN4 should start investigating requirements for SSB adaptation for UEs in connected mode that do not imply that the SSB is a CD-SSB.

Observation 4: PRACH adaptation applies for UE in connected as well as in idle/inactive mode. Not many additional agreements have been made for PRACH adaptation.

Observation 5: RAN4 already defines minimum requirements for the measurement procedures at transitions in the configurations. Change of the periodicity of SSB and PRACH is a similar transition.

Proposal 8: RAN4 should investigate what requirements are impacted by those periodicity transitions by SSB and PRACH adaptation in time domain.

Proposal 9: Postpone RAN4 work on paging occasion adaptation in time domain till more progress in RAN1 and RAN2 has been made.

# References

- RP-241650, Revised WID: Enhancements of network energy savings for NR, RAN #104, June 2024

- TS 38.133, Requirements for support of radio resource management, v18.6.0, June 2024

- TS 38.321, Medium Access Control (MAC) protocol specification, v18.2.0, June 2024

- RP-240169, Status Report Enhancements of network energy savings for NR, RAN #103 March 2024

- RP-241182, Status Report Enhancements of network energy savings for NR, RAN #104 June 2024