3GPP TSG-RAN WG4 Meeting # 112                                                                 R4-2411727

Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:	8.10.2.2

Source: 	CMCC

Title: 	(NR_ATG_enh-Core) Discussion on UE RF requirements for ATG with inter-band CA

Document for:	Discussion

# 1. Introduction

RAN#103 meeting approved New WID on Enhancements for Air-to-ground network for NR in Rel-19. RF core requirements for intra-band co-located and inter-band co-located DL CA shall be specified in this WID. This paper will focus on the UE RF requirements for ATG with inter-band CA.

# 2. Discussion

## 2.1. BCS

The description for ATG BS with DL CA in the WID is shown as below:

- Specify the RF and RRM core requirements for intra-band co-located and inter-band co-located DL CA [RAN4]: - FR1 intra-band contiguous CA - Example band combination: n79C - FR1+FR1 inter-band CA - Example band combination: n3+n39

For the inter-band CA combination CA_n3-n39, we noticed that this band combination has been specified in the current specification. The table in TS 38.101-1 could be the starting point for BCS of CA_n3-n39. However, the approved TP of CA_n3-n39 only considered UL n3. In the operators’ opinion, to not limit the flexibility of future ATG frequency use, we should reserve the possibility of using band n39 UL.  Besides, considering the difference in RF architecture between ATG CPE and TN UE, the RF architecture of ATG UE to achieve n39 DL should be discussed independently in this work item. If the RF architecture is feasible, UL n39 should be supported

Proposal 1: CA_n3-n39 with UL n39 should not be precluded.

Proposal 2: Table 1 as the starting point for BCS of CA_n3A-n39A.

Table 1: BCS for CA_n3A-n39A

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n3A-n39A | - | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n39 | 5, 10, 15, 20, 25, 30, 35, 40 |  |

## 2.2. antenna type

During last meeting, we have reached the agreement as below.

Agreement: • No limitation on antenna types for ATG CA • FFS on whether to assume omni-antenna type can be assumed for both band n3 and n39 in DL CA_n3-n39 • FFS on whether new capability is needed for ATG CA

Considering the implementation freedom for ATG UE, RAN4 agreed to introduce two ATG UE types to distinguish the antenna types for ATG UE (for 2 GHz, to use the omni antenna for ATG UE, and for 4 GHz, to use the antenna array for ATG UE). For band combination CA_n3-n39, the frequency range of these two bands is around 2GHz, and the omni antenna should be assumed for this band combination. But from a deployment flexibility perspective, we recommend prioritizing the UE RF architecture supported by both antenna types.

Proposal 3: Omni-antenna type could be assumed for both band n3 and n39 in DL CA_n3-n39.

As for the new capability for ATG CA, we think there is no need to define it. Since ATG CA is implemented based on hardware capabilities and specified per band.

Proposal 4: No need to define new capability for ATG CA.

## 2.3 Rx requirements for inter-band CA

Since the TP of CA_n3-n39 had been approved, RIB,c are shown as below.

Table 3: ΔRIB,c

| Inter-band CA combination | ΔRIB,c for NR bands (dB)* |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration** |  |
| CA_n3-n39 | - | - |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

Considering the difference between ATG UE and TN UE RF architecture, such as different antenna types and different antenna sizes, we think delta RIB could be used as the baseline.

Proposal 5: delta RIB in TS 38.101-1 could be used as the baseline.

Based on the reference RF architecture for CA_n3-n39, the MSD is shown as below. Since the RF architecture of this band combination on the aircraft has not yet been determined, the MSD value could not be reused directly.

Table 4: MSD test configuration due to cross band isolation for CA_n3-n39

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n3 | n39 | 1770 | 30 | 15 | 50 (RBstart=110) | 1882.5 | 5 | 2.1 dB | >ACLR2 |

Proposal 6: RF architecture for CA_n3-n39 on CPE should be first discussed clearly before discussing MSD value.

According to the TP, there is no need to specify OOB blocking exception for CA_n3-n39, but some clarification about how to apply FDL_low and FDL_high. Referring to CA_20A-28A, the following note can be introduced into table 7.6.3-2 of TS 38.101-1. So there is no need to OOB blocking exception for CA_n3-n39, but the OOB blocking requirements could be reused if other CA band combinations are introduced in the future

NOTE 11: For a UE supporting CA_n3A-n39A and higher order band combinations in which CA_n3A-n39A is a subset, the requirements for Band n3 and Band n39 apply with FDL_low given by the lower limit of the restricted operating frequency range in Band n3 and FDL_high by Band n39.

Proposal 7: OOB blocking requirements could be reused, and there is no need to specify OOB blocking exception for CA_n3-n39.

# 3. Conclusions

In this contribution, ATG UE with inter band CA requirements are discussed with the following proposals:

Proposal 1: CA_n3-n39 with DL n39 should not be precluded.

Proposal 2: Table 1 as the starting point for BCS of CA_n3A-n39A.

Table 1: BCS for CA_n3A-n39A

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n3A-n39A | - | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n39 | 5, 10, 15, 20, 25, 30, 35, 40 |  |

Proposal 3: Omni-antenna type could be assumed for both band n3 and n39 in DL CA_n3-n39.

Proposal 4: No need to define new capability for ATG CA.

Proposal 5: delta RIB in TS 38.101-1 could be used as the baseline.

Proposal 6: RF architecture for CA_n3-n39 on CPE should be first discussed clearly before discussing MSD value.

Proposal 7: OOB blocking requirements could be reused, and there is no need to specify OOB blocking exception for CA_n3-n39.

# 4. References

[1] RP-240839, New WID on Enhancements for Air-to-ground network for NR