3GPP TSG-RAN WG4 Meeting # 112                                    R4-2411730

Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:	8.22.2.1

Source: 	CMCC

Title: 	(NR_LPWUS-Core) Discussion on LP-WUS UE system parameters requirements

Document for:	Discussion

# 1. Introduction

In RAN plenary #103 meeting, revised WID of LP-WUS is approved with following RF part objectives:

- Specify the necessary RAN4 core requirement(s) to support the feature (RAN4).

  - Specify UE low-power wake-up receiver requirements, at least REFSENS, ACS and ASCS requirements with consideration of possible new methodology to assess the low-power wake-up receiver performance

    - Define guard RBs for ACS and ASCS cases

    - Study testability of above requirements

    - Consider impacts of different architecture and impairments, and set requirements that enable all types of reasonable implementation

  - Study and if necessary specify or support by declaration, the corresponding BS requirements, e.g., dynamic range for LP-WUS/LP-SS.

    - Current NR BS requirements is baseline

In RAN4#110-bis, the LP-WUS RF requirements had been discussed and the WF was approved in [1]. This paper will focus on some issue about system parameters.

- Discussion

2.1 one or multi-Sets of requirements

For LP-WUS, its waveform and modulation mode are much different from the legacy NR signal which will impact the final PAPR. Different LP-WUS signal means different categories of receivers, considering the difference between these two receivers, it is difficult to define one set of requirements to meet all RF requirements. According to the current research progress of RAN1, OOK receivers use envelope detection, while OFDM receivers use sequence detection. In addition, OFDM receivers have better performance, which also means that while ensuring the same coverage distance of Msg3, OFDM receivers have better SNR and NF. To accommodate implementation flexibility, it is better to set requirements that could cover all implementations, two sets of RF requirements are needed for LP-WUS, such as NF and SNR.

Proposal 1: Two sets of requirements could be set for OOK-based receivers and OFDM-based receivers.

Proposal 2: SNR and NF could be different for these two types.

### 2.2 Channel raster for LP-WUR

Issue 1-1-7: Channel raster for LP-WUR Agreements: RAN4 further check whether no channel raster is needed for LP-WUR.

Both in-band case and standalone case should be considered at this stage. For the standalone case, the center frequency should correspond to the channel raster frequency points of the corresponding working frequency bands. For in-band case, since LP-WUS RBs will be transmitted together with NR RBs, the existing requirements of NR channel raster could be reused for LP-WUS. In summary, channel raster is needed and the existing requirements could be reused.

Proposal 3: Channel raster is needed for LP-WUS and the existing requirements could be reused.

### 2.3 Channel bandwidth and RB numbers

During the last meeting, we reached the agreement that 11PRB for LP-WUS signal with 30kHz SCS, and for RB number for 15KHz SCS depending on RAN1 decision. RAN1 had two options for the number of RBs, 11 RBs and 22 RBs, and these two options should be considered by RAN4.

Proposal 4: Consider 11RBs and 22RBs for LP-WUS signal with 15kHz SCS

# 3. Conclusions

Proposal 1: Two sets of requirements could be set for OOK-based receivers and OFDM-based receivers.

Proposal 2: SNR and NF could be different for these two types.

Proposal 3: Channel raster is needed for LP-WUS and the existing requirements could be reused.

Proposal 4: Consider 11RBs and 22RBs for LP-WUS signal with 15kHz SCS

# 4. Reference

- R4-2410569, WF for [111][136] NR_LPWUS_UERF