3GPP TSG-RAN WG4 Meeting # 112                                                               R4-2411731

Maastricht, Netherlands, 19th – 23rd August, 202

Agenda item:	8.22.2.2

Source: 	CMCC

Title: 	(NR_LPWUS-Core) Discussion on LP-WUS UE RF Rx requirements of REFSENS, ASCS and ACS

Document for:	Discussion

# 1. Introduction

In RAN plenary #103 meeting, revised WID of LP-WUS is approved with following RF part objectives:

- Specify the necessary RAN4 core requirement(s) to support the feature (RAN4).

  - Specify UE low-power wake-up receiver requirements, at least REFSENS, ACS and ASCS requirements with consideration of possible new methodology to assess the low-power wake-up receiver performance

    - Define guard RBs for ACS and ASCS cases

    - Study testability of above requirements

    - Consider impacts of different architecture and impairments, and set requirements that enable all types of reasonable implementation

  - Study and if necessary specify or support by declaration, the corresponding BS requirements, e.g., dynamic range for LP-WUS/LP-SS.

    - Current NR BS requirements is baseline

In this contribution, we focus on discuss of UE LP-WUS RF requirements including REFSENSE, ACS and ASCS.

- Discussion

2.1 REFSENSE requirements

During the last meeting, RAN4 reached the following agreement below. Questions about performance metrics, SNR, and NF still need to be discussed.

Issue 2-2-3: SNR value (not requirement) for REFSENS Agreement: - Stick to last meeting agreements: Wait for RAN1 SNR progress Issue 2-2-4: NF Gap between LR and MR for REFSENS (assume MR as 9dB) Agreement: - RAN4 further discuss potential NF for OOK-based and OFDM-based receiver. - Linked to resolution of Issue 2-2-8 Issue 2-2-6: IM value for REFSENS Agreement: - RAN4 further discuss potential IM value for OOK-based and OFDM-based receiver. Issue 2-2-8: Whether a baseline architecture is needed for LP-WUS receiver Agreement: - RAN4 will further discuss and decide baseline architecture(s) for OOK-based receiver and OFMD-based receiver.

SNR value for REFSENS

Considering SNR is relevant to RF and demodulation and RAN1 has two options of OFDM signal and OOK signal, the required SNR for demodulation on OOK and OFDM sequence could be different, so two sets of SNR values could be specified.

Proposal 1: Specify two sets of SNR for LP-WUS.

NF Gap between LR and MR

Since RAN1 had an agreement about SNR evaluation:

Agreement: For evaluation of LP-WUS and LP-SS design to achieve coverage of PUSCH for message3 from RANl perspective, at least the following SNR values should be considered: - SNR= -3dB for NF of LR=NF of MR+ 8dB - SNR= -0.5dB for NF of LR = NF of MR+ 5dB - SNR=2dB for NF of LR=NF of MR+ 2dB Note l: The NF of MR is assumed as 7dB

Considering the difference between OFDM and OOK signals, two sets of noise figure requirements should be specified. For the OFDM signal, the NF value of the OFDM receiver and legacy UE receiver are similar, so we could assume the legacy UE receiver noise figure value 9dB as the baseline for the OFDM signal.

Proposal 2: Specify two sets of noise figure values for LP-WUS noise figure.

Proposal 3: Set 9dB as the OFDM noise figure for the OFDM signal as the baseline.

For the OOK signal receiver, the NF value is much larger than the NF of legacy UE. During the study item phase, we listed some example NF value ranges:

- RF envelope detection: The noise figure is in the range of 12~22 dB.

- Heterodyne architecture with IF envelope detection: The noise figure is in the range of 9~15 dB.

- Homodyne/zero-IF architecture with baseband envelope detection: The noise figure is in the range of 10~16 dB.

Consider there are three value ranges for different envelope detection. We could choose the same NF range between these three frequency ranges for the LP-WUS signal, which is 12-15dB. Since RAN1 chooses 7dB as the NF of MR, and the gap between MR and LR is 2/5/8dB, it is reasonable to choose 12-15dB as the noise figure for the OOK signal.

Proposal 4: Choose 12-15 noise figure as the baseline for the OOK signal.

For legacy NR main radio receiver, 2.5dB has been used. Considering the LP-WUS implementation, 2.5dB could be used as baseline. Since there is no interference from its own transmitter, the IM requirement of LP-WUS could be set to less than 2.5dB.

Proposal 5: Legacy IM for NR main radio could be used as baseline.

2.1 ASCS and ACS

About how to define ASCS, the following two factors should both be taken into consideration, the ASCS value in dB scale and the required guard RB. If we only define guard RB requirement without any ASCS value definition, there is no restriction of LP-WUR in-band selectivity. In extreme cases, LP-WUR with the worst performance may also pass the test since there is only guard RB limitation. If we only define ASCS value without any guard RB limitation. In extreme cases, LP-WUR with the worst performance could also pass the test as long as guard RB is large enough.

ACS and ASCS could not be specified directly, so we should define the test case of ASCS requirement in the spec. The test parameter should include power level, bandwidth, MDR and some other parameters. The MDR will be checked in the test with the guard BRs defined in the spec.

Proposal 6: For ASCS requirement, RAN4 should specify the guard RB and test case to check whether the MDR is within X%.

# 3. Conclusions

In this contribution, ASCS, ACS and REFSENSE are discussed with following observations and proposals.

Proposal 1: Specify two sets of SNR for LP-WUS.

Proposal 2: Specify two sets of noise figure values for LP-WUS noise figure.

Proposal 3: Set 9dB as the OFDM noise figure for the OFDM signal as the baseline.

Proposal 4: Choose 12-15 noise figure as the baseline for the OOK signal.

Proposal 5: Legacy IM for NR main radio could be used as baseline.

Proposal 6: For ASCS requirement, RAN4 should specify the guard RB and test case to check whether the MDR is within X%.

# 4. Reference

- R4-2410569, WF for [111][136] NR_LPWUS_UERF