3GPP TSG-RAN WG4 Meeting # 112                                    R4-2411733

Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:	8.22.3

Source: 	CMCC

Title: 	(NR_LPWUS-Core) Discussion on LP-WUS BS RF requirements

Document for:	Discussion

# 1. Introduction

In RAN plenary #103 meeting, revised WID of LP-WUS is approved with following RF part objectives:

- Specify the necessary RAN4 core requirement(s) to support the feature (RAN4).

  - Specify UE low-power wake-up receiver requirements, at least REFSENS, ACS and ASCS requirements with consideration of possible new methodology to assess the low-power wake-up receiver performance

    - Define guard RBs for ACS and ASCS cases

    - Study testability of above requirements

    - Consider impacts of different architecture and impairments, and set requirements that enable all types of reasonable implementation

  - Study and if necessary specify or support by declaration, the corresponding BS requirements, e.g., dynamic range for LP-WUS/LP-SS.

    - Current NR BS requirements is baseline

In RAN4#110-bis, the LP-WUS BS RF requirements had been discussed and the WF was approved in [1]. This paper will focus on some issue about LP-WUS BS RF requirements.

- Discussion

2.1 Applicable BS type for LP-WUS

FFS on the following proposals for next meeting: • Proposal 1: Not to set restriction on applicable BS types to support LP-WUS. • Proposal 2: To decide on the applicable BS types after the applicable frequency range and bands for LP-WUS have been decided. • Proposal 3: Consider BS type 1-C as applicable type to further discuss of LP-WUS power boosting. FFS other BS types. • Proposal 4: Depend on power boosting level for certain BS type.

To ensure the integrity of LP-WUS spec and meet all companies’ deployment demand, We had reach the agreement during last meeting that not to limit the example bands for LP-WUS. Based on this view, we think it is better not to limit the applicable BS types for LP-WUS.

Proposal 1: Not to set restriction on applicable BS types to support LP-WUS.

Following lists the multiple bands output from study item phase. It seems for LP-WUR using RF envelop detection architectures, it is more appropriate to support only single band operation. For different LP-WUR RF architecture, minimum supported operation bands number is different.

RF envelop detection architecture is more appropriate for single-band operation. IF envelop detection and BB envelop detection are more appropriate for multi-bands operation. Multi-band here still means that only one band at a time is being received. Multi-band capability for other architectures is FFS.

To reduce LP-WUR cost, usually one individual LP-WUR would only support one or several operation bands. The same logic applies for gNB side, supporting only one operation band for LP-WUR seems also OK for most case. But one issue is that when the roaming UE enters to current network but it doesn’t support current LP-WUS operation band. So it seems supporting multi-band would be more reasonable. Besides, from gNB implementation point of view, the RF part of legacy gNB could be reused to support LP-WUS and the cost to support multiple bands LP-WUS would not be increased at gNB side. So it’s better to also define multi-band requirements at gNB side.

As analyzed above, to reduce LP-WUR cost, usually one individual LP-WUR would only support one or several operation bands. The same logic applies for gNB side, supporting only one operation band for LP-WUR seems also OK for most case. But one issue is that when the roaming UE enters to current network but it doesn’t support current LP-WUS operation band. So it seems supporting multi-band would be more reasonable. Besides, from gNB implementation point of view, the RF part of legacy gNB could be reused to support LP-WUS and the cost to support multiple bands LP-WUS would not be increased at gNB side. So it’s better to also define multi-band requirements at gNB side.

Proposal 2: it’s suggested to also define multi-band requirements at gNB side for LP-WUS.

2.2 LP-WUS power boosting

There are still some issues should to be discussed:

1.1 Manufacture declaration on LP-WUS WF FFS on the following options for next meeting: - Option 1: Set the LP-WUS power boosting a complete manufacture declaration feature, including whether supporting LP-WUS power boosting and the supported boosting level. - Option 2: Minimum power boosting level in core specification together with manufacturer declaration in the conformance test specification. 1.3. Whether to preclude small CBW for consideration of LP-WUS power boosting WF FFS on the following options for next meeting: - Option 1: Consider a power degradation limit, e.g., 2dB, for validating a configuration for LP-WUS representing by (EPRE ratio, channel bandwidth). - Option 2: Only consider LP-WUS power boosting for CBWs larger than 20MHz. - Option 3: Focus on CBWs >= 10MHz and different power boosting values can be considered for different CBW. - Option 4: Not preclude small CBW for LP-WUS power boosting.

The main reason for supporting power boosting is to improve the coverage of Msg3. As of the last meeting, RAN1 hasn't decided whether to support power boosting or not, and RAN1 listed the following approach to achieve the coverage of Msg 3.

Companies propose the following schemes to improve the coverage achieved by LP-WUS and LP-SS: - Power boosting, which may not be always available for all gNBs - Time domain repetition - Spatial diversity with time domain repetition, which requires to be used with time domain repetition and precoder is transparent to OOK based receiver - Frequency domain diversity with time domain repetition - Time domain spreading code - Multiple beam transmissions/beam sweeping

Observation 1: RAN1 listed several schemes to improve the coverage of Msg3, and power boosting has not yet been decided to be used.

First of all, we don't think any bands should be excluded from LP-WUS. It means that the bandwidth of the operation band should not be limited.

Proposal 3: CBW for MR should not be limited.

Concerning current RAN1 simulations, for low frequency bands, no other schemes are needed to achieve coverage. For the middle frequency band, some schemes such as power boosting are needed to improve the WUS signal coverage. Considering there is no clear conclusion from RAN1 on whether to adopt power boosting, and different frequency bands or bandwidths have different boosting levels, it is suggested that the LP-WUS power boosting is a complete manufacture declaration feature.

Proposal 4: Set the LP-WUS power boosting a complete manufacture declaration feature.

# 3. Conclusions

Proposal 1: Not to set restriction on applicable BS types to support LP-WUS.

Proposal 2: it’s suggested to also define multi-band requirements at gNB side for LP-WUS.

Observation 1: RAN1 listed several schemes to improve the coverage of Msg3, and power boosting has not yet been decided to be used.

Proposal 3: CBW for MR should not be limited.

Proposal 4: Set the LP-WUS power boosting a complete manufacture declaration feature.

# 4. Reference

- R4-2409956, Way Forward for [111][314] NR_LPWUS