3GPP TSG-RAN WG4 Meeting #112											R4-2411735

Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:	8.19.2.3

Source: 	CMCC

Title: 	(NR_duplex_evo-Core) Discussion on existing Rx requirements for SBFD

Document for:	Discussion

# 1. Introduction

In TR 38.858, the impacts on SBFD requirements have been analyzed. RAN4 has identified which legacy requirements are still applicable, which legacy requirements are not applicable and which new requirements are needed.

In last meeting, WF on SBFD BS requirement is approved with several agreements. [1]

In this contribution, we continue the RF requirements discussion based on the output of study item phase.

- Discussion

2.1 ACS requirements

Following capture the agreements from TR 38.858 for ACS requirements:

- ACS requirement and the interference level shall be determined by RAN4 co-existence study, and for the definition of ACS requirement: - - Conducted ACS: Take the existing wanted signal of ACS requirement by using the existing reference sensitivity level. - - OTA ACS: The OTA sensitivity degradation shall be taken into account to determine the level of wanted signal and interference signal mean power.

Similar to ACLR requirements, adjacent-channel co-existence simulation only focuses on not less than 10% grid shift values, and 0% grid shift is avoided. If we do not define co-location ACS requirement. Add the note in the spec that the ACS requirement is not applied to the co-location scenario.

Proposal 1: If co-location ACS is not defined, the note in the spec that the ACS requirement is not applied to the co-location scenario is needed.

2.2 In-band blocking

Last meeting we have approved the WF below:

- Way forward: - Companies are encouraged to provide the evaluation results for in-band blocking in the next meeting: - The scenarios to be studied are Scenario 1, 3, 5, 6, 9, defined in TR 38.858 - The case to be studied is Case 3 (TDD DL to SBFD UL subband). - FFS the power level which should be collected from the simulation results to derive in-band blocking levels: - Option-1: absolute power level of interference from adjacent channel only, i.e., SBFD network internal interference should not be considered in the simulation, from the perspective of definition of in-band blocking requirement. - Other options are not precluded. - FFS the reference point for the power level which should be before or after RX beamforming. - Option-1: after RX subarray beamforming gain and before array gain, i.e, at TAB. - FFS different grid-shift values should be considered. - Option-1: 10%. - Option-2: Other percentage values are not precluded. - FFS how to use the power level probability distributions to derive the in-band blocking levels - Option-1: Find the X% tile of the UL SBFD wideband received power CDF to define the blocking requirement in each of the down-selected scenarios. - FFS the value of X - FFS the impact of CLI handling scheme introduced in RAN1.

From the perspective of definition of in-band blocking requirement, SBFD network internal interference should not be considered in the simulation. So when we collect power level from simulation result, only collect power level of interference from adjacent channel.

Proposal 2: Only collect power level of interference from adjacent channel to derive in-band blocking levels.

The reference point for the power level should be before Rx beamforming, since the reference point is before in the previous simulation. And we suggest to consider 100% grid-shift.

Proposal 3: the reference point for the power level should be before RX beamforming

Proposal 4: 100% grid-shift should be considered

# 3. Conclusions

In this contribution, SBFD RF requirements are discussed with following observations and proposals.

Proposal 1: If co-location ACS is not defined, the note in the spec that the ACS requirement is not applied to the co-location scenario is needed.

Proposal 2: Only collect power level of interference from adjacent channel to derive in-band blocking levels.

Proposal 3: the reference point for the power level should be before RX beamforming

Proposal 4: 100% grid-shift should be considered

# 4. Reference

[1] TR 38.858,

[2] R4-2409958, Way Forward for [110bis][313] NR_duplex_evo, Samsung