3GPP TSG-RAN WG4 Meeting #111											R4-2411736

Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:	8.19.1

Source: 	CMCC

Title: 	(NR_duplex_evo-Core) Discussion on SBFD general part

Document for:	Discussion

# 1. Introduction

In TR 38.858, the impacts on SBFD requirements have been analyzed. RAN4 has identified which legacy requirements are still applicable, which legacy requirements are not applicable and which new requirements are needed.

In last meeting, WF on SBFD BS requirement is approved with several agreements. [2]

In this contribution, we continue the general part discussion based on the summary approved in last meeting.

- Discussion

  - BS RF Specification structure for SBFD requirements

In last meeting, the candidate options are listed as below.

Options of how to introduce BS RF new requirements for SBFD-capable BS: - Option 1: Creating new sub-clauses in TS 38.104 (similar as UE feature in TS38.101) - Option 2: Embedding the corresponding new RF requirements for SBFD in the corresponding section of TS 38.104 (similar as NB-IoT RF requirement in TS36.104)

To be honest, it’s a little early to discuss the RF spec structure for SBFD since we have no idea of how to define all SBFD RF requirements. It’s better to discuss this issue after we finish all RF requirements and before we start to write the spec. But since companies has provided this issue, we list our views.

Different from other BS spec, there are candidate new requirements for SBFD, e.g. ASCS, transient period. For such new RF requirements, it’s better to create new sub-clauses in TS 38.104 since such requirements only apply for SBFD gNB, which is much similar as UE feature in TS 38.101. For the legacy RF requirements, some of them are still applicable for SBFD with the same limit but others may need to be updated with new limit, e.g. Tx inter-modulation. for spec consistency, it’s also capture all legacy RF requirements applicable for SBFD into the new sub-clause. For the RF requirements that are the same for legacy gNB and SBFD, it’s better just state that the RF requirements in sub-clause still applies instead of simple copy-paste.

Proposal 1: for RF spec structure of SBFD requirements, it’s better to create new sub-clauses in TS 38.104 which capture all new and existing RF requirements that applies for SBFD.

  - Operating bands for SBFD

In last meeting, the candidate options are listed as below.

Issue 1-1-1: SBFD as band specific or general feature to all TDD bands - Agreement: - SBFD is a feature with requirements of which can potentially be applied to all TDD band, under the following considerations: - Option 1: Declaration based method - The band supported for SBFD shall be manufacturer declaration based - The channel bandwidth supported for SBFD shall be manufacturer declaration based - The supported channel bandwidth can be impacted by the subband/guard band size discussion - Option 2: General principle but with consideration(s) for a specific band - General principle: The channel bandwidth shall be larger than X MHz - FFS the value of X - X can be different for high and middle TDD bands - For a specific band, the following further restriction(s) can be considered: - Certain restriction can be provided by operator(s) for certain band - Other restrictions are not precluded.

Different from UE spec, in BS spec there are no band specific RF requirements for most bands except for the system parameters. The exception is for the unlicensed band and band n104. Considering the deployment of SBFD, it is better to define the general principle for SBFD. We suggest that the channel bandwidth for middle TDD bands should be equal or larger than 30MHz, and the channel bandwidth for high TDD bands should be equal to or larger than 60MHz.

Proposal 2: Define the general principle for SBFD:

- Channel bandwidth for middle TDD bands should be equal to or larger than 30MHz.

- Channel bandwidth for high TDD bands should be equal to or larger than 60MHz.

  - Guard band and subband configurations

In last meeting, the candidate options are listed as below.

Issue 1-2-2: How to handle guard band and subband configurations in specification - Agreement: - Add clarification that only DUD and DU patterns are specified for the sub-band configuration. - FFS which channel bandwidth(s) or all channel bandwidths shall be defined for RF requirements - For a certain channel bandwidth which RAN4 agree to introduce RF requirements: - FFS RAN4 only define the UL/DL subbands configuration(s) for RF requirements - FFS which UL/DL subbands configuration(s) will be defined in RAN4 - FFS Guard band size is declaration based and can be different for different BS classes - FFS the limitation on the maximum guard band - FFS possible range for UL/DL subband sizes - From RAN4 perspective, FFS restriction or no restriction to RAN1 definition for UL/DL subband sizes within the transmission configuration for this channel bandwidth, except: - 1RB granularity (already introduced in RAN1)

During the study phase and according to the WID, the common understanding is that it’s within RAN4 scope to study/specify the size of the sub-band/guardband. Considering some implementations use RF filters to avoid blocking and self-interference, it’s better to specify a limited number of subband size for each channel bandwidth with corresponding guardband. As for the channel bandwidth definition, wider channel bandwidth would fully utilize SBFD performance gain and it’s more challenging to use narrower channel bandwidth to support SBFD. But considering the deployment, we want to be able to deploy SBFDs in some middle TDD bands as well. From our point of view, 100MHz, 60MHz, and 30MHz are preferred to support SBFD.

Proposal 3: 100MHz, 60MHz and 30MHz are suggested as typical channel bandwidth to support SBFD.

As we know, one of the advantages of SBFD is its flexible frame structure. To maintain this advantage, we believe that no restrictions should be set on subband size. Therefore, the value of DL and UL subband size should not be fixed in the spec, and one or more typical sizes for different channel bandwidths should be specified. Since the subband size is not fixed in the spec, the guard band also should not be fixed. One typical value or the maximum value of the guard band size should be specified in the spec, and some guard RB could be used as DL or UL in actual operation.

Proposal 4: To keep the flexibility of SBFD, no restriction should be set for subband size, and some typical subband size could be specified in the spec.

Proposal 5: One typical value or the maximum value of the guard band size should be specified in the spec.

# 3. Conclusions

In this contribution, SBFD RF requirements are discussed with following observations and proposals.

Proposal 1: for RF spec structure of SBFD requirements, it’s better to create new sub-clauses in TS 38.104 which capture all new and existing RF requirements that applies for SBFD.

Proposal 2: Define the general principle for SBFD:

- Channel bandwidth for middle TDD bands should be equal to or larger than 30MHz.

- Channel bandwidth for high TDD bands should be equal to or larger than 60MHz.

Proposal 3: 100MHz, 60MHz and 30MHz are suggested as typical channel bandwidth to support SBFD.

Proposal 4: To keep the flexibility of SBFD, no restriction should be set for subband size, and some typical subband size could be specified in the spec.

Proposal 5: One typical value or the maximum value of the guard band size should be specified in the spec.

# 4. Reference

[1] TR 38.858,

[2] R4-2409958, Way Forward for [110bis][313] NR_duplex_evo, Samsung