3GPP TSG-RAN WG4 Meeting #112											R4-2411737

Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:	8.19.2.2

Source: 	CMCC

Title: 	(NR_duplex_evo-Core) Discussion on existing Tx requirements for SBFD

Document for:	Discussion

# 1. Introduction

In TR 38.858, the impacts on SBFD requirements have been analyzed. RAN4 has identified which legacy requirements are still applicable, which legacy requirements are not applicable and which new requirements are needed.

In last meeting, WF on SBFD BS requirement is approved with several agreements. [1]

In this contribution, we continue the RF requirements discussion based on the output of study item phase.

- Discussion

  - WA gNB requirements

In last meeting, the issue of whether to define WA gNB requirements have been discussed. The common understanding is that WA gNB requirement should be defined in this work item. But further discuss the priority of WA gNB, i.e. whether WA is low priority or not. From our point of view, the support of gNB class is declaration basis which means this WA gNB is not mandatory to support for all vendors. So at work phase, we can focus on RF requirement discussion and all gNB classes with the same high priority

Proposal 1: it’s suggested to define WA, MR, LA gNB requirement for SBFD with same priority.

2.3 ACLR requirements

Following capture the agreements from TR 38.858 for ACLR requirements:

For ACLR requirement, it shall be defined outside of the whole carrier instead of sub-band for SBFD DL symbols/slots and ACLR requirement is still defined as the ratio of sum of TX power within the whole carrier to the adjacent carrier.

And during last meeting, we have reached the following agreement:

Issue 2-3-1: The necessity of Co-location ACLR requirement - Agreement: - RAN4 further study the ACLR requirement by applying the existing ACLR requirement for SBFD-capable BS in SBFD symbols/slots - FFS the applicable deployment scenario where different ACLR requirement could be useful, whether it justify the different ACLR requirement.

For the SBFD network, adjacent-channel co-existence simulation only focuses on not less than 10% grid shift values, and 0% grid shift is avoided. But 0% grid shift is the worst case. So it seems co-location ACLR requirements or equivalent requirements are needed. We list two options:

Option 1: Not define co-location ACLR requirement. Add the note in the spec that the ACLR requirement is not applied to the co-location scenario.

Option 2: Define the co-location ACLR requirement. If co-location ACLR is defined, it is necessary to ensure that the receiver of the BS is not blocked

Proposal 2: If co-location ACLR is defined, then it is necessary to ensure that the receiver of the BS is not blocked, otherwise the note in the spec that the ACLR requirement is not applied to the co-location scenario is needed.

# 3. Conclusions

In this contribution, SBFD RF requirements are discussed with following observations and proposals.

Proposal 1: it’s suggested to define WA, MR, LA gNB requirement for SBFD with same priority.

Proposal 2: If co-location ACLR is defined, then it is necessary to ensure that the receiver of the BS is not blocked, otherwise the note in the spec that the ACLR requirement is not applied to the co-location scenario is needed

# 4. Reference

[1] R4-2409958, Way Forward for [110bis][313] NR_duplex_evo