3GPP TSG-RAN WG4 Meeting #112									     	R4-2411738

Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:	7.3.3

Source: 	CMCC

Title: 	Discussion on MSD requirements with intra-band contiguous UL CA

Document for:	Discussion

# 1. Introduction

At the last meeting, the issue about test points for inter-band BC with intra-band UL CA in one band. The companies did not reach an agreement on this issue, and left it to the Rel-19. As operator, we think this issue is very important, and it also relevant to the MSD values for many CA band combinations. Therefore, in this meeting we will continue to express our views on this issue

- Discussion

When simultaneous Rx/Tx is supported for the inter-band UL CA with intra-band contiguous UL CA band combinations, the intermodulation products will cause more REFSENS degradation. Consider this situation, RAN4 introduced MSD requirements for inter-band UL CA with intra-band contiguous UL CA in one band. There are three types of such MSD requirements, such as inter-band CA with cross-band DL interference and inter-band CA with triple beat issue.

For R18 TDD MSD test configuration guidelines, small numbers of RBs or even only 1 RB in each UL carrier are specified, and we think it is an extreme scenario that doesn't occur in operators’ networks. In our opinion, we introduced UL intra-band CA to enable an increase in UL throughput by introducing a larger channel bandwidth. Usually, a network will not schedule another carrier and configure UL intra-band CA if one carrier is not fully utilized. The test case that 1 RB is specified for each carrier of the intra-band CA will not appear in realistic network resource allocations, because one single carrier could achieve the throughput.

Observation 1: The test case that 1 RB is specified for each carrier of the intra-band CA will not appear in realistic network resource allocations, because one single carrier could achieve the throughput.

During the last meeting discussion, some companies believed that the current testing configuration was reasonable. Maybe we should to think about the motivation why we need to specify MSD and why we need to introduce CA. As we mentioned above, there are many deviations between current testing configurations and actual usage situations, so it cannot represent typical scenario.

Observation 2: Typical scenario should be considered when we set MSD test configuration in Rel-19.

In our opinion, we think the extreme scenario in the network should not be considered and cross-band MSD requirements resulting from intra-band contiguous UL CA configured with other test configurations (e.g. fully allocated maximum aggregated BW) should be introduced.

Proposal 1: Do not consider only the MSD requirements resulting from intra-band contiguous UL CA configured with 1RB+1RB allocations.

Since the issue of testing configuration had not being resolved in R19, MSD of some band combinations, such as CA_n40A-n41C and CA_n41-n79 had been postponed to R19. MSD values of these band combinations should not only consider the 1RB+1RB test case.

Proposal 2: MSD values of these band combinations should not only consider the 1RB+1RB test case..

# 3. Conclusions

In this contribution, question have been discussed with following observations and proposals.

Observation 1: The test case that 1 RB is specified for each carrier of the intra-band CA will not appear in realistic network resource allocations, because one single carrier could achieve the throughput.

Observation 2: Typical scenario should be considered when we set MSD test configuration in Rel-19.

Proposal 1: Do not consider only the MSD requirements resulting from intra-band contiguous UL CA configured with 1RB+1RB allocations.

Proposal 2: MSD values of these band combinations should not only consider the 1RB+1RB test case..

# 4. Reference