3GPP TSG-RAN WG4 Meeting #112				                                                                                        R4-2411742

Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda Item:	7.20.2

Source: 	CMCC

Title: 			  UE RF requirements for n28 PC2

Document for:	  Discussion

- Introduction

In last RAN plenary, a WI on Introduction of Power Class 2 and UE 40MHz Channel Bandwidth in NR band n28 had been approved in [1], the corresponding objectives are as follows:

The objectives of this WI are to introduce requirements for UE Power Class 2 and 40MHz Channel Bandwidth for NR band n28 as optional features, including the following aspects: - Introduce requirements for PC2 and those associated with following bullets - Both 1Tx and 2Tx PC2 are supported - UL-MIMO is supported. - Specify A-MPR requirements for PC2 - Introduce 40MHz Channel bandwidth located in UL: 703 - 743MHz and DL: 758 - 798MHz for band n28 supporting PC3 and PC2. - Allow flexible channel allocations within the frequency range (UL: 703 - 743MHz and DL: 758 - 798MHz) for BW<40MHz. - Release independence of UE 40MHz Channel bandwidth to Rel-15 is anticipated. - No changes to the existing requirements (38.101-1 v18.5.0) (including A-MPR values and RB allocation regions) for power class 3 with existing channel bandwidths up to and including 30MHz to avoid impacts on existing power class 3 networks. In the event of new regulatory requirements, this assumption can be revisited.

In this contribution, we discuss the requirements for n28 PC2.

- Discussion

In Rel-18, the NS_17 A-MPR requirements for n28 PC2 had been discussed and the agreements are:

<Agreement>:  For a UE implementing the dual-duplexer on band n28, no A-MPR for NS_17 is needed for PC2 as well as PC3, for BW of 5MHz and 10MHz.

Due to the different implementation of single duplexer and dual duplexer, different A-MPR requirements are required.  In Rel-18, it had already been agreed that for a UE implementing the dual-duplexer on band n28, no A-MPR for NS_17 is needed for PC2 as well as PC3, for BW of 5MHz and 10MHz. During the Rel-19 WI discussion, there is desire from companies to remove the UE architecture description and focus on the requirements discussion. Hence, RAN4 first needs to discuss the UE architecture assumption and whether one set or two sets requirements should be defined in Rel-19.

Proposal 1: RAN4 first to discuss the UE architecture assumption and whether one set or two sets requirements should be defined in Rel-19.

Since it had already been captured in the WID that no changes to existing requirements (including A-MPR values and RB allocation regions) for power class 3, there is no need to discuss the PC3 and/or LTE requirements for n28.

Proposal 2: No need to discuss the PC3 and/or LTE requirements for n28.

- Reference

- RP-241660, New WID: Introduction of Power Class 2 and UE 40MHz Channel Bandwidth in NR band n28, CMCC CBN.

[2] R4-2410558, WF on A-MPR for n28, Apple (approved).

[3] R4-2406708, WF on band n28 full band duplexer, Apple.

[4] R4-2403629, WF on HPUE for FDD bands, China Unicom, Skyworks, RAN4#110 (approved).