3GPP TSG-RAN WG4 Meeting #112				                                                   R4-2411743

Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda Item:	7.20.3

Source: 	CMCC

Title: 			  UE RF requirements for introducing 40MHz

Document for:	  Discussion

- Introduction

In last RAN plenary, a WI on Introduction of Power Class 2 and UE 40MHz Channel Bandwidth in NR band n28 had been approved in [1], the corresponding objectives are as follows:

The objectives of this WI are to introduce requirements for UE Power Class 2 and 40MHz Channel Bandwidth for NR band n28 as optional features, including the following aspects: - Introduce requirements for PC2 and those associated with following bullets - Both 1Tx and 2Tx PC2 are supported - UL-MIMO is supported. - Specify A-MPR requirements for PC2 - Introduce 40MHz Channel bandwidth located in UL: 703 - 743MHz and DL: 758 - 798MHz for band n28 supporting PC3 and PC2. - Allow flexible channel allocations within the frequency range (UL: 703 - 743MHz and DL: 758 - 798MHz) for BW<40MHz. - Release independence of UE 40MHz Channel bandwidth to Rel-15 is anticipated. - No changes to the existing requirements (38.101-1 v18.5.0) (including A-MPR values and RB allocation regions) for power class 3 with existing channel bandwidths up to and including 30MHz to avoid impacts on existing power class 3 networks. In the event of new regulatory requirements, this assumption can be revisited.

In this contribution, we discuss the introduction of 40MHz for n28.

- Discussion

According to the WID, 40MHz is introduced for channel bandwidth located in UL: 703 - 743MHz and DL: 758 - 798MHz for band n28 supporting PC3 and PC2.

For the existing channel bandwidth<40MHz, flexible channel allocation should be allowed. Hence for the UE supporting 40MHz, a new note should be added in Table 5.3.5-1:

NOTE 7:	For UEs supporting 40MHz, for the 20 MHz bandwidth, the minimum requirements are specified for NR UL carrier frequencies confined to either 713-733 MHz or 728-738 MHz. For the 25 MHz bandwidth, the minimum requirements are specified for NR UL carrier frequencies confined to either 715.5-730.5 MHz or 730.5-735.5 MHz. For the 30MHz bandwidth, the minimum requirements are specified for NR UL carrier frequencies confined to 718-728MHz or 733MHz.  For the 40MHz bandwidth, the minimum requirements are specified for NR UL transmission bandwidth configuration confined to 703-743MHz.

Proposal 1: it is proposed to add a new NOTE for n28 40MHz as follows:

NOTE X:	For UEs supporting 40MHz, for the 20 MHz bandwidth, the minimum requirements are specified for NR UL carrier frequencies confined to either 713-733 MHz or 728-738 MHz. For the 25 MHz bandwidth, the minimum requirements are specified for NR UL carrier frequencies confined to either 715.5-730.5 MHz or 730.5-735.5 MHz. For the 30MHz bandwidth, the minimum requirements are specified for NR UL carrier frequencies confined to 718-728MHz or 733MHz.  For the 40MHz bandwidth, the minimum requirements are specified for NR UL transmission bandwidth configuration confined to 703-743MHz.

In the WID, the release independence of UE 40MHz channel bandwidth to Rel-15 is anticipated. The reason is that companies would like to check whether new signaling is required or not to support 40MHz. According to TS38.331, the Rel-15 supportedBandwidth IE already supports indicating 40MHz, hence it can be confirmed that 40MHz is release independence from Rel-15.

Proposal 2: it is proposed to confirm that UE 40MHz channel bandwidth is release independent from Rel-15.

- Conclusion

In this contribution, we discuss the support of 40MHz in n28. The proposals are:

Proposal 1: it is proposed to add a new NOTE for n28 40MHz as follows:

NOTE X:	For UEs supporting 40MHz, for the 20 MHz bandwidth, the minimum requirements are specified for NR UL carrier frequencies confined to either 713-733 MHz or 728-738 MHz. For the 25 MHz bandwidth, the minimum requirements are specified for NR UL carrier frequencies confined to either 715.5-730.5 MHz or 730.5-735.5 MHz. For the 30MHz bandwidth, the minimum requirements are specified for NR UL carrier frequencies confined to 718-728MHz or 733MHz.  For the 40MHz bandwidth, the minimum requirements are specified for NR UL transmission bandwidth configuration confined to 703-743MHz.

Proposal 2: it is proposed to confirm that UE 40MHz channel bandwidth is release independent from Rel-15.

- Reference

- RP-241660, New WID: Introduction of Power Class 2 and UE 40MHz Channel Bandwidth in NR band n28, CMCC CBN.

[2] R4-2410558, WF on A-MPR for n28, Apple (approved).

[3] R4-2406708, WF on band n28 full band duplexer, Huawei.

[4] R4-2403629, WF on HPUE for FDD bands, China Unicom, Skyworks, RAN4#110