3GPP TSG-RAN WG4 Meeting #112				                                                   R4-2411744

Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda Item:	8.7.2

Source: 	CMCC

Title: 			  PC2 requirements for TDD RedCap and eRedCap

Document for:	  Discussion

- Introduction

In last RAN plenary, a WI on NR power class 2 RedCap (Reduced Capability) UE in FR1 had been approved in [1], the corresponding objectives are as follows:

The objectives of the core part are as follows: - Apply the PC2 RF requirements for non-RedCap UE to RedCap UE, based on 1Tx architecture with 26 dBm PA. - The PC2 RF requirements include: UE maximum output power, Tx power tolerance, MPR, A-MPR and RSD requirements if needed. - For TDD bands, PC2 RF requirements for non-RedCap UE can be re-used for PC2 RedCap UE in a band agnostic way. - Note: Not differentiate the applicable form factors for TDD bands in the specification. - Check at RAN #106 on whether to define PC2 RF requirements for FDD bands and the applicable form factors for FDD bands (if defined). Note 1: Only the operating bands for which PC2 on non-RedCap UE has been introduced can be considered for RedCap UE. - Note 2: PC2 for Rel-17 Redcap and Rel-18 eRedcap are considered for this WI.

In this contribution, we discuss the PC2 requirements for TDD RedCap and eRedCap UE.

- Discussion

In existing spec, non-RedCap PC3 requirements including UE maximum output power, Tx power tolerance are applied to RedCap UE as follows:

In the WID, it is already confirmed that:

For TDD bands, PC2 RF requirements for non-RedCap UE can be re-used for PC2 RedCap UE in a band agnostic way.

o	Note: Not differentiate the applicable form factors for TDD bands in the specification.

Until RAN#106, the WI will focus on TDD PC2 RedCap UE. All the PC2 requirements for non-RedCap UE can also apply to RedCap. The agreed assumption is that RedCap is based on 1Tx architecture, same as legacy RedCap requirements. In our view, no additional clarification is needed on 1Tx part.

Even though some example bands are listed in the WID. However, for all the TDD bands, PC2 requirements for non-RedCap UE can be reused for PC2 RedCap. It is not necessary to specify any band table, only one sentence is enough to clarify that non-RedCap PC2 requirements can be applied to RedCap UE.

## 6.2I Transmitter power for RedCap ### 6.2I.1 Maximum output power for RedCap For Redcap and eRedCap UE, the requirements for power class 3 specified in clause 6.2.1 apply. For RedCap and eRedCap UE, the requirements for power class 2 specified in clause 6.2.1 [for TDD bands] apply.

- Conclusion

In order to support PC2 for RedCap and eRedCap on TDD bands, following changes can be applied.

## 6.2I Transmitter power for RedCap ### 6.2I.1 Maximum output power for RedCap For Redcap and eRedCap UE, the requirements for power class 3 specified in clause 6.2.1 apply. For RedCap and eRedCap UE, the requirements for power class 2 specified in clause 6.2.1 [for TDD bands] apply.

- Reference

- RP-241665, New WID on NR power class 2 RedCap (Reduced Capability) UE in FR1.