3GPP TSG-RAN WG4 Meeting # 112		R4-2411754

# Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda Item:	5.8.4

Source: 	CMCC

Title: 			  Discussion on k1 value and range correction for ATG

Document for:	Discussion

- Introduction

In last meeting, ATG k1 issue was raised by HW that the size and the range of k1 for ATG 30D4S6U TDD pattern are beyond the IE DL-DataToUL-ACK-v1700. [1]

In this contribution, we provide our proposal about how to address the issue.

- Background

The current mapping relationship between PDSCH and HARQ-ACK for 30D4S6U in TS38.101-4:

| The number of slots between PDSCH and corresponding HARQ-ACK information (Note 3) |  | 34-i if mod(i,40) = 0, 1, 2, 3, 4 35-i if mod(i,40) = 5, 6, 7, 8, 9 36-i if mod(i,40) = 10, 11, 12, 13, 14 37-i if mod(i,40) = 15, 16, 17, 18, 19 38-i if mod(i,40) = 20, 21, 22, 23, 24 39-i if mod(i,40) = 25, 26, 27, 28, 29 |
| --- | --- | --- |

Such mapping method results in k1 value having a range of (34, 33, 32, ....12 , 11, 10) and a size of 25.

Observation 1: The current mapping relationship between PDSCH and HARQ-ACK for 30D4S6U results in the k1 value having a range of (34, 33, 32, ....12 , 11, 10) and a size of 25.

In last meeting, RAN2 introduced a new k1 signalling for ATG with size 8 and range 0 to 31, i.e. DL-DataToUL-ACK-r18

DL-DataToUL-ACK-r18 ::=                    SEQUENCE (SIZE (1..8)) OF INTEGER (0..31)

dl-DataToUL-ACK, dl-DataToUL-ACK-DCI-1-2

List of timing for given PDSCH to the DL ACK (see TS 38.213 [13], clause 9.1.2). The field dl-DataToUL-ACK applies to DCI format 1_1 and the field dl-DataToUL-ACK-DCI-1-2 applies to DCI format 1_2 (see TS 38.212 [17], clause 7.3.1 and TS 38.213 [13], clause 9.2.3). The dl-DataToUL-ACK-v1700 is applicable for NTN and dl-DataToUL-ACK-r17 is applicable for up to 71 GHz. The dl-DataToUL-ACK-r18 is applicable for ATG. If dl-DataToUL-ACK-r16 or dl-DataToUL-ACK-r17 or dl-DataToUL-ACK-v1700 or dl-DataToUL-ACK-r18 is signalled, UE shall ignore the dl-DataToUL-ACK (without suffix). The value -1 corresponds to "inapplicable value" for the case where the A/N feedback timing is not explicitly included at the time of scheduling PDSCH. The fields dl-DataToUL-ACK-r17 and dl-DataToUL-ACK-DCI-1-2-r17 are only applicable for SCS of 480 kHz or 960 kHz. The field dl-DataToUL-ACK-r18 applies to DCI format 1_1 and the field dl-DataToUL-ACK-DCI-1-2-r18 applies to DCI format 1_2 (see TS 38.212 [17], clause 7.3.1 and TS 38.213 [13], clause 9.2.3).

Observation 2: RAN2 introduced a new k1 signalling for ATG with size 8 and range 0 to 31, i.e. DL-DataToUL-ACK-r18

However, although RAN2 introduced new IE DL-DataToUL-ACK-r18 with size 8 and range 0 to 31, the issue still exists.

Observation 3: The size and the range of current k1 for ATG 30D4S6U TDD pattern are beyond the IE DL-DataToUL-ACK-r18.

- Discussion

To control the k1 size and range, the timing relationship between PDSCH and HARQ-ACK should be remapped, we propose following solution:

| The number of slots between PDSCH and corresponding HARQ-ACK information (Note 3) |  | 34 if mod(i,40) = 0, 1, 2, 3, 4, 5 28 if mod(i,40) = 6, 7, 8, 9, 10, 11 22 if mod(i,40) = 12, 13, 14, 15, 16, 17 16 if mod(i,40) = 18, 19, 20, 21, 22, 23 10 if mod(i,40) = 24, 25, 26, 27, 28, 29 |
| --- | --- | --- |

As we known, the koffset was introduced in ATG system, which is cellspecifickoffset in SIB22.

As specified in TS 38.214, Clause 5.3, the koffset will be used in the PDSCH processing procedure time

Specifically, in TS 38.213, Clause 9, it defines how to use the koffset, the slots between PDSCH and corresponding HARQ-ACK information is k1+2μ-μ_K_offset *koffset

By configure koffset = 2, the k1 value in above table will be:

| The number of slots between PDSCH and corresponding HARQ-ACK information (Note 3) |  | 34 if mod(i,40) = 0, 1, 2, 3, 4, 5; k1=30, koffset =2 28 if mod(i,40) = 6, 7, 8, 9, 10, 11; k1=24, koffset=2 22 if mod(i,40) = 12, 13, 14, 15, 16, 17; k1=18, koffset =2 16 if mod(i,40) = 18, 19, 20, 21, 22, 23; k1=12, koffset =2 10 if mod(i,40) = 24, 25, 26, 27, 28, 29; k1=6, koffset =2 |
| --- | --- | --- |

The k1 size will be 5, and the range is within (0...31). The issue can be resolved.

Proposal 1: Revise the timing relationship between PDSCH and HARQ-ACK to:

| The number of slots between PDSCH and corresponding HARQ-ACK information (Note 3) |  | 34 if mod(i,40) = 0, 1, 2, 3, 4, 5 28 if mod(i,40) = 6, 7, 8, 9, 10, 11 22 if mod(i,40) = 12, 13, 14, 15, 16, 17 16 if mod(i,40) = 18, 19, 20, 21, 22, 23 10 if mod(i,40) = 24, 25, 26, 27, 28, 29 |
| --- | --- | --- |

- Conclusion

Based on the discussion above, the following observations and proposals are concluded.

Observation 1: The current mapping relationship between PDSCH and HARQ-ACK for 30D4S6U results in the k1 value having a range of (34, 33, 32, ....12 , 11, 10) and a size of 25.

Observation 2: RAN2 introduced a new k1 signalling for ATG with size 8 and range 0 to 31, i.e. DL-DataToUL-ACK-r18

Observation 3: The size and the range of current k1 for ATG 30D4S6U TDD pattern are beyond the IE DL-DataToUL-ACK-r18.

Proposal 1: Revise the timing relationship between PDSCH and HARQ-ACK to:

| The number of slots between PDSCH and corresponding HARQ-ACK information (Note 3) |  | 34 if mod(i,40) = 0, 1, 2, 3, 4, 5 28 if mod(i,40) = 6, 7, 8, 9, 10, 11 22 if mod(i,40) = 12, 13, 14, 15, 16, 17 16 if mod(i,40) = 18, 19, 20, 21, 22, 23 10 if mod(i,40) = 24, 25, 26, 27, 28, 29 |
| --- | --- | --- |

- Reference

- R4-2408953, [NR_ATG-Perf] Discussion on K1 value for new TDD pattern for ATG PDSCH demodulation performance requirements, Huawei, HiSilicon