3GPP TSG-RAN WG4 Meeting # 112      	R4-2411758

# Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda Item:	8.10.4

Source: 	CMCC

Title: 			 (NR_ATG_enh-Core) Overview of RRM core requirements for R19 ATG

Document for:	Discussion

- Introductions

In RAN#103-e meeting, the work item on Enhancement for Air-to-ground network for NR has been approved. According to the latest revised WID [1], the RRM core requirements for intra-band co-located and inter-band co-located DL CA should be specified. We copy the objective as below for information:

The core part of the work item includes: - Specify the RF and RRM core requirements for intra-band co-located and inter-band co-located DL CA [RAN4]: - FR1 intra-band contiguous CA - Example band combination: CA_n79C - FR1+FR1 inter-band CA - Example band combination: CA_n3A-n39A - Specify the RF requirements for support of UL MIMO with 2TX for single CC for UE. [RAN4] - Specify signaling and UE capabilities if necessary [RAN2].

In this contribution, we discuss the potential RRM impact considering ATG specific CA scenario, and then identify the type of RRM requirements that RAN4 need to further develop and specify.

- Discussion

  - General

Scenario

Based on the approved WID, R19 ATG enhancement work item focus on FR1 DL co-located CA scenario, including intra-band contiguous CA and inter-band CA. For other ATG scenario characteristics like UE speed, ISD and so on, the R18 working assumption will be reused.

Proposal 1: Focus on FR1 DL co-located CA scenario, including intra-band contiguous CA and inter-band CA.

Proposal 2: For other ATG scenario characteristics like UE speed, ISD and so on, the R18 working assumption will be reused.

ATG UE antenna type

RAN4#111 meeting has achieved following agreement about antenna type [2]:

Issue 3-1: clarify the antenna type for each band for inter-band CA

Agreement:

- No limitation on antenna types for ATG CA

- FFS on whether to assume omni-antenna type can be assumed for both band n3 and n39 in DL CA_n3-n39

- FFS on whether new capability is needed for ATG CA

For intra-band contiguous CA, same antenna type should be applied on each carrier, including one or more omni-directional antenna(s) and antenna array.

For inter-band CA, as agreed in RF session in RAN4#111 meeting, there is no limitation on antenna types. In other words, either different or same antenna type can be applied on different carriers.

Besides, if antenna array is used, there may exists two cases, case 1 is that UE only have one panel and can only form one Rx beam, case 2 is that UE have two antenna panels and can form two simultaneous Rx beams.

Proposal 3: For intra-band contiguous CA, same antenna type should be applied on each carrier, including one or more omni-directional antenna(s) and antenna array.

Proposal 4: For inter-band CA, either different or same antenna type can be applied on different carriers.

Proposal 5: If only antenna array is used, then may exists two cases:

- Case 1: UE only have one antenna panel and form one Rx beam

- Case 2: UE have two antenna panels and can form two simultaneous Rx beams

  - RRC IDLE/INACTIVE mobility requirements

Side conditions for cell re-selection

RF agreed to define relaxed receiver sensitivity for inter-band CA, the value is FFS.

Issue 3-4-1: ATG Rx requirements applicability

Agreement:

- Following requirements needs further analysis:

  - delta RIB,

  - MSD,

  - Diversity requirements

    - FFS whether current ATG single carrier requirement is enough

Since RF has already agreed to define relaxed receiver sensitivity for ATG inter-band CA [2], the side conditions exceptions for cell re-selection shall be defined, the legacy methodology exception applicability defined in Clause 4.2 and Clause B.3.2.1 can be reused with update of the reference clause of ΔRIB,c. We copy it for information:

In the requirements of clause 4.2, the exceptions for side conditions apply as follows: - for the UE capable of CA, the applicable exceptions for side conditions are specified in Annex B, clause B.3.2.1 for UE supporting CA in FR1;

B.3.2.1 Receiver sensitivity relaxation for UE supporting CA in FR1 For a UE configured with inter-band carrier aggregation with active uplink in NR band, if there is a relaxation of receiver sensitivity ΔRIB,c>0 dB as defined in clause 7.3A.3 of TS 38.101-1 [18], the relevant side conditions specifying received power levels (SSB_RP and Io) shall be increased by the amount Δ=ΔRIB,c defined for the corresponding downlink NR bands.

Proposal 6: For cell re-selection, define the exceptions of side conditions for UE supporting CA in FR1. The legacy methodology exception applicability defined in Clause 4.2 and Clause B.3.2.1 can be reused with update of the reference clause of ΔRIB,c.

EMR

Firstly, we think RAN4 should discuss whether EMR feature need to be introduced in ATG.

The feature early measurement report was introduced to facilitate the SCell addition, and help network to configure a proper SCell by reporting the configured SCC measurement result in RRC_IDLE/INACTIVE mode.

However, in ATG co-located scenario, network could derive coarse SCell quality by PCell quality and known frequency gap information, we fail to see strong demand and significant gain to introduce this feature.

Observation 1: Fail to see strong demand and significant gain to introduce EMR in ATG co-located deployment.

Proposal 7: No need to introduce EMR in ATG CA if no strong demand and significant gain observed.

  - Timing requirements

MRTD

With DL CA, the MRTD requirement at UE side should be defined. For ATG, we can first focus on co-located intra-band contiguous CA and inter-band CA case in WID.

For intra-band contiguous CA, the TAE is 260ns, the propagation delay difference is 0 due to co-located. The MRTD should be 260ns if defined. While in legacy, there is no MRTD intra-band contiguous CA requirement, because 260ns can be handled by every UE since it is smaller than 2/CP. Follow similar logic, the MRTD requirement for ATG intra-band contiguous CA is not needed.

For inter-band CA, the legacy FR1 MRTD requirement is 33µs which including 3µs TAE and propagation delay difference. Although propagation delay difference between PCell and SCell is 0 in ATG co-located deployment, we think 33µs MRTD can be reused for ATG UE which is more powerful than legacy hand-held UE, such approach is also more friendly to backward compatibility.

Proposal 8: No need to introduce MRTD requirement for ATG intra-band contiguous CA.

Proposal 9: Define MRTD requirement for ATG inter-band CA, legacy 33µs requirement can be reused.

  - Signalling characteristics requirements

SCell Activation and Deactivation Delay

In our perspective, the following features can be introduced for ATG, the current FR1 requirement can be the starting point, and we can further study whether further improvement is needed or not due to co-located deployment. To be mentioned, the requirements related to multiple downlink SCells are deprioritized since example bands are 2CCs in WID.

- SCell Activation Delay Requirement for Deactivated SCell

- SCell Deactivation Delay Requirement for Activated SCell

- Direct SCell Activation at SCell addition

- Direct SCell Activation at Handover

- Direct SCell Activation at RRCResume

- Fast SCell Activation Delay Requirement for Deactivated SCell

- SCell Activation Delay Requirement for Deactivated SCell with the L3 reporting during activation.

Proposal 10: For SCell activation and deactivation, following features can be introduced for ATG, reuse the current FR1 requirements as the starting point, and further study whether further improvement is needed or not due to co-located deployment.

- SCell Activation Delay Requirement for Deactivated SCell

- SCell Deactivation Delay Requirement for Activated SCell

- Direct SCell Activation at SCell addition

- Direct SCell Activation at Handover

- Direct SCell Activation at RRCResume

- Fast SCell Activation Delay Requirement for Deactivated SCell

- SCell Activation Delay Requirement for Deactivated SCell with the L3 reporting during activation.

Active BWP switch delay on multiple CCs

For DCI based BWP switch delay and RRC based BWP switch delay on multiple CCs , due to NR-DC is not in the scope, only simultaneous BWP switch delay scenario need to be considered, and the legacy requirement can be reused.

For Timer based BWP switch delay on multiple CCs, both simultaneous and non-simultaneous BWP switch delay scenario should be considered, and the legacy requirement can be reused.

Proposal 11: For Active BWP switch delay on multiple CCs, introduce following requirements, which can reuse the legacy:

- Simultaneous DCI based BWP switch delay on multiple CCs

- Simultaneous RRC based BWP switch delay on multiple CCs

- Simultaneous and non-simultaneous Timer based BWP switch delay on multiple CCs

Beam failure recovery in SCell

The delay requirement for beam failure recovery in SCell shall be considered, the legacy procedure and requirement can be reused.

Proposal 12: Introduce Beam failure recovery in SCell for ATG CA, the legacy procedure and requirement can be reused.

Pre-configured measurement gap activation/deactivation delay upon SCell activation/deactivation

In Rel-18 ATG single carrier scenario, the pre-configured measurement gap is supported. When CA is supported Rel-19 ATG, it is natural to introduce the SCell activation/deactivation as an event for triggering pre-configured measurement gap activation/deactivation, the legacy requirement can be reused.

Proposal 13: Introduce Pre-configured measurement gap activation/deactivation delay requirement upon SCell activation/deactivation, the legacy requirement can be reused.

Interruptions with Standalone NR Carrier Aggregation

For the following cases, the interruption will occur on PCell and activated SCell.

- SCell addition/release

- SCell activation/deactivation

- Direct SCell activation

- Fast SCell activation

- Measurements on deactivated SCC

- SCell dormancy

- Active BWP switching

- UE-specific CBW change

- When identifying CGI of an NR cell with autonomous gaps

- NR SRS antenna port switching

The interruption requirement shall be defined, the legacy requirement can be reused for ATG.

Proposal 14: Introduce the following interruption requirement, the legacy requirement can be reused.

- Interruptions at SCell addition/release

- Interruptions at SCell activation/deactivation

- Interruptions at direct SCell activation

- Interruptions at fast SCell activation

- Interruptions during measurements on deactivated SCC

- Interruptions due to SCell dormancy

- Interruptions due to Active BWP switching Requirement

- Interruptions due to UE-specific CBW change

- Interruptions when identifying CGI of an NR cell with autonomous gaps

- Interruptions at NR SRS antenna port switching

  - Measurement procedure requirement

CSSF for FR1 only CA

Due to both PCC and SCC need to be measured under CA configuration, the CSSFoutside_gap shall be updated to cover SCC measurement. The legacy scaling scheme can be reused, that PCC occupy one measurement searcher resource, SCC and other inter-frequency MO with no measurement gap share another measurement searcher resource.

Proposal 15: Reuse the legacy scheme when defining the CSSFoutside_gap for ATG FR1 only CA, that PCC occupy one measurement searcher resource, SCC and other inter-frequency MO with no measurement gap share another measurement searcher resource.

Measurement Gap

Due to only FR1 CA configured, the measurement gap applicability rule and scheduling applicability for SA single carrier shall be also applied for NR CA configuration.

Proposal 16: The measurement gap applicability rule and scheduling applicability for SA single carrier shall be also applied for NR CA configuration.

Besides, since R19 only investigate co-located CA, then only aligned frame boundaries scenario will be existed. The interruption time on an SCC is same as the single carrier case.

Proposal 17: When define the SCC interruption time, only consider aligned frame boundaries scenario, the interruption requirement in single carrier case can be reused for CA configuration.

Capabilities for Support of Event Triggering and Reporting Criteria

For UE capability of supporting NR reporting criteria of category intra-frequency, the SCell serving frequency shall be counted in the number of configured NR serving frequencies carrier frequency.

Proposal 18: For UE capability of supporting NR reporting criteria of category intra-frequency, the SCell serving frequency shall be counted in the number of configured NR serving frequencies carrier frequency.

Pre-configured measurement gap requirement

When define the requirements and the requirement applicability for network-controlled and autonomous activation/deactivation mechanism, it is natural to introduce the SCell related triggering conditions as an event for triggering pre-configured measurement gap, and the legacy procedure and requirement can be reused.

Proposal 19: Introduce the SCell related triggering conditions as an event for triggering pre-configured measurement gap, and the legacy procedure and requirement can be reused.

SCell activation triggered measurement reporting requirement

If SCell Activation for Deactivated SCell with the L3 reporting during activation is supported for ATG, then the measurement reporting requirement shall also be introduced, legacy requirement can be reused.

Proposal 20: If SCell Activation for Deactivated SCell with the L3 reporting during activation is supported for ATG, then the measurement reporting requirement shall also be introduced, legacy requirement can be reused.

Deactivated SCell measurement

The deactivated SCell measurement requirement shall be introduced, including Time period for PSS/SSS detection, Time period for time index detection, Measurement period for intra-frequency measurements without gaps for FR1 deactivated SCell. The requirements shall be discussed case by case according to different antenna type.

For ATG UE with one or more omni-directional antenna on SCC, the legacy requirement can be reused, which defined in Table 9.2.5.1-4, Table 9.2.5.1-6, and Table 9.2.5.2-3 in TS 38.133.

For ATG UE with antenna array on SCC, due to co-located deployment, UE could determine beam direction by serving gNB location, thus, the legacy requirement can be reused.

Proposal 21: Introduce Time period for PSS/SSS detection, Time period for time index detection, Measurement period for intra-frequency measurements without gaps for FR1 deactivated SCell. The legacy requirement can be reused, for ATG UE with one or more omni-directional antenna and ATG UE with antenna array on SCC.

Scheduling availability of UE performing measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1

For RLM, BFD, CBD, intra/inter-frequency measurement without MG, L1-RSRP measurement, L1-SINR measurement, scheduling availability of UE performing measurement with a different subcarrier spacing than PDSCH/PDCCH shall be discussed for CA configuration.

When intra-band carrier aggregation in FR1 is configured, due to one set of receiver will be applied on each carrier, the scheduling restrictions on FR1 serving PCell should apply to all serving cells in the same band on the symbols that fully or partially overlap with restricted symbols.

When inter-band carrier aggregation within FR1 is configured, due to UE mandatory capable of two sets of receiver, there are no scheduling restrictions on FR1 serving cell(s) configured in other bands than the bands in which PCell is configured.

Proposal 22: For scheduling availability of UE performing measurement with a different subcarrier spacing than PDSCH/PDCCH for RLM, BFD, CBD, intra/inter-frequency measurement without MG, L1-RSRP measurement, and L1-SINR measurement

- When intra-band carrier aggregation in FR1 is configured, the scheduling restrictions on FR1 serving PCell should apply to all serving cells in the same band on the symbols that fully or partially overlap with restricted symbols.

- When inter-band carrier aggregation within FR1 is configured, there are no scheduling restrictions on FR1 serving cell(s) configured in other bands than the bands in which PCell is configured.

Scheduling availability of UE performing measurements on FR1 TDD band

For intra/inter-frequency measurement without MG and CSI-RS based intra-frequency measurements, scheduling availability of UE performing measurements on FR1 TDD band shall be discussed for TDD CA configuration.

When TDD intra-band carrier aggregation is performed, due to one set of antenna and receiver will be applied on each carrier, the scheduling restrictions due to a given serving cell also apply to all other serving cells in the same band on the symbols that fully or partially overlap with aforementioned restricted symbols.

When TDD inter-band carrier aggregation is performed, the scheduling restrictions due to a given serving cell should also apply to another serving cell in a different band on the symbols that fully or partially overlap with the aforementioned restricted symbols, if UE does not have the capability of supporting simultaneousRxTxInterBandCA for this band pair.

Proposal 23: For scheduling availability of UE performing measurements on FR1 TDD band for intra/inter-frequency measurement without MG and CSI-RS based intra-frequency measurements

- When TDD intra-band carrier aggregation is performed, the scheduling restrictions due to a given serving cell also apply to all other serving cells in the same band on the symbols that fully or partially overlap with aforementioned restricted symbols.

- When TDD inter-band carrier aggregation is performed, the scheduling restrictions due to a given serving cell should also apply to another serving cell in a different band on the symbols that fully or partially overlap with the aforementioned restricted symbols, if UE does not have the capability of supporting simultaneousRxTxInterBandCA for this band pair.

Scheduling availability of UE with the antenna array performing measurements on FR1

For intra/inter-frequency measurement without MG and CSI-RS based intra-frequency measurements, scheduling availability of UE with the antenna array performing measurements on FR1 shall be discussed.

When intra-band carrier aggregation in FR1 is configured, due to one set of antenna will be applied on each carrier, the scheduling restrictions due to a given serving cell also apply to all other serving cells in the same band on the symbols that fully or partially overlap with aforementioned restricted symbols.

When inter-band carrier aggregation is performed:

For ATG UE with the antenna array on PCC and one or more omni-directional antenna on SCC, the scheduling restrictions due to PCell will not apply to another serving cell in a different band.

For ATG UE with the one or more omni-directional antenna on PCC and antenna array on SCC, the scheduling restrictions due to SCell will not apply to another serving cell in a different band.

For ATG UE with the antenna array on both PCC and SCC, and only one Rx beam can be formed, the scheduling restrictions due to a given serving cell should also apply to another serving cell in a different band on the symbols that fully or partially overlap with the aforementioned restricted symbols.

For ATG UE with the antenna array on both PCC and SCC, and two Rx beams can be formed simultaneously, he scheduling restrictions due to a given serving cell will not apply to another serving cell in a different band.

Proposal 24: For scheduling availability of UE with the antenna array performing measurements on FR1 for intra/inter-frequency measurement without MG and CSI-RS based intra-frequency measurements:

- When intra-band carrier aggregation in FR1 is configured, the scheduling restrictions due to a given serving cell also apply to all other serving cells in the same band on the symbols that fully or partially overlap with aforementioned restricted symbols.

- When inter-band carrier aggregation is performed:

  - For ATG UE with the antenna array on PCC and one or more omni-directional antenna on SCC, the scheduling restrictions due to PCell will not apply to another serving cell in a different band.

  - For ATG UE with the one or more omni-directional antenna on PCC and antenna array on SCC, the scheduling restrictions due to SCell will not apply to another serving cell in a different band.

  - For ATG UE with the antenna array on both PCC and SCC, and only one Rx beam can be formed, the scheduling restrictions due to a given serving cell should also apply to another serving cell in a different band on the symbols that fully or partially overlap with the aforementioned restricted symbols.

  - For ATG UE with the antenna array on both PCC and SCC, and two Rx beams can be formed simultaneously, he scheduling restrictions due to a given serving cell will not apply to another serving cell in a different band.

- Conclusion

In this contribution, we identify the potential RRM impact of ATG features. The proposals are:

Proposal 1: Focus on FR1 DL co-located CA scenario, including intra-band contiguous CA and inter-band CA.

Proposal 2: For other ATG scenario characteristics like UE speed, ISD and so on, the R18 working assumption will be reused.

Proposal 3: For intra-band contiguous CA, same antenna type should be applied on each carrier, including one or more omni-directional antenna(s) and antenna array.

Proposal 4: For inter-band CA, either different or same antenna type can be applied on different carriers.

Proposal 5: If only antenna array is used, then may exists two cases:

- Case 1: UE only have one antenna panel and form one Rx beam

- Case 2: UE have two antenna panels and can form two simultaneous Rx beams

Proposal 6: For cell re-selection, define the exceptions of side conditions for UE supporting CA in FR1. The legacy methodology exception applicability defined in Clause 4.2 and Clause B.3.2.1 can be reused with update of the reference clause of ΔRIB,c.

Observation 1: Fail to see strong demand and significant gain to introduce EMR in ATG co-located deployment.

Proposal 7: No need to introduce EMR in ATG CA if no strong demand and significant gain observed.

Proposal 8: No need to introduce MRTD requirement for ATG intra-band contiguous CA.

Proposal 9: Define MRTD requirement for ATG inter-band CA, legacy 33µs requirement can be reused.

Proposal 10: For SCell activation and deactivation, following features can be introduced for ATG, reuse the current FR1 requirements as the starting point, and further study whether further improvement is needed or not due to co-located deployment.

- SCell Activation Delay Requirement for Deactivated SCell

- SCell Deactivation Delay Requirement for Activated SCell

- Direct SCell Activation at SCell addition

- Direct SCell Activation at Handover

- Direct SCell Activation at RRCResume

- Fast SCell Activation Delay Requirement for Deactivated SCell

- SCell Activation Delay Requirement for Deactivated SCell with the L3 reporting during activation.

Proposal 11: For Active BWP switch delay on multiple CCs, introduce following requirements, which can reuse the legacy:

- Simultaneous DCI based BWP switch delay on multiple CCs

- Simultaneous RRC based BWP switch delay on multiple CCs

- Simultaneous and non-simultaneous Timer based BWP switch delay on multiple CCs

Proposal 12: Introduce Beam failure recovery in SCell for ATG CA, the legacy procedure and requirement can be reused.

Proposal 13: Introduce Pre-configured measurement gap activation/deactivation delay requirement upon SCell activation/deactivation, the legacy requirement can be reused.

Proposal 14: Introduce the following interruption requirement, the legacy requirement can be reused.

- Interruptions at SCell addition/release

- Interruptions at SCell activation/deactivation

- Interruptions at direct SCell activation

- Interruptions at fast SCell activation

- Interruptions during measurements on deactivated SCC

- Interruptions due to SCell dormancy

- Interruptions due to Active BWP switching Requirement

- Interruptions due to UE-specific CBW change

- Interruptions when identifying CGI of an NR cell with autonomous gaps

- Interruptions at NR SRS antenna port switching

Proposal 15: Reuse the legacy scheme when defining the CSSFoutside_gap for ATG FR1 only CA, that PCC occupy one measurement searcher resource, SCC and other inter-frequency MO with no measurement gap share another measurement searcher resource.

Proposal 16: The measurement gap applicability rule and scheduling applicability for SA single carrier shall be also applied for NR CA configuration.

Proposal 17: When define the SCC interruption time, only consider aligned frame boundaries scenario, the interruption requirement in single carrier case can be reused for CA configuration.

Proposal 18: For UE capability of supporting NR reporting criteria of category intra-frequency, the SCell serving frequency shall be counted in the number of configured NR serving frequencies carrier frequency.

Proposal 19: Introduce the SCell related triggering conditions as an event for triggering pre-configured measurement gap, and the legacy procedure and requirement can be reused.

Proposal 20: If SCell Activation for Deactivated SCell with the L3 reporting during activation is supported for ATG, then the measurement reporting requirement shall also be introduced, legacy requirement can be reused.

Proposal 21: Introduce Time period for PSS/SSS detection, Time period for time index detection, Measurement period for intra-frequency measurements without gaps for FR1 deactivated SCell. The legacy requirement can be reused, for ATG UE with one or more omni-directional antenna and ATG UE with antenna array on SCC.

Proposal 22: For scheduling availability of UE performing measurement with a different subcarrier spacing than PDSCH/PDCCH for RLM, BFD, CBD, intra/inter-frequency measurement without MG, L1-RSRP measurement, and L1-SINR measurement

- When intra-band carrier aggregation in FR1 is configured, the scheduling restrictions on FR1 serving PCell should apply to all serving cells in the same band on the symbols that fully or partially overlap with restricted symbols.

- When inter-band carrier aggregation within FR1 is configured, there are no scheduling restrictions on FR1 serving cell(s) configured in other bands than the bands in which PCell is configured.

Proposal 23: For scheduling availability of UE performing measurements on FR1 TDD band for intra/inter-frequency measurement without MG and CSI-RS based intra-frequency measurements

- When TDD intra-band carrier aggregation is performed, the scheduling restrictions due to a given serving cell also apply to all other serving cells in the same band on the symbols that fully or partially overlap with aforementioned restricted symbols.

- When TDD inter-band carrier aggregation is performed, the scheduling restrictions due to a given serving cell should also apply to another serving cell in a different band on the symbols that fully or partially overlap with the aforementioned restricted symbols, if UE does not have the capability of supporting simultaneousRxTxInterBandCA for this band pair.

Proposal 24: For scheduling availability of UE with the antenna array performing measurements on FR1 for intra/inter-frequency measurement without MG and CSI-RS based intra-frequency measurements:

- When intra-band carrier aggregation in FR1 is configured, the scheduling restrictions due to a given serving cell also apply to all other serving cells in the same band on the symbols that fully or partially overlap with aforementioned restricted symbols.

- When inter-band carrier aggregation is performed:

  - For ATG UE with the antenna array on PCC and one or more omni-directional antenna on SCC, the scheduling restrictions due to PCell will not apply to another serving cell in a different band.

  - For ATG UE with the one or more omni-directional antenna on PCC and antenna array on SCC, the scheduling restrictions due to SCell will not apply to another serving cell in a different band.

  - For ATG UE with the antenna array on both PCC and SCC, and only one Rx beam can be formed, the scheduling restrictions due to a given serving cell should also apply to another serving cell in a different band on the symbols that fully or partially overlap with the aforementioned restricted symbols.

  - For ATG UE with the antenna array on both PCC and SCC, and two Rx beams can be formed simultaneously, he scheduling restrictions due to a given serving cell will not apply to another serving cell in a different band.

- Reference

- RP-241213, Revised WID on Enhancements for Air-to-ground network for NR, CMCC.

- R4-2410687, WF on Rel-19 ATG UE requirements, CMCC