3GPP TSG-RAN WG4 Meeting # 112      	R4-2411759

# Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda Item:	8.16.2

Source: 	CMCC

Title: 			 (NR_demod_Ph5-Perf) Discussion on interference suppressing performance for 8Rx UE

Document for:	Discussion

- Introduction

In RAN#103-e meeting, the work item on Demodulation performance requirements enhancements has been approved. According to the latest revised WID [1], the UE interference suppressing performance requirements for 8Rx CPE/FWA/vehicle/industrial devices should be specified, including inter-cell interference and intra-cell inter-user interference scenario. We copy the objective as below for information:

- Define the following UE performance requirements for 8Rx CPE/FWA/vehicle/industrial devices: - PDSCH demodulation and CQI reporting requirements with inter-cell interference with MMSE-IRC receiver - Reuse the Rel-17 interference model for 2/4Rx UE as a baseline - PDSCH demodulation requirements with intra-cell inter-user interference with MMSE-IRC receiver - Reuse the existing interference model for 2/4Rx UE as a baseline

In this contribution, we will discuss the network scenario and some test parameters.

- Discussion

  - PDSCH demodulation requirements with MMSE-IRC receiver for inter-cell interference scenario

Interference Scenario

In Rel-17 MMSE-IRC receiver for suppressing inter-cell interference research from Demodulation enhancement WI, both Homogeneous and Heterogeneous network deployment was finally specified. For Rel-19 8Rx UE MMSE-IRC receiver, these two interference scenarios should also be introduced, reuse the Rel-17 interference model as a baseline.

Proposal 1: Introduce both Homogeneous and Heterogeneous network deployment, reuse the Rel-17 interference model as a baseline.

Network Type and SCS/CBW

We propose to reuse Rel-17 network type and SCS/CBW setup, which are:

FR1 FDD sync network, 15kHz/10MHz

FR1 TDD sync network, 30kHz/40MHz

Proposal 2: reuse Rel-17 network type and SCS/CBW setup, which are:

- FR1 FDD sync network, 15kHz/10MHz

- FR1 TDD sync network, 30kHz/40MHz

Rank for serving cell

In UE 8Rx performance requirement, Rank 2/4/8 was specified corresponding to 2/4/8Tx respectively. However, when we considering interference scenario, that UE locates on cell edge with low SINR, it is hard to reach 8 layer transmission. Hence, we propose to consider Rank 2 and Rank 4 as the starting point.

Proposal 3: For serving cell, consider Rank 2 and Rank 4 transmission as the starting point.

Propagation condition for serving cell

In UE 8Rx performance requirement, TDLC300-100 is used for rank 2 transmission and TDLA30-10 is used for rank 4 transmission.

However, for TDLC300ns delay spread model, the maximum excess tap delay will reach 2595ns, the minimum coherence bandwidth is about 0.385MHz, which is smaller than 2PRB and larger than 1PRB in 30kHz scenario. For now, we are not sure whether 1PRB granularity could acquire relatively accurate interference-plus-noise covariance estimation result for 8Rx. Therefore, we propose to use TDLA30-10 as the baseline, and FFS whether interference suppressing gain can be reached in TDLC300-100 by simulation.

Proposal 4: For serving cell propagation condition, consider TDLA30-10 as the starting point, and further decide whether to cover TDLC300-100 by simulating interference suppressing gain.

Other parameters

For TDD pattern, MCS for serving cell, PDSCH configuration, DMRS configuration, TRS configuration, HARQ, precoding, Tx EVM, Test metric and other parameters, we propose to reuse the configuration from 8Rx requirement as the baseline.

| Parameter |  | Value |
| --- | --- | --- |
| TDD pattern |  | 7D1S2U S=6D:4G:4U |
| MCS for serving cell |  | Rank 2: MCS19 (Table 1) Rank 4: MCS17 (Table 1) |
| PDSCH configuration | Mapping type | Type A |
|  | k0 | 0 |
|  | Starting symbol (S) | 2 |
|  | Length (L) | 12 |
|  | PRB bundling type | Static |
|  | PRB bundling size | 2 |
| PDSCH DMRS configuration | DMRS Type | Type 1 |
|  | Number of additional DMRS | 1 |
|  | Maximum number of OFDM symbols (maxLength) for DL | Single-symbol |
| TRS configuration |  | Symbol#{5, 9} |
| Overhead |  | 0 |
| Codebook for PDCCH for PDSCH tests |  | Keep same number of Tx for PDSCH and PDCCH during PDSCH test. Set “codebookMode” to 1 |
| Number of HARQ Processes |  | TDD: 8 FDD: 4 |
| HARQ ACK/NACK bundling |  | Multiplexed |
| Maximum HARQ transmissions |  | 4 |
| Redundancy version coding sequence |  | {0,2,3,1} |
| PDSCH & PDSCH DMRS Precoding configuration |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with PRB bundling granularity |
| Tx EVM (Explicitly modeled in the simulation) |  | 64QAM:6% 256QAM:3% |
| Test metric |  | SNR@70% max TP |

Proposal 5: For TDD pattern, MCS, PDSCH configuration, DMRS configuration, TRS configuration, HARQ, precoding, Tx EVM, Test metric and other parameters, reuse the configuration from 8Rx requirement as the baseline.

For PCI and SSB configuration of serving cell and interference cell(s), Rank/MCS, precoding, timing and frequency offset, and other interference cell modeling parameters, we propose to reuse the configuration from R17 2/4Rx inter-cell interference suppressing requirement as the baseline.

| Parameter | Value |
| --- | --- |
| Physical cell ID | Physical cell ID of 0 for the serving cell, and cell ID 1 for interferer 1, cell ID 2 for interferer 2 in case of 2 interference cells |
| SSB configuration | For homogeneous network, all SSBs (serving cell and interference cells) are in the same time/frequency resources, i.e., first SSB in Slot #0 for all cells. For heterogeneous network, serving cell SSB and interference cell(s) SSB(s) are in the different time/frequency resources, i.e., first SSB in Slot #0 forserving cell, and second SSB in Slot #0 for interference cell. |
| Transmission rank of interfering PDSCH | random rank with 70% and 30% probability for rank 1 and rank 2 transmission in the interfering cell(s) |
| Precoding of interfering PDSCH | Random precoding with single panel type I codebook per slot and per PRB bundling granularity, with PRB bundling size of 2. |
| Modulation order of interfering PDSCH | Use 16QAM randomly modulated symbols for initial simulation assumption |
| Timing and frequency offset | The serving cell is 1.5 us for interfering cell 1 and -0.5 us for interfering cell 2 The serving cell is 300 Hz for interfering cell 1 and -100 Hz for interfering cell 2 |
| Interference cell modeling | 2Tx Interfering cell has same channel model as for serving cell and it is independently generated for each cell. Interfering cell has full PDSCH allocation on all slots where serving cell has PDSCH grant. |

Proposal 6: For PCI and SSB configuration of serving cell and interference cell(s), Rank/MCS, precoding, timing and frequency offset, and other interference cell modeling parameters, reuse the configuration from R17 2/4Rx inter-cell interference suppressing requirement as the baseline.

Release independent

If no new signalling or UE capability introduced, the requirement should be released independent from Rel-18.

Proposal 7: the requirement should be released independent from Rel-18 if no new signalling or UE capability introduced.

  - CQI reporting requirements with MMSE-IRC receiver for inter-cell interference scenario

Propagation channel

AWGN static channel can be used as baseline, further study whether to involve fading channel.

Proposal 8: Use AWGN static channel as the baseline, further study whether to involve fading channel.

Antenna configuration and Rank for serving cell

For antenna configuration, the setup 4T8R from 8Rx CQI reporting requirement can be reused.

For rank, due to inter-cell interference is considered, lower than Rank 4 reporting is also possible under low SINR scenario, we propose no RI restriction configured.

Proposal 9: Consider 4T8R and no RI restriction configured for simulation.

Other configuration for serving cell

For other serving cell configuration, the setup from 8Rx CQI reporting requirement can be reused for simulation.

Proposal 10: For other serving cell configuration, the setup from 8Rx CQI reporting requirement can be reused for simulation.

Interference profile

INR 10.04dB can be reused for initial simulation.

Proposal 11: Reuse INR 10.04dB for initial simulation.

CSI-IM and NZP CSI-RS assumption

The agreement from R17 2/4Rx inter-cell interference suppressing requirement can be reused for simulation, that:

CSI-IM on serving cell overlapping with PDSCH from interference cell, and

NZP CSI-RS on serving cell overlapping with NZP CSI-RS from interference cell

Proposal 12: For CSI-IM and NZP CSI-RS assumption, reuse agreement from R17 2/4Rx inter-cell interference suppressing requirement for simulation, which are:

- CSI-IM on serving cell overlapping with PDSCH from interference cell

- NZP CSI-RS on serving cell overlapping with NZP CSI-RS from interference cell

  - PDSCH demodulation requirements with MMSE-IRC receiver for intra-cell inter-user interference scenario

Interference Scenario

As in WID, the existing interference model for 2/4Rx UE should be used as the baseline, which one target UE with one co-scheduled UE.

Besides, for 8Tx scenario, more than one paired UE may be scheduled in real MU-MIMO network. We also have interest to further study whether interference suppressing gain can be achieved by MMSE-IRC receiver in such scenario.

Proposal 13: Use 1 target UE + 1 interference UE as starting point for initial simulation.

Proposal 14: For 8Tx, further study whether interference suppressing gain can be achieved by MMSE-IRC receiver in more than one paired UE scenario.

Duplex mode/SCS/CBW/TDD pattern

To minimize test effort, we propose to test the most common configuration, that FDD 15kHz SCS with 10MHz CBW, and TDD 30kHz SCS with 40MHz CBW and 7D1S2U TDD pattern.

Proposal 15: Use FDD 15kHz/10MHz and TDD 30kHz/40MHz 7D1S2U for initial simulation

Propagation condition

TDLC300-100 and TDLA30-10 was usually used for define requirements.

However, for TDLC300ns delay spread model, the maximum excess tap delay will reach 2595ns, the minimum coherence bandwidth is about 0.385MHz, which is smaller than 2PRB and larger than 1PRB in 30kHz scenario. For now, we are not sure whether 1PRB granularity could acquire relatively accurate interference-plus-noise covariance estimation result for 8Rx. Therefore, we propose to use TDLA30-10 as the baseline, and FFS whether interference suppressing gain can be reached in TDLC300-100 by simulation.

Proposal 16: For propagation condition, consider TDLA30-10 as the starting point, and further decide whether to cover TDLC300-100 by simulating interference suppressing gain.

Antenna configuration and rank

We propose to cover 2T8R, 4T8R and 8T8R for initial simulation. Same rank value for both target and paired UE can be considered for simplification, Rank 2+2 for 2T8R, Rank 4+4 for 4T8R, Rank 8+8 for 8T8R when 1 paired UE scheduled.

Proposal 17: Following rank and antenna configuration can be used for initial simulation

- Rank 2+2 for 2T8R

- Rank 4+4 for 4T8R

- Rank 8+8 for 8T8R when 1 paired UE scheduled

MCS

For target UE, propose to use the assumption from 8Rx requirement, that MCS19 Table 1 for Rank 2, MCS17 Table 1 for Rank 4 and MCS 17 Table 1 for Rank 8. For paired UE, propose to use random 16QAM signal generation as legacy.

Proposal 18: Following MCS configuration can be used for initial simulation

- For target UE, MCS19 Table 1 for Rank 2, MCS17 Table 1 for Rank 4 and MCS 17 Table 1 for Rank 8

- For paired UE, random 16QAM

Codebook and precoding configuration

Similar as 8Rx requirement, single Panel Type I codebook can be taken as the baseline. For target UE, use random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination. For co-scheduled UE, select the precoding matrix to ensure orthogonality with target UE. The precoding granularity and PRB bundling size can be assumed as 2PRBs.

Proposal 19: For initial simulation, use following configuration as the baseline:

- single Panel Type I codebook

- precoding granularity and PRB bundling size set as 2PRBs

- for target UE, random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination

- for co-scheduled UE, select the precoding matrix to ensure orthogonality with target UE

- Conclusion

In this contribution, we provides some basic simulation assumptions for UE 8Rx interference suppressing requirement. The proposals are:

Proposal 1: Introduce both Homogeneous and Heterogeneous network deployment, reuse the Rel-17 interference model as a baseline.

Proposal 2: reuse Rel-17 network type and SCS/CBW setup, which are:

- FR1 FDD sync network, 15kHz/10MHz

- FR1 TDD sync network, 30kHz/40MHz

Proposal 3: For serving cell, consider Rank 2 and Rank 4 transmission as the starting point.

Proposal 4: For serving cell propagation condition, consider TDLA30-10 as the starting point, and further decide whether to cover TDLC300-100 by simulating interference suppressing gain.

Proposal 5: For TDD pattern, MCS, PDSCH configuration, DMRS configuration, TRS configuration, HARQ, precoding, Tx EVM, Test metric and other parameters, reuse the configuration from 8Rx requirement as the baseline.

Proposal 6: For PCI and SSB configuration of serving cell and interference cell(s), Rank/MCS, precoding, timing and frequency offset, and other interference cell modeling parameters, reuse the configuration from R17 2/4Rx inter-cell interference suppressing requirement as the baseline.

Proposal 7: the requirement should be released independent from Rel-18 if no new signalling or UE capability introduced.

Proposal 8: Use AWGN static channel as the baseline, further study whether to involve fading channel.

Proposal 9: Consider 4T8R and no RI restriction configured for simulation.

Proposal 10: For other serving cell configuration, the setup from 8Rx CQI reporting requirement can be reused for simulation.

Proposal 11: Reuse INR 10.04dB for initial simulation.

Proposal 12: For CSI-IM and NZP CSI-RS assumption, reuse agreement from R17 2/4Rx inter-cell interference suppressing requirement for simulation, which are:

- CSI-IM on serving cell overlapping with PDSCH from interference cell

- NZP CSI-RS on serving cell overlapping with NZP CSI-RS from interference cell

Proposal 13: Use 1 target UE + 1 interference UE as starting point for initial simulation.

Proposal 14: For 8Tx, further study whether interference suppressing gain can be achieved by MMSE-IRC receiver in more than one paired UE scenario.

Proposal 15: Use FDD 15kHz/10MHz and TDD 30kHz/40MHz 7D1S2U for initial simulation

Proposal 16: For propagation condition, consider TDLA30-10 as the starting point, and further decide whether to cover TDLC300-100 by simulating interference suppressing gain.

Proposal 17: Following rank and antenna configuration can be used for initial simulation

- Rank 2+2 for 2T8R

- Rank 4+4 for 4T8R

- Rank 8+8 for 8T8R when 1 paired UE scheduled

Proposal 18: Following MCS configuration can be used for initial simulation

- For target UE, MCS19 Table 1 for Rank 2, MCS17 Table 1 for Rank 4 and MCS 17 Table 1 for Rank 8

- For paired UE, random 16QAM

Proposal 19: For initial simulation, use following configuration as the baseline:

- single Panel Type I codebook

- precoding granularity and PRB bundling size set as 2PRBs

- for target UE, random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination

- for co-scheduled UE, select the precoding matrix to ensure orthogonality with target UE

- Reference

[1] RP-241297, Revised WID on NR demodulation performance: Phase 5, China Telecom, NTT DOCOMO.