3GPP TSG-RAN WG4 Meeting # 112      	R4-2411760

# Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda Item:	8.16.3

Source: 	CMCC

Title: 		    (NR_demod_Ph5-Perf) Discussion on BS demodulation requirements for interference suppressing with MMSE-IRC receiver

Document for:	Discussion

- Introduction

In RAN#103-e meeting, the work item on Demodulation performance requirements enhancements has been approved. According to the latest revised WID [1], the BS interference suppressing performance requirements with MMSE-IRC receiver should be specified. We copy the objective as below for information:

- Define FR1 PUSCH performance requirements with inter-cell interference in homogeneous and heterogeneous deployment based on the following assumptions: - Receiver type: MMSE-IRC receiver - Scenarios: - FDD synchronous deployments - TDD synchronous deployments with aligned UL:DL configurations among cells - Slot-based transmission and aligned SCS among cells - Reuse LTE based interference profile in TR 36.884 as a starting point. Other interference profiles are not precluded.

In this contribution, we will discuss the network scenario and some test parameters.

- Discussion

Interference Scenario

In current scope, homogeneous and heterogeneous deployment was agreed to be specified. Furthermore, for TDD, the UL:DL configurations among cells should be aligned.

Beyond the current scope of WID, in HetNet deployment, we think different TDD pattern is also a potential scenario, that Micro cell use more U slots to fulfill UL capacity demand. In such case, there is cross link interference, that the D or S slot of Macro Cell will cause interference to the U slots of Micro Cell. We support to extend the scope to also cover BS CLI problem in HetNet unaligned TDD pattern scenario.

Proposal 1: Extend the scope to also cover BS CLI problem in HetNet unaligned TDD pattern scenario.

Interference profile

By reusing LTE based interference profile in TR 36.884 as a starting point, DIP1 = -1.11 for Interferer 1 and DIP2 = -10.91 for Interferer 2 should be used for Homogeneous network, DIP1 = -0.43 for Interferer 1 and DIP2 = -13.78 for Interferer 2 should be used for Heterogeneous network.

Proposal 2: Reuse LTE based interference profile in TR 36.884 for initial simulation, that are DIP1 = -1.11 and DIP2 = -10.91 for Homogeneous network, DIP1 = -0.43 and DIP2 = -13.78 for Heterogeneous network.

SCS/CBW

10MHz/15kHz and 40MHz/30kHz can be used as baseline for initial simulation.

Proposal 3: For initial simulation, take 10MHz/15kHz and 40MHz/30kHz as baseline.

Propagation condition

For initial simulation purpose, propose to use TDLA30-10 as the baseline, and further evaluate whether performance gain can be achieved under TDLC300-100 for Homogeneous network.

Proposal 4: For initial simulation, use TDLA30-10 as the baseline, and further evaluate whether performance gain can be achieved under TDLC300-100 for Homogeneous network.

Antenna configuration

Same antenna configuration can be used among target and interferer(s) in order to minimize test scope. 1T2R, 1T4R, and 1T8R can be used as the starting point.

Proposal 5: Same antenna configuration can be applied among target and interferer(s), take 1T2R, 1T4R, and 1T8R as the starting point for initial simulation.

- Conclusion

In this contribution, we provides some basic simulation assumptions for BS 8Rx interference suppressing requirement. The proposals are:

Proposal 1: Extend the scope to also cover BS CLI problem in HetNet unaligned TDD pattern scenario.

Proposal 2: Reuse LTE based interference profile in TR 36.884 for initial simulation, that are DIP1 = -1.11 and DIP2 = -10.91 for Homogeneous network, DIP1 = -0.43 and DIP2 = -13.78 for Heterogeneous network.

Proposal 3: For initial simulation, take 10MHz/15kHz and 40MHz/30kHz as baseline.

Proposal 4: For initial simulation, use TDLA30-10 as the baseline, and further evaluate whether performance gain can be achieved under TDLC300-100 for Homogeneous network.

Proposal 5: Same antenna configuration can be applied among target and interferer(s), take 1T2R, 1T4R, and 1T8R as the starting point for initial simulation.

- Reference

[1] RP-241297, Revised WID on NR demodulation performance: Phase 5, China Telecom, NTT DOCOMO.