3GPP TSG-RAN WG4 Meeting # 112		R4-2411764

# Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda Item:	8.26.3

Source: 	CMCC

Title: 			 (IoT_NTN_Ph3-Core)  Discussion on LS from RAN2 of Msg3 transmission timing

Document for:	Discussion

- Introduction

RAN2 has sent an LS to RAN4 and RAN1 about the RAN2’s agreement related to Msg3 transmission for uplink capacity enhancement [1]. In the LS, RAN2 asked RAN4 and RAN1 about following questions:

Q1: Whether an RRC Idle UE with a pre-compensated TA (i.e., the one used for Msg1 transmission during random access for IoT NTN) can satisfy the required timing accuracy for Msg3 transmission without Msg1/Msg2? Q2: If the answer for Q1 is no, from RAN4 and RAN1 perspective, how the required timing accuracy for Msg3 transmission can be satisfied in this case?

In this contribution, we give our analysis for NB-IOT UE and eMTC UE respectively.

- Discussion

For NB-IOT UE over NTN, the UE initial UL transmission timing error for the first transmission in a DRX cycle or the first transmission in a repetition period (R>1) for NPUSCH and NPRACH shall be less than or equal to Te, which equals to 97Ts = 80Ts +17Ts. 80Ts is the legacy NB-IOT UE initial transmission timing error, 17Ts is the additional error caused by GNSS error and satellite location derivation error. The reference point for the UE initial transmit timing control requirement shall be the downlink timing of the serving NB-IoT cell minus NTA_Ref+NTAoffset+NTA,common+NTA, UE−specific×Ts.

When contention-based Msg3 transmission without Msg1/Msg2 configured, for RRC IDLE UE with a pre-compensated TA (i.e., the one used for Msg1 transmission during random access for IoT NTN), the current initial transmission timing error Te = 97Ts should also be applied for Msg3 with NTA_Ref is defined as 0.

To avoid ISI and potential ICI, the required timing accuracy for Msg3 is up to CP/2. We list the relationship between required timing accuracy of different channel and Te requirement in Table 1

Table 1 Relationship between required timing accuracy of different channel and Te requirement

| Channel | Tcp/2 (required timing accuracy) | Te for NB-IOT | Te for NB-IOT over NTN |
| --- | --- | --- | --- |
| NPUSCH (Msg 3) | 80Ts for NCP for the first OFDM symbol 72Ts for NCP for the others 256Ts for ECP | 80Ts | 97Ts |
| NPRACH (Msg 1) | 1024Ts for preamble format 0 4096Ts for preamble format 1 | 80Ts | 97Ts |

Based on the Table 1, we have following observations:

For legacy NB-IOT UE over TN network, 80Ts required accuracy larger than NCP/2 of NPUSCH OFDM symbols except the first OFDM symbol, which will cause ISI issue.

For NB-IOT UE over NTN network, 97Ts required accuracy larger than NCP/2 of NPUSCH OFDM symbols, which will cause ISI issue.

Observation 1: For both legacy NB-IOT and NB-IOT over NTN, Te is equal to or larger than NCP/2 of NPUSCH symbols, which will cause ISI issue.

In our understanding, the performance degradation caused by ISI in legacy NB-IOT and NB-IOT over NTN is tolerable considering power consumption demand.

However, when UE use Msg3 for network initial access, whether the performance degradation caused by ISI is tolerable or not should be further studied.

If not tolerable, there are several methods to address this issue:

- For contention-based Msg3 transmission to complete an EDT-like transaction, use ECP by default

- Define enhanced UL transmit timing requirement assuming UE can perform FFT with larger size.

Proposal 1: For NB-IOT over NTN contention-based Msg3 transmission to complete an EDT-like transaction, whether the performance degradation caused by ISI is tolerable or not should be further studied, and if not, following methods can be considered:

- For contention-based Msg3 transmission to complete an EDT-like transaction, use ECP by default

- Define enhanced UL transmit timing requirement assuming UE can perform FFT with larger size.

For eMTC UE over NTN, the UE initial UL transmission timing error for the first transmission in a DRX cycle or the first transmission in a repetition period (R>1) for PUCCH, PUSCH, SRS and PRACH shall be less than or equal to Te_NTN_M1, which equals to 41Ts for CEMode A and 65Ts for CEMode B.The reference point for the UE initial transmit timing control requirement shall be the downlink timing of the serving cell minus NTA_Ref+NTAoffset+NTA,common+NTA, UE−specific×Ts.

When contention-based Msg3 transmission without Msg1/Msg2 configured, for RRC IDLE UE with a pre-compensated TA (i.e., the one used for Msg1 transmission during random access for IoT NTN), the current initial transmission timing error Te_NTN_M1 should also be applied for Msg3 with NTA_Ref is defined as 0.

For both CEMode A and CEMode B, the Te_NTN_M1 is smaller than CP/2. The required timing accuracy can be satisfied.

Proposal 2: For eMTC over NTN contention-based Msg3 transmission to complete an EDT-like transaction, the required timing accuracy can be satisfied.

- Conclusion

Based on the discussion above, the following observations and proposals are concluded.

Observation 1: For both legacy NB-IOT and NB-IOT over NTN, Te is equal to or larger than NCP/2 of NPUSCH symbols, which will cause ISI issue.

Proposal 1: For NB-IOT over NTN contention-based Msg3 transmission to complete an EDT-like transaction, whether the performance degradation caused by ISI is tolerable or not should be further studied, and if not, following methods can be considered:

- For contention-based Msg3 transmission to complete an EDT-like transaction, use ECP by default

- Define enhanced UL transmit timing requirement assuming UE can perform FFT with larger size.

Proposal 2: For eMTC over NTN contention-based Msg3 transmission to complete an EDT-like transaction, the required timing accuracy can be satisfied.

- Reference

- R2-2405769, LS on UL synchronization for contention based Msg3 transmission without Msg1/Msg2, Huawei, HiSilicon