# 3GPP TSG-RAN WG4 Meeting #112	R4-2411768

# Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:	8.20.3.2

Source: 	CMCC

Title: 	Discussion on A-IoT device RF requirements

Document for:	Discussion

# 1. Introduction

In RAN plenary #102 meeting, a new SID[1] of A-IoT is approved with following RAN4 lead RF part objectives:

- Coexistence study of Ambient IoT and NR/LTE.

- RF requirements study for Ambient IoT:

  - Ambient IoT BS transmission and reception

  - Ambient IoT Device, as per the General Scope, transmission and reception

  - Intermediate node (UE), as per the General Scope, transmission and reception

This is the first meeting of RF requirements discussion, in this contribution we list our analysis for A-IoT devices.

- Discussion

  - system parameter

For system parameter, it’s suggested to define one unified set of system parameter for A-IoT BS, A-IoT intermediate UE and devices.

Proposal 1: it’s suggested to define one unified set of system parameter for A-IoT BS, A-IoT intermediate UE and devices.

- operation band

For band specific requirements, to reduce workload, it’s suggested to only choose one example band for further study. Band n8 the 900MHz is suggested as example band.

Proposal 2: it’s suggested to only choose band n8 as example band.

- Last meeting system parameter related agreements are listed as below:

Agreement: - RAN4 will define the D2R and/or R2D channel bandwidth and operating bands for A-IoT - Wait for the conclusions from other WGs to discuss the detailed parameters. - FFS on whether to have the same or different channel bandwidths for devices, BS and intermediate node

- Bandwidth related

Following bandwidths for D2R has been defined for the purpose of the study in RAN1

Agreement The following bandwidths for D2R are defined for the purpose of the study: - Transmission bandwidth, Btx,D2R: The frequency resources scheduled by a reader for a D2R transmission from one device. - FFS in agenda 9.4.2.3: how frequency resources scheduled by a reader are determined - Occupied bandwidth, Bocc,D2R: The transmission bandwidth plus the potential associated intra A-IoT guard-bands totalling Bguard,D2R - Note: this guard band is not for coexistence with NR/LTE - If/how to define guard band for coexistence between A-IoT D2R and NR/LTE is up to RAN4. - Bocc,D2R >= Btx,D2R - Possible values of each bandwidth are FFS

- For transmission bandwidth, this is related to UL information data, data coded scheme, modulation scheme etc, which is something like the scheduled frequency resources and is not specified into current UE spec. It seems we don’t need to define transmission bandwidth requirements, instead such terminology is mainly used for evaluation/analysis.

Proposal 3: no need to define transmission bandwidth in RAN4.

- For occupied bandwidth, this is mainly used to evaluate the intra A-IoT guard-band rather than used for co-existence with NR/LTE. So from RAN1 point of view, this guard RB value is mainly used to reduce or even avoid interference for device FDM operation case or for A-IoT reader FDM operation case, e.g. reducing the interference from device that using first occupied carrier to the reader that using another occupied carrier. Such interference could be even larger if reader or device have bad frequency accuracy performance. Current RAN4 co-existence evaluation doesn’t include device nor reader FDM operation mode, so we can’t conclude guard RB from simulation results.

- Observation 1: Intra-A-IoT system guard RB is mainly used to reduce or even avoid interference for device FDM operation case or for A-IoT reader FDM operation case. Such interference would be even worse if reader or device have bad frequency accuracy performance.

- For standalone spectrum deployment mode, another bandwidth related terminology is the channel bandwidth. RAN4 needs to further discuss the candidate channel bandwidth for standalone spectrum deployment mode. Based on current data rate and modulation scheme assumption, 5MHz bandwidth is enough for standalone spectrum deployment mode. For in-band and guard band spectrum deployment modes, channel bandwidth also seems necessary as common concept which may be used by reader.

Proposal 4: RAN4 further discuss the candidate channel bandwidth for all three spectrum deployment modes. For standalone mode, it seems 5MHz bandwidth is enough based on current assumed data rate and modulation scheme. For other two spectrum deployment mode, the channel bandwidth also seems necessary as common concept which may be used by reader.

- Besides intra-A-IoT guard band, other guard band is required to avoid interference between A-IoT system and NR/LTE system

According to current evaluation priority, RAN4 only focus on in-band spectrum mode, if final simulation results show that legacy assumed Tx leakage and Rx selectivity performance is not enough. Then guard band could be help to enhance such performance and avoid interference between A-IoT and NR/LTE system.

For guard band spectrum deployment mode, we should also check whether any guard band between edge of max transmission bandwidth of NR carrier and the edge of A-IoT transmission bandwidth is needed or not. Here the guard band is not need to be in granularity of RB, instead it could be in any granularity. Detailed illustration of guard band is listed as below

Proposal 5: RAN4 further discuss whether any guard band is needed or not to avoid interference between A-IoT system and NR/LTE system.

- For guard-band spectrum deployment mode, guard band can be in any granularity.

- For in-band spectrum deployment mode, guard band could help to enhance current Tx leakage and Rx selectivity performance and detailed value can wait for co-existence results.

- Channel spacing/ channel raster and syn raster

There is no SSB anymore for A-IoT. So syn raster is not applicable for all three spectrum deployment mode. For guard-band mode, the channel raster and channel spacing is not applicable. For the in-band mode, the channel spacing is not applicable.

Proposal 6: syn raster is not applicable for all three spectrum deployment mode. For guard-band mode, the channel raster and channel spacing are not applicable. For in-band mode, the channel spacing is not applicable.

- Standalone: channel raster, channel spacing

- In-band: channel raster

- Guard-band: N/A

2.2 Device class

Currently there are three types of devices that are under study by RAN1 in study phase as below:

Agreement For the purpose of the study, RAN1 uses the following terminologies: - Device 1: ~1 µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, neither DL nor UL amplification in the device. The device’s UL transmission is backscattered on a carrier wave provided externally. - Device 2a: ≤ a few hundred µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, both DL and/or UL amplification in the device. The device’s UL transmission is backscattered on a carrier wave provided externally. - Device 2b: ≤ a few hundred µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, both DL and/or UL amplification in the device. The device’s UL transmission is generated internally by the device.

Based on above terminologies, three devices have different architecture with different RF requirement, e.g. backscattered power or active output power. It’s reasonable to discuss RF requirements separately for these three device types.

Proposal 7: as starting point, it’s suggested to discuss RF requirements separately for three device types.

2.3 Device 1

For device 1, considering its low cost, it’s better to use RFID spec as baseline which only has output power and unwanted emission requirements. Besides, for Rx part, it’s better to add REFSENSE requirements.

Proposal 8: for device 1, taking RFID RF requirements as reference which only define output power and unwanted emission requirements. Besides, REFSENSE requirement is also needed.

2.4 Device 2a

Following list RAN1 agreement as below for information

Agreement For D2R, study: Manchester encoding, FM0 encoding, Miller encoding, no line coding. - FFS: Mapping(s) from bit(s) to line-code codewords - FFS: How to achieve small frequency shift in baseband and/or FDM(A) among devices - Aspects to study include: - Spectrum shape - Complexity - Power consumption - BER, BLER - Resilience to SFO - If there is any relation to CFO

Following table lists our initial analysis for Tx requirements of device 2a.

| requirements | Applicable or not |
| --- | --- |
| Transmitter power | All three kinds of device have different output power level. RAN4 further discuss how to define power class based on different device type and architecture. |
| Output power dynamic | Not applicable |
| Power control | Not applicable |
| Transmit signal quality | Frequency error: wait for modulation scheme conclusion EVM: wait for modulation scheme conclusion Carrier leakage: may be needed based on the design of small frequency shift in baseband In-band emission: needed. |
| RF spectrum emissions | Out of band emission: at least for standalone, it’s needed. For in-band and guard-band spectrum deployment mode, FFS how to define such requirements. Spurious emission requirement: needed to meet regulatory requirement. if large frequency shifter is considered for device 2a, RAN4 should further check the spurious emission at specific frequency where harmonic occurs. |

Proposal 9: at least for transmit signal quality related requirement, RAN4 should wait for RAN1 conclusion of How to achieve small frequency shift in baseband and/or FDM(A) among devices.

Proposal 10: if RAN4 only simulate in-band spectrum mode, RAN4 further discuss whether/how to define out of band emission requirement.

Following table list our initial analysis for Tx requirements.

| requirements | Applicable or not |
| --- | --- |
| REFSENSE | Necessary and needs to be separately defined for different devices types. |
| Max input level | Necessary and may needs to be separately defined for different devices types. |
| ACS | For standalone, FFS and details based on architecture. |
| blocking | In-band blocking: necessary Out of band blocking: Further discuss the out of band blocking performance based on RF architecture discussion |
| Spurious response | The same analysis as out of band blocking |
| Spurious emission | Spurious emission: to meet regulatory requirement |

Based on above analysis, we have following proposals.

Proposal 11: REFSENSE and max input level needs to be separately defined for different devices types.

Proposal 12: RAN4 needs to further discuss the out of band blocking performance based on RF architecture discussion.

2.5 Device 2b

For device 2b, it’s suggested to use UE RF framework as baseline and further discuss whether certain relaxation is needed or not.

Proposal 13: For device 2b, it’s suggested to use UE RF framework as baseline and further discuss whether certain relaxation is needed or not.

# 3. Conclusions

In this contribution, RF requirements for devices are discussed with following observations and proposals.

Proposal 1: it’s suggested to define one unified set of system parameter for A-IoT BS, A-IoT intermediate UE and devices.

Proposal 2: it’s suggested to only choose band n8 as example band.

Proposal 3: no need to define transmission bandwidth requirements in RAN4.

- Observation 1: Intra-A-IoT system guard RB is mainly used to reduce or even avoid interference for device FDM operation case or for A-IoT reader FDM operation case. Such interference would be even worse if reader or device have bad frequency accuracy performance.

Proposal 4: RAN4 further discuss the candidate channel bandwidth for all three spectrum deployment modes. For standalone mode, it seems 5MHz bandwidth is enough based on current assumed data rate and modulation scheme. For other two spectrum deployment mode, the channel bandwidth also seems necessary as common concept which may be used by reader.

Proposal 5: RAN4 further discuss whether any guard band is needed or not to avoid interference between A-IoT system and NR/LTE system.

- For guard-band spectrum deployment mode, guard band can be in any granularity.

- For in-band spectrum deployment mode, guard band could help to enhance current Tx leakage and Rx selectivity performance and detailed value can wait for co-existence results.

Proposal 6: syn raster is not applicable for all three spectrum deployment mode. For guard-band mode, the channel raster and channel spacing are not applicable. For in-band mode, the channel spacing is not applicable.

- Standalone: channel raster, channel spacing

- In-band: channel raster

- Guard-band: N/A

Proposal 7: as starting point, it’s suggested to discuss RF requirements separately for three device types.

Proposal 8: for device 1, taking RFID RF requirements as reference which only define output power and unwanted emission requirements. Besides, REFSENSE requirement is also needed.

Proposal 9: at least for transmit signal quality related requirement, RAN4 should wait for RAN1 conclusion of how to achieve small frequency shift in baseband and/or FDM(A) among devices.

Proposal 10: if RAN4 only simulate in-band spectrum mode, RAN4 further discuss whether/how to define out of band emission requirement.

Proposal 11: REFSENSE and max input level needs to be separately defined for different devices types.

Proposal 12: RAN4 needs to further discuss the out of band blocking performance based on RF architecture discussion.

Proposal 13: For device 2b, it’s suggested to use UE RF framework as baseline and further discuss whether certain relaxation is needed or not.

# 4. Reference

- R